3GPP TSG-WG SA2 Meeting #164	S2-2407858

Maastricht, Netherlands, August 19 - 23, 2024	(revision of S2-240)

Source:	Nokia,

Title:	TR 23.700-70 KI#2: Conclusion for Supporting PDU Set Info. Identification for E2E Encrypted XRM Traffic

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: This pCR proposes to conclude the KI#2.

# 1	Discussion

This paper provides conclusion of KI#2: Support PDU Set information identification for end-to-end encrypted XRM traffic. It is based on CMCC’s paper with additions justified by this discussion clause and by a companion discussion document

IETF based QUIC aware proxying works as follows

UPF

UE

In 3GPP we split up the IETF client between UE and UPF

QUIC aware proxying relies on the Client and Proxy regenerating actual CID (negotiated over QUIC in a ciphered way, so not visible to UPF) based on Virtal CID exchanged between the client (here UPF) and the Proxy (AS in our case).

If new QUIC CID values have been negotiated between the UE AS anytime during the lifetime of the QUIC connection (and this happens only in a QUIC end to end ciphered way) the UPF does not know these values and can’t regenerate them.

Hence we propose

UE supports the HTTP/3 Client functionality, which establishes a UDP tunnel to the UPF (where HTTP/3 Proxy operates) with the connect-UDP upgrade token and to set QUIC-Aware proxying (if supported by the UE). The reception of the connect-UDP upgrade token from the UE acts as a trigger for the UPF to generate connect-UDP upgrade token to the AS and to request set QUIC-Aware proxying.

# 2	Proposal

It is proposed to include the below changes into TR 23.700-70.

First Changes

### 8. Conclusions

The following principles will be used for the normative work :

.

- The normative R19 work will apply to UE to XRM server communication not to UE-UE XRM communication.

- The XRM metadata shall be encrypted and integrity protected between the UPF and the AS.

- Media over QUIC (MoQ) [9] and Proxy-UDP-in-HTTP/3[38]+QUIC-Aware Proxying[40] should be supported in normative work.

- For MoQ, the PDU set information is accommodated via Metadata in MoQ. UPF supports the MoQ relay functionality and identifies the PDU Set information from the MoQ metadata. The detailed information include:

- AF provides the Protocol Description and the requirement for MoQ in order to support the encryption information identification from the PDUs in PDU Set following the AF session with QoS procedure. The request may include AS address.

- PCF determines PCC rules based on AF request, to instruct SMF/UPF to identify PDU Set information for MoQ traffic.

- SMF provides N4 rule to UPF to activate the MoQ relay functionality.

- UPF supports the MoQ relay functionality and identifies the PDU Set information from the MoQ metadata. For DL, UPF includes the identified PDU Set information into GTP-U header as specified in Rel-18.

- The XRM metadata to be considered in the MoQ case are:…

- The XRM metadata are to encrypted and integrity protected between the UPF and the AS by mechanisms defined by….

- the format of the XRM metatadata exchanged between the UPF and the AS are defined by.....

NOTE: There is no 3GPP specification impact for UE. The application in UE uses MoQ protocol defined in [9] to obtain a MoQ relay address via DNS procedure and establish MoQ connection with MoQ reply in UPF.

- For Proxy-UDP-in-HTTP/3 [38] + QUIC-Aware proxying [40], XRM metadata information is included in HTTP datagram via a new datagram mode or via packet transforms in QUIC-Aware proxying[40]. UPF supports the HTTP/3 Client functionality. The detailed information include:

- AF provides the Protocol Description for XRM metadata Information, and the AS address to enable an encapsulation protocol connection between the UPF and AS. This is done with using the AF session with QoS procedure, and is meant to support the establishment of a UDP tunnel with the connect-UDP upgrade token.

- PCF generates a PCC rule including the indication of UDP tunnel establishment and AS address.

- SMF provides corresponding N4 rules to the UPF including the indication of UDP tunnel establishment and AS address.

- UPF supports the HTTP/3 Client functionality, which establishes a UDP tunnel to the AS (where HTTP/3 Proxy operates) with the connect-UDP upgrade token.

- For downlink packets received by the UPF over N6, N4 configuration information indicates the UPF to:

  - Extract the encapsulated ciphered UDP packet from the received IP packet. QUIC packets sent in forwarded mode are retrieved by removing the metadata with a packet transform.

  - Retrieve XRM metadata from information contained within the encapsulation protocol and include the corresponding information into GTP-U header using similar mechanisms as defined in Rel-18 (new GTP-u header information may be defined to support the output of other KI)..

  - Routing the decapsulated ciphered UDP packet (without the XRM metadata) over a QoS flow within GTP-u towards the RAN.

- XRM metadata may correspond to PDU set and DRX related information defined in R18 but may also correspond to XRM metdata defined as output of other Key Issues of this TR such as KI 4.

- the format of the XRM metatadata exchanged between the UPF and the AS will be defined by 3GPP SA4.

- The UPF and the AS negotiate in-band per QUIC-Aware proxying [40] whether QUIC-Aware proxying applies and whether it can apply in forward mode

- HTTP datagram formats do not need IETF standardization. If HTTP header fields are used to indicate support for the specific format, the names of such fields need to be registered with IANA. The specific datagram format will be specified by CT working groups.

- New packet transforms do not require IETF standardization, but the name of the packet transform must be registered with IANA. The specific packet transform will be specified by CT working groups.

- The Application Server acting as a QUIC aware proxying server sends downlink packets via the encapsulation protocol and includes additional XRM metadata provided in-band within the encapsulation protocol to assist the UPF to identify XRM metadata.

-	if the end-to-end connection between the UE and the Application Server is based on QUIC, to avoid re-encapsulation and re-encryption, all XRM payload packets carried on QUIC packets with short header should be forwarded using the Forwarded Mode in QUIC-Aware Proxying using HTTP [40] whrere the AS acts as a QUIC-Aware proxying server. The XRM metadata is encoded into the XRM packets by use of a packet transform that is applied to proxied QUIC packets. The XRM metadata shall be encrypted and integrity protected, using a shared key that is exchanged by QUIC-Aware proxying mechanisms during the negotiation of the transform, and an initialization vector that is obtained fromthe end-to-end encrypted QUIC packet. Further details of the encoding is to be specified in stage 3.

  - Otherwise the XRM payload packets are forwarded using Connect-UDP [38]. The XRM metadata is encoded into the XRM payload packets in a HTTP Datagram. Further details of the HTTP Datagram encoding is to be specified in stage 3

- UE may support the HTTP/3 Client functionality, which establishes a UDP tunnel to the UPF (where HTTP/3 Proxy operates) with the connect-UDP upgrade token and to set QUIC-Aware proxying (if supported by the UE). The reception of the connect-UDP upgrade token from the UE acts as a trigger for the UPF to generate connect-UDP upgrade token to the AS and to request QUIC-Aware proxying with the AS.

NOTE: This option avoids the change of QUIC-Aware proxying mode from Forward mode to Tunnel mode when during the lifetime of the QUIC connection the UE or of the AS changes the Connection ID(s) to be used on the QUIC connection;

- how the UE determines the addressing information for the proxy in the UPF will be defined during the normative phase.

.

End of Changes