3GPP TSG-WG SA2 Meeting #164	S2-2407898

# Maastricht, Netherlands, August 19th – 23rd, 2024	 (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0240 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Policy provisioning for PDU session with local SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion as in the cluase 8.1 of TR 23.700-49: - I-SMF can be configured with or receive the local offloading policy from PCF via SMF which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN and shall select local UPF(s). - the SMF may forward AF provided traffic influence information to I-SMF. This contribution proposes to add corresponding descriptions into the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause 6.X.3 for support of policy provisioning for Local-SBO PDU sessions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Forwarding local offloading policy and PCC rule to I-SMF is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 4894, TS 23.503 CR 1335 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.X.3	Policy provisioning for Local-SBO PDU session

6.X.3.1	General

For Local-SBO PDU session, the I-SMF is responsible for local traffic offloading. In this scenario, traffic routing related policy (e.g. AF traffic influence information in PCC rule, local offloading information) need be provided to the I-SMF.

6.X.3.2	Local offloading policy handling for Local-SBO PDU session

The Local Offloading Policy is generated by PCF and retrieved by SMF when the PDU session is authorized for Local-SBO as described in clause 6.X.2. The Local offloading Policy may also be provided with an Offload Identifier. The Offload Identifier is assigned by PCF, and is unique within the PLMN.The I-SMF for Local-SBO PDU session may also be preconfigured with Local Offloading policy and tagged with Offload Identifier(s).

For Local-SBO PDU session, the SMF generates Local Offloading Information based on the received Local Offloading Policy and sends the Local Offloading Information to I-SMF, as described in clause 6.X.2.

The detailed information of Local Offloading Policy is described in clause 6.4 of TS 23.503 [4].

6.X.3.3	PCC rule handling for Local-SBO PDU session

6.X.3.3.1	AF request on PDU Sessions supporting Local-SBO

For Local-SBO PDU Sessions, the AF may send to NEF an AF request to influence traffic routing as described in caluse 4.3.6 of TS 23.502[3]. When the SMF receives AF traffic influence information in the PCC rule, it provides to I-SMF with the AF traffic influence information associated with the DNAI(s) supported by I-SMF.

NOTE X: The AF does not need to to be aware of whether the PDU session is supporting Local-SBO.

When receving the AF traffic influence information, the I-SMF generates N4 rules for local traffic routing (e.g. PDR, FAR) based on EAS Deployment Information and the AF traffic influence information received from the SMF. The I-SMF configures the ULCL/BP based on the generated N4 rules.

# * * * * End of changes * * * *