3GPP TSG-WG SA2 Meeting #164	S2-2407903

# Maastricht, Netherlands, August 19th – 23rd, 2024	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0241 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 EAS discovery for Local-SBO PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion as described in the cluase 8.1 of TR 23.700-49, I-SMF based approach is concluded for the key issue of the Session Breakout model. This contribution proposes to name this scenario as “Local-SBO” and add corresponding descriptions into the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add a new clause 6.X for support of the local traffic routing with I-SMF for reducing impact on central SMF (Local-SBO). 2. Add Local-SBO related descriptions in the clause of abbreviations, architecture and EASDF service. |  |  |  |  |  |  |  |  |
| ` |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | I-SMF based solution for Local-SBO PDU session to reduce the impact on central SMF for discovery EAS is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2, 6.X(new), 7.1.2.1, 7.1.2.2, 7.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5483, TS 23.502 CR 4895 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [2].

C-DNS	Central DNS

C-NEF	Central NEF

C-PSA UPF	Central PSA UPF

EAS	Edge Application Server

EASDF	Edge Application Server Discovery Function

ECS	Edge Configuration Server

EDC	Edge DNS Client

EEC	Edge Enabler Client

EES	Edge Enabler Server

EHE	Edge Hosting Environment

L-DN	Local part of DN

L-DNS	Local DNS

L-NEF	Local NEF

L-PSA UPF	Local PSA UPF

Local-SBO	Local traffic Session BreakOut

HR-SBO	Home Routed Session BreakOut

# * * * * 2nd change * * * *

4.2	Reference Architecture for Supporting Edge Computing

The reference architectures for supporting Edge Computing are based on the reference architectures specified in clause 4.2 of TS 23.501 [2]. The following reference architectures for non-roaming, LBO roaming and HR with Session Breakout (HR-SBO) roaming scenarios further depict the relationship between the 5GS and a DN where Edge Application Servers (EASs) are deployed in an EHE.

Figure 4.2-1 depicts 5GS architecture for non-roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-1: 5GS providing access to EAS with UL CL/BP for non-roaming scenario

Figure 4.2-2 depicts 5GS architecture for non-roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-2: 5GS providing access to EAS without UL CL/BP for non-roaming scenario

Figure 4.2-3 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-3: 5GS providing access to EAS with UL CL/BP for LBO roaming scenario

Figure 4.2-4 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-4: 5GS providing access to EAS without UL CL/BP for LBO roaming scenario

Figure 4.2-5 depicts 5GS architecture for HR-SBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-5: 5GS providing access to EAS with UL CL/BP for HR-SBO roaming scenario

NOTE 1:	Only some of the 5GS NFs are shown in the above reference architecture figures. In the above figures, the split between the UPF acting as UL CL/BP and the UPF acting as local PSA is illustrative.

NOTE 2:	Only the control plane of EASDF is depicted in the figure, the user plane between the EASDF and the UPF (i.e. over which the DNS messages are exchanged) is part of N6. Additionally, the EADSF may have direct connectivity with the local parts of one or more Data Networks.

NOTE 3:	For the HR-SBO roaming scenario, there can be other UPF(s) located in VPLMN between the UPF acting UL CL/BP and the UPF acting as remote PSA in HPLMN.

Figure 4.2-X depicts 5GS architecture for non-roaming Local-SBO scenario supporting Edge Computing with UL CL/BP controlled by I-SMF.

Figure 4.2-X: 5GS providing access to EAS for non-roaming Local-SBO scenario

# * * * * 3rd change * * * *

6.X	Support of the local traffic routing with I-SMF for reducing impact on central SMF (Local-SBO)

6.X.1	General

The SMF may be deployed centrally in the network. To support the traffic offloading for an established PDU session, edge computing related information, e.g., EAS deployment information need be aware by central SMF.

Edge computing related information may be changed frequently (e.g., EAS deployment Information is changed due to the EAS instance is on or off), which leads to the update to central SMF frequently. To reduce the impact to the central SMF, local SMF(s) (i.e. SMF deployed locally) are introduced and manage the edge computing related information. With the assistance of the local SMF the EAS discovery and rediscovery can be supported with less impact to the central SMF.

The assistance of local SMF only applied to the non-roaming PDU session. For session breakout PDU session, local SMF act as I-SMF role to control the session traffic to be offloaded to local network, which also called Local-SBO. After the establishment of the Local-SBO PDU Session, the I-SMF (local SMF) is in charge of edge computing handling.

To select proper SMFs for the Local-SBO PDU session (i.e. select local SMF as I-SMF and central SMF as SMF), following SMF capabilities are considered by AMF:

-	Support Local-SBO. SMF that supports this feature shall support acting as SMF or I-SMF of Local-SBO PDU sessions.

-	Support local edge computing management. SMF that supports this feature shall support acting as I-SMF of Local-SBO PDU sessions (e.g. aware of EAS Deployment Information, control EASDF for EAS discovery as an I-SMF).

The reference architecture supporting this scenario is depicted in Figure 4.2-X in clause 4.2.

6.X.2	Procedure

6.X.2.1	General

This clause describes the authorization and EAS (re)discovery procedure of the local traffic offloading using Local-SBO PDU session.

6.X.2.2	PDU Session establishment for supporting Local-SBO

When the UE is in non-roaming scenario, the PDU Session establishment supporting Local-SBO is same as clause 6.7.2.2 by replacing V-SMF, H-SMF, V-UPF, V-EASDF, VPLMN Specific Offloading Policy/Information and HR-SBO with I-SMF, SMF, I-UPF, EASDF, local offloading control Policy/Information and Local-SBO accordingly, and with following differences:

1.	During the Registration procedure, the AMF receives Local-SBO allowed indication as part of the SMF Selection Subscription data per DNN/S-NSSAI from the UDM in the step 14b of the procedure in the clause 4.2.2.2.1 of TS 23.502 [3].

2.	During the PDU Session Establishment procedure for non-roaming as described in clause 4.3.2.2.1 of TS 23.502 [3], if the AMF had received the Local-SBO allowed indication for the requested DNN/S-NSSAI in the step 1, the AMF selects both an I-SMF and a SMF supporting Local-SBO. The selected I-SMF also need support local edge computing management and serves the area where UE camps.

The AMF sends a Local-SBO allowed indication to the I-SMF in the step 3a of clause 4.3.2.2.2 of TS 23.502 [3] with V-SMF replaced by I-SMF.

If the I-SMF supporting Local-SBO receives the Local-SBO allowed indication from AMF, it performs step 2 of clause 6.7.2.2 with following differences:

- The EASDF is controlled by I-SMF (i.e. the I-SMF creates/updates/deletes the DNS context on the EASDF).

-	when Neasdf_DNSContext_Create Request service operation is invoked to obtain the EASDF IP address, HPLMN ID is not needed due to in non-roamng scenario.

-	When sending the request to the SMF for the establishment of the Local-SBO PDU Session, the EASDF IP address and supported DNAI(s) are included in the request. The supported DNAI(s) are used for PCC rule handling in SMF.	The SMF authorizes the request for Local-SBO based on SM subscription data and once authorized. If local-SBO is authorized for the PDU session, the SMF performs as H-SMF as described in step 2 of clause 6.7.2.2 with following differences:

- Retrieveing the optional local offloading control policy from PCF.

- session AMBR for the local part of DN is not needed.

- HPLMN address information is replaced by the Central UPF address information.

- DNS server IP address of HPLMN is replaced by DNS server IP address provided by SMF.

The local offloading control policy includes the IP range(s) and/or FQDN(s) of traffic to be routed to local part of DN. The detailed information of local offloading control Policy is described in clause 6.4 of TS 23.503 [4]. The SMF generates local offloading control information based on the local offloading control policy.

3.	The I-SMF configures the EASDF with the DNS handling rules using the local offloading control information received from SMF.

The I-SMF configures the EASDF with the DNS server address provided by the SMF. If I-SMF has not received the DNS server address provided by SMF in step 2, a default DNS server may be configured to EASDF.

If the I-SMF has interacted with the EASDF in step 2, then the I-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the EASDF.

The I-SMF configures the UL CL UPF and PSA UPF selected in the step 2 to forward DNS messages to EASDF.

4A.	EAS Discovery procedure with I-SMF and EASDF is performed as described in clause 6.X.2.3.

6.X.2.3	EAS Discovery Procedure with EASDF for Local-SBO

I-SMF

EASDF

UE

DNS server

UPF

ULCL/BP

UPF

L-PSA

1. DNS query

2. The step 8 to 15 of the procedure in clause 6.2.3.2.2 by replacing SMF with I-SMF

UPF

L-PSA

SMF

3. ULCL/BP insertion with interaction between I-SMF and SMF

4. The step 17 to 18 of the procedure in clause 6.2.3.2.2 by replacing SMF with I-SMF

5. DNS response

I-SMF

EASDF

UE

DNS server

UPF

ULCL/BP

UPF

L-PSA

1. DNS query

2. The step 8 to 15 of the procedure in clause 6.2.3.2.2 by replacing SMF with I-SMF

UPF

L-PSA

SMF

3. ULCL/BP insertion with interaction between I-SMF and SMF

4. The step 17 to 18 of the procedure in clause 6.2.3.2.2 by replacing SMF with I-SMF

5. DNS response

Figure 6.X.2.3-1: Procedure for EAS Discovery with EASDF for local-SBO PDU session

When the UE is in non-roaming scenario, the EAS Discovery Procedure with EASDF for Local-SBO is same as clause 6.7.2.3,  by replacing V-SMF, H-SMF, V-UPF, V-EASDF, VPLMN Specific Offloading Policy/Information, HPLMN address information, HPLMN DNS server and HR-SBO with I-SMF, SMF, I-UPF, EASDF, local offloading control Policy/Information, Central UPF address information, DNS server provided by SMF and Local-SBO accordingly, where all 5GC functions are all belong to the same PLMN, and with following differences:

1.	If the target FQDN of the DNS query is not part of the FQDN authorized by the SMF, the option a) is performed.

Editor’s note: Whether option b) is needed to be supported in Local-SBO is FFS.

3.	The Authorized DL Session AMBR is not needed.

If the end-to-end delay requirement is provided by AF, the SMF need instruct the UPF to meausure the N6 delay and take that delay measurement into account for local PSA selection.

# * * * * 4th change * * * *

7.1.2	Neasdf_DNSContext Service

7.1.2.1	General

Service description: This service enables the consumer to create, update, or delete DNS context in EASDF and to Subscribe to DNS message related reporting from EASDF.

DNS contexts in EASDF include rules on how EASDF is to handle DNS messages.

This service also can be supported by V-EASDF in VPLMN for HR scenario supporting HR-SBO.

This service also can be used between EASDF and I-SMF to support Local-SBO PDU session.

7.1.2.2	Neasdf_DNSContext_Create Service Operation

Service operation name: Neasdf_DNSContext_Create

Description: Create a DNS context in EASDF.

Input, Required: UE IP address, DNN, S-NSSAI, Notification Endpoint.

Input, Optional: HPLMN ID, DNS message handling rules, N6 traffic routing information (towards the Local PSA-UPF).

NOTE 1:	In HR-SBO scenario, the V-SMF can invoke Neasdf_DNSContext_Create service in order to obtain the IP address of the V-EASDF while the UE UP address has not yet been determined. If the V-SMF is not aware of the UE IP address when invoking this service operation, the V-SMF can set UE IP address as unspecified such as zero value.

NOTE X:	In Local-SBO scenario, the I-SMF can invoke Neasdf_DNSContext_Create service in order to obtain the IP address of the EASDF while the UE UP address has not yet been determined. If the I-SMF is not aware of the UE IP address when invoking this service operation, the I-SMF can set UE IP address as unspecified such as zero value.

DNS message detection and Actions(s) are specified in clause 6.2.3.2.2.

NOTE 2:	N6 traffic routing information can contain the IP address the Local PSA-UPF.

Output, Required: If successful, IP address of the EASDF, EASDF Context ID, Result Indication.

Output, Optional: DNS Security Information of EASDF.

7.1.2.3	Neasdf_DNSContext_Update Service Operation

Service operation name: Neasdf_DNSContext_Update

Description: Update the DNS context in EASDF, or indicate EASDF to forward the DNS Response to UE.

Input, Required: EASDF Context ID, updated DNS message handling rules.

Input, Optional: UE IP address, N6 traffic routing information (towards the Local PSA-UPF).

NOTE:	In HR-SBO scenario, the V-SMF can provide the UE IP address received from the H-SMF to update DNS Context of the V-EASDF via this Service Operation.

NOTE X:	In Local-SBO scenario, the I-SMF can provide the UE IP address received from the SMF to update DNS Context of the EASDF via this Service Operation.

Output, Required: Result Indication.

# * * * * End of changes * * * *