SA WG2 Meeting #164	S2-2407948

19 - 23 August, 2024, Maastricht, Netherland	(Revision of S2-240xxxx)

Source: 	China Mobile

Title: 	KI #1, Update of conclusion for KI#1

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This contribution provides update of conclusion for key issue #1.

1	Discussion

This contribution provides the update of conclusion for KI#1.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-77.

FIRST CHANGE

8	Conclusions

## 8.1	Conclusions of KI #1

For KI#1 on "Extensible IMS mechanism supporting IMS events in the context of DC communication" the following solution principles are proposed:

- In this release, only exposure of IMS AS related events are supported as described below. The DCSF does not support the event exposure mechanism in this release, i.e. DC3 and DC4 is not specified in this release.

1.	A new NEF service, based on SUBSCRIBE/NOTIFY requests, is defined for the IMS events in the context of DC communication.

2.	The NEF service must be extensible and shall support events of IMS AS that are IMS DC related and non-IMS DC related.

3.	Subscriber specific and non-subscriber specific must be supported. A subscriber specific event is e.g. an event like “user A has registered to the network or has established a call”, a non-subscriber event is e.g. an event like “a user has called number 800-123456” where 800-123456 is a service number.

A subscriber specific request includes the targeted subscribers. The targeted subscriber(s) can be either one distinct subscriber, or multiple subscribers given as a list of distinct subscribers or a wildcarded IMS user identity. The events in the request are common for all targeted subscribers. This request is proxied to the HSS instance(s) serving the given subscribers.

4.	HSS shall support the handling of subscriber specific event subscription requests. To that effect, the event subsription requests that are subscriber specific are sent to HSS which handles registered and unregistered UEs and ensures that a subscription is persistent when UE registers, deregisters and re-registers in IMS. That is, the HSS stores the subscription request and sends the subscription request to the IMS AS whenever the IMS AS instance supporting the requested event is assigned to the UE. The information about the subscription events stored in HSS is only relevant and consumed by HSS. NEF and AF use a new HSS SBI service to manage subscription requests (e.g. subscribe/unsubscribe) via HSS for subscriber specific events.

5.	The IMS AS instance assigned to a UE during IMS registration shall be registered in HSS. Existing SBI Nhss_ImsUECM service used for S-CSCF registration over N70 is enhanced to allow the registration of the IMS AS instance in HSS over N71.

6.	Subscription requests that are non-subscriber specific are sent directly to all related IMS AS instances.

NOTE 0:	How the related IMS AS instances to a event are determined is decided in normative phase.

7.	For the purpose of optimization and following 5GC exposure principles, any Notification related to a subscription is sent directly to the NF (e.g. NEF or AF) that triggered the subscription request using Nimsas_SessionEventControl Notify service operation.

NOTE 1: 	The need to specify an Sh Diameter version of these procedures is determined during the normative phase.

8.	Optionally, events supported by an event producer (i.e. IMS AS ) can be discovered via NRF.

NOTE 2: 	Events to be exposed are defined in normative phase.

NOTE 3:	Events like application download is not supported in Release 19 but the event “ADC establishment for a specific application” is supported.

9.	Subscription requests to IMS events may be optimized; event categories shall be defined to group a collection of individual events in an event category. The categories are defined during the normative phase.

End of CHANGES