3GPP TSG-WG SA2 Meeting # 164	S2-2407985

# Maastricht, NL, 19 - 23 August 2024										(revision of S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3797 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MPS support for Messaging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS4msg |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce the MPS support for SMS based on the outcome of the study for FS_MPS4msg |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updade the general descripton for subscription-related priority mechanism to cover the MPS support for SMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.3.18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[3]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[4]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[5]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[6]	3GPP TS 23.203: "Policy and charging control architecture".

[7]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[8]	3GPP TS 43.129: "Packet-switched handover for GERAN A/Gb mode; Stage 2".

[9]	3GPP TS 23.003: "Numbering, addressing and identification".

[10]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[11]	3GPP TS 43.022: "Functions related to MS in idle mode and group receive mode".

[12]	3GPP TS 25.304: "UE procedures in idle mode and procedures for cell re-selection in connected mode".

[13]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[14]	3GPP TS 29.060: "GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[15]	3GPP TS 43.051: "GERAN Overall description - Stage 2".

[16]	3GPP TS 25.401: "UTRAN overall description".

[17]	IETF RFC 1034 (1987): "Domain names – concepts and facilities" (STD 13).

[18]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[19]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[20]	Void.

[21]	Void.

[22]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[23]	3GPP TS 44.064: "Mobile Station - Serving GPRS Support Node (MS-SGSN); Logical Link Control (LLC) Layer Specification".

[24]	3GPP TS 23.251: "Network Sharing; Architecture and functional description".

[25]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[26]	IETF RFC 768: "User Datagram Protocol".

[27]	3GPP TS 23.221: "Architectural requirements".

[28]	3GPP TS 23.008: "Organization of subscriber data".

[29]	3GPP TS 23.078: "Customized Applications for Mobile network Enhanced Logic (CAMEL) Phase X; Stage 2".

[30]	3GPP TS 23.236: "Intra-domain connection of Radio Access Network (RAN) nodes to multiple Core Network (CN) nodes".

[31]	IETF RFC 3588: "Diameter Base Protocol".

[32]	IETF RFC 4861: "Neighbor Discovery for IP Version 6 (IPv6)".

[33]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[34]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[35]	IETF RFC 4960: "Stream Control Transmission Protocol".

[36]	3GPP TS 36.413: "Evolved Universal Terrestrial Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[37]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[38]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[39]	Void.

[40]	3GPP TS 33.102: "3G Security; Security architecture".

[41]	3GPP TS 33.401: "3GPP System Architecture Evolution: Security Architecture".

[42]	3GPP TS 48.018: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN); BSS GPRS Protocol (BSSGP)".

[43]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[44]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[45]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[46]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[47]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[48]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[49]	3GPP TS 22.042: "Network Identity and Time Zone (NITZ) service description; Stage 1".

[50]	Void.

[51]	3GPP TS 32.240: "Charging architecture and principles".

[52]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[53]	3GPP TS 24.285: "Allowed Closed Subscriber Group (CSG) List; Management Object (MO)".

[54]	3GPP TS 23.261: "IP flow mobility and seamless Wireless Local Area Network (WLAN) offload; Stage 2".

[55]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

[56]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[57]	3GPP TS 23.271: "Functional stage 2 description of LCS".

[58]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[59]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[60]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[61]	3GPP TS 29.303: "Domain Name System Procedures; Stage 3".

[62]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[63]	IETF RFC 3810: "Multicast Listener Discovery Version 2 (MLDv2) for IPv6".

[64]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[65]	IETF RFC 4291: "IP Version 6 Addressing Architecture".

[66]	3GPP TS 22.368: "Service Requirements for Machine-Type Communications (MTC); Stage 1".

[67]	3GPP TS 22.011: "Service Accessibility".

[68]	3GPP TS 22.153: "Multimedia priority service".

[69]	3GPP TS 24.368: "Non-Access Stratum (NAS) configuration Management Object (MO)".

[70]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[71]	3GPP TS 23.002: "Network Architecture".

[72]	3GPP TS 23.007: "Restoration procedures".

[73]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[74]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[75]	3GPP TS 23.380: "IMS Restoration Procedures".

[76]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[77]	IETF RFC 5795: "The RObust Header Compression (ROHC) Framework".

[78]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[79]	3GPP TS 29.128: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications".

[80]	3GPP TS 22.101: "Service aspects; Service principles".

[81]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[82]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[83]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[84]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[85]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[86]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[87]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access -E-UTRA); Medium Access Control -MAC) protocol specification".

[88]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[89]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[90]	3GPP TS 38.401: "NG-RAN; Architecture description".

[91]	3GPP TS 29.674: "Interface between the UE radio Capability Management Function (UCMF) and the Mobility Management Entity (MME); Stage 3".

[92]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[93]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[94]	IETF RFC 8415: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[xx]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

# * * * * Next change * * * *

#### 4.3.18.1	General

Multimedia Priority Service (MPS) allows certain subscribers (i.e. Service Users as per TS 22.153 [68]) priority access to system resources in situations such as during congestion, creating the ability to deliver or complete sessions of a high priority nature. Service Users are government-authorized personnel, emergency management officials and/or other authorized users. MPS supports priority sessions on an "end-to-end" priority basis.

MPS is based on the ability to invoke, modify, maintain and release sessions with priority, and deliver the priority media packets under network congestion conditions. MPS is supported in a roaming environment when roaming agreements are in place and where regulatory requirements apply.

NOTE 1:	If a session terminates on a server in the Internet (e.g. web-based service), then the remote end and the Internet transport are out of scope for this specification.

MPS is supported for Service Users using UEs connecting via 3GPP access. MPS is also supported for Service Users using UEs that support connecting via Trusted or Untrusted non-3GPP access via WLAN as specified in TS 23.402 [2] for MPS. ePDG selection is according to clause 4.5.4 in TS 23.402 [2].

A Service User obtains priority access to the Radio Access Network by using the Access Class Barring mechanism according to TS 36.331 [37] and TS 22.011 [67]. This mechanism provides preferential access to UEs based on its assigned Access Class. If a Service User belongs to one of the special access-classes as defined in TS 22.011 [67], the UE has preferential access to the network compared to ordinary users in periods of congestion.

MPS subscription allows users to receive priority services, if the network supports MPS. The same MPS subscription applies to access via 3GPP access and non-3GPP access via WLAN. MPS subscription entitles a USIM with special Access Class(es). MPS subscription includes indication for support of Priority EPS Bearer Service, IMS priority service and CS Fallback priority service support for the end user. Priority level regarding Priority EPS Bearer Service and IMS are also part of the MPS subscription information. The usage of priority level is defined in TS 23.203 [6] and TS 23.228 [52].

A MPS subscription may include optional MPS for Messaging indication which sets(enables)/clears(disables) the MPS priority treatment of messaging service (i.e. SMS over NAS, SMS over IP and messaging over IMS). The MPS for Messaging indication is used in EPC entities and not delivered to UE via NAS message. The detailed MPS for Messaging indication handling is further specified in TS 23.272 [58] for SMS over NAS via MME, in TS 23.204 [xx] for SMS over IP and in TS 23.228 [52] for messaging over IMS. The MPS for Messaging indication in subscription data may be provisioned via SCS/AS as specified in TS 23.682 [74].

NOTE 2:	The MPS subscription on the USIM is also used for 3GPP procedures with priority treatment over WLAN for UEs that support MPS over Trusted and Untrusted non-3GPP access via WLAN feature.

NOTE 3:	The above statements for the Priority EPS Bearer Service are also applicable for the MPS for Data Transport Service.

An MPS Service User is treated as an On Demand MPS subscriber or not, based on regional/national regulatory requirements. On Demand service is based on Service User invocation/revocation explicitly and applied to the PDN connections for an APN. When not On Demand, MPS service does not require invocation, and provides priority treatment for all EPS bearers for a given Service User after attachment to the EPS network.

NOTE 4:	According to regional/national regulatory requirements and operator policy, On-Demand MPS Service Users can be assigned the highest priority.

Since the Service User has an access class within the range for priority services, the Establishment Cause in RRC connection request is set to highPriorityAccess. When the eNodeB receives mobile initiated signalling with establishment cause set to highPriorityAccess, the eNodeB handles the RRC connection request with priority. When the MME receives and verifies mobile initiated signalling with establishment cause set to highPriorityAccess, the MME establishes the S1 bearer with priority.

The terminating network identifies the priority of the MPS session and applies priority treatment, including paging with priority, to ensure that the MPS session can be established with priority to the terminating user (either a Service User or normal user).

Priority treatment for MPS includes priority message handling, including priority treatment during authentication, security, and location management procedures.

Priority treatment for MPS session requires appropriate ARP and QCI (where necessary for non-GBR bearers) setting for bearers according to the operator's policy.

When an MPS session is requested by a Service User, the following bearer management principles apply in the network:

-	EPS bearers (including default bearer) employed in an MPS session shall be assigned ARP value settings appropriate for the priority level of the Service User.

-	Setting ARP pre-emption capability and vulnerability for MPS bearers, subject to operator policies and depending on national/regional regulatory requirements.

-	Pre-emption of non-Service Users over Service Users during network congestion situation, subject to operator policy and national/regional regulations.

Priority treatment is applicable to IMS based multimedia services, priority EPS bearer services (PS data without IMS interaction) and CS Fallback.

For Multimedia Priority services any EPC functions, procedures and capabilities are provided according to this clause's specification except when specified differently in the following clauses.

# * * * * End of changes * * * *