3GPP TSG-SA2 Meeting #164	S2-2407990

# Maastricht, Netherlands, August 19 - 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.203 | CR | 1139 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of MPS priority for Messaging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS4msg |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support MPS priority for Messaging |  |  |  |  |  |  |  |  |
| Summary of change: |  | First change: Added a reference to TS23.204 in clause 2. Second change: Expanded the text on dynamic invocation in clause 6.1.11.1 for Data Transport Service and MPS for Messaging. Third change: Added text to extend PCC rules and QoS rule generation to MPS for Messaging in clause 6.1.11.2. Fourth change: Added IMS Signaling Bearer priority support for MPS for Messaging in new clause 6.1.11.X. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of MPS priority for Messaging services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.11.1, 6.1.11.2, 6.1.11.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***** First change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 41.101: "Technical Specifications and Technical Reports for a GERAN-based 3GPP system".

[2]	Void.

[3]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[4]	IETF RFC 4006: "Diameter Credit-Control Application".

[5]	3GPP TS 23.207: "End-to-end Quality of Service (QoS) concept and architecture".

[6]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[7]	3GPP TS 23.125: "Overall high level functionality and architecture impacts of flow based charging; Stage 2".

[8]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[9]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[10]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[11]	3GPP TR 33.919: "3G Security; Generic Authentication Architecture (GAA); System description".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	Void.

[14]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[15]	"WiMAX End-to-End Network Systems Architecture" (http://www.wimaxforum.org/technology/documents).

[16]	3GPP TS 23.003: "Numbering, addressing and identification".

[17]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[18]	3GPP TS 23.402: "Architecture Enhancements for non-3GPP accesses".

[19]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[20]	3GPP2 X.S0057-B v2.0: "E UTRAN - HRPD Connectivity and Interworking: Core Network Aspects", July 2014.

[21]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[22]	3GPP TS 29.213: "Policy and Charging Control signalling flows and QoS parameter mapping".

[23]	3GPP TS 23.261: "IP Flow Mobility and seamless WLAN offload; Stage 2".

[24]	3GPP TS 23.198: "Open Service Access (OSA); Stage 2".

[25]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[26]	3GPP TS 29.335: "User Data Convergence (UDC); User Data Repository Access Protocol over the Ud interface; Stage 3".

[27]	3GPP TS 22.115: "Service aspects; Charging and billing".

[28]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[29]	3GPP TS 23.139:"3GPP-Fixed Broadband Access Interworking".

[30]	Broadband Forum TR-203: "Interworking between Next Generation Fixed and 3GPP Wireless Access" (work in progress).

[31]	Broadband Forum TR-134: "Policy Control Framework " (work in progress).

[32]	3GPP TS 25.467: "UTRAN architecture for 3G Home Node B (HNB); Stage 2".

[33]	Broadband Forum TR-291: "Nodal Requirements for Interworking between Next Generation Fixed and 3GPP Wireless Access" (work in progress).

[34a]	Broadband Forum TR-124 issue 2: "Functional Requirements for Broadband Residential Gateway Devices".

[34b]	Broadband Forum TR-124 issue 3: "Functional Requirements for Broadband Residential Gateway Devices".

[35]	Broadband Forum TR-101: "Migration to Ethernet-Based Broadband Aggregation".

[36]	Broadband Forum TR-146: "Internet Protocol (IP) Sessions".

[37]	Broadband Forum TR-300: "Nodal Requirements for Converged Policy Management".

[38]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[39]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[40]	Broadband Forum TR-092: "Broadband Remote Access Server (BRAS) Requirements Document".

[41]	Broadband Forum TR-134: "Broadband Policy Control Framework (BPCF)".

[42]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[43]	3GPP TS 23.161: "Network-based IP flow mobility and Wireless Local Area Network (WLAN) offload; Stage 2".

[44]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[45]	3GPP TS 26.114: "Multimedia telephony over IP Multimedia Subsystem (IMS); Multimedia telephony; media handling and interaction".

[46]	3GPP TS 23.179: "Functional architecture and information flows to support mission-critical communication service; Stage 2".

[47]	IETF RFC 6066: "Transport Layer Security (TLS) Extensions: Extension Definitions".

[48]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[49]	3GPP TS 22.011: "Service accessibility".

[50]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[51]	3GPP TS 22.261: "Service requirements for the 5G system; Stage1".

[52]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[53]	3GPP TS 26.238: "Uplink streaming".

[54]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[55]	3GPP TS 23.221: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Architectural Requirements".

[XX]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic Internet Protocol (IP) access; Stage 2".

# ***** Second change *****

#### 6.1.11.1	Architecture model and Reference points

Subscription data for MPS is provided to PCC through the Sp reference point. To support MPS service, the PCRF shall subscribe to changes in the MPS subscription data for Priority EPS Bearer Service. Dynamic invocation for MPS Voice/Video is provided from an AF, using the Priority indicator, over Rx. Dynamic invocation for Data Transport Service is provided by sending an MPS for Data Transport Service request to the PCRF over Rx. Activation for MPS for Messaging may be controlled by setting (enabling)/clearing (disabling) an MPS for Messaging indication in the HSS, for example via an SCS/AS, and by optionally sending a request containing the MPS for Messaging indication to the PCRF over Rx for the PCRF to modify the IM CN signalling bearer for MPS for Messaging.

# ***** Third change *****

#### 6.1.11.2	PCC rule authorization and QoS rule generation

For MPS service, the PCRF shall generate the corresponding PCC/QoS rule(s) with the ARP/QCI parameters as appropriate for the prioritized service.

For non-MPS service, the PCRF shall generate the corresponding PCC/QoS rule(s) as per normal procedures, without consideration whether the MPS Priority EPS Bearer Service is active or not, but upgrade the ARP/QCI values suitable for MPS when the Priority EPS Bearer Service is invoked. When the Priority EPS Bearer Service is revoked, the PCRF shall change the ARP/QCI values modified for Priority EPS Bearer Service to appropriate values.

NOTE 1:	The above statements for the Priority EPS Bearer Service are also applicable for the MPS for Data Transport Service, and for MPS for Messaging Service.

Whenever one or more AF sessions of an MPS service are active within the same PDN connection, the PCRF shall ensure that the ARP priority level of the default bearer is at least as high as the highest ARP priority level used by any authorized PCC rules belonging to an MPS service. If the ARP pre-emption capability is enabled for any of the authorized PCC rules belonging to an MPS service, the PCRF shall also enable the ARP pre-emption capability for the default bearer.

NOTE 2:	This ensures that services using dedicated bearers are not terminated because of a default bearer with a lower ARP priority level or disabled ARP pre-emption capability being dropped during mobility events.

NOTE 3:	This PCRF capability does not cover interactions with services other than MPS services.

# ***** Fourth change *****

#### 6.1.11.X	Bearer Priority for MPS for Messaging

MPS for Messaging (specified in TS 23.401 [17], clause 4.3.18.X) requires MPS priority on the IM CN signalling bearer for messages delivered via SMS over IP (TS 23.204 [XX]) and via IMS Messaging (TS 23.228 [39] clause 5.16). IMS Session-based Messaging also requires MPS priority for media requests.

When the P-CSCF obtains MPS for Messaging indication, as specified in TS 23.228 [39], the P-CSCF may send an MPS for Messaging indication to the PCRF over the Rx interface to request the PCRF to modify the IM CN signalling bearer for MPS for Messaging. Upon receipt of this request, the PCRF shall authorize the request against the presence of an MPS EPS priority subscription. If the IM CN signalling bearer does not already receive MPS priority treatment, for example based on a prior request for MPS service (e.g., voice/video) that upgraded the IM CN signalling bearer, or based on the IMS Signalling Priority information in the SPR (see clause 6.2.4), and under consideration of the requirements described in clause 6.1.11.2, the PCRF shall also:

-	modify the ARP of the default bearer to the IMS APN as appropriate for the MPS for Messaging Service; and

-	modify the ARP and QCI in all the PCC/QoS rules that describe the IM CN signalling traffic to the values appropriate for MPS for Messaging Service.

When the PCRF receives an MPS for Messaging clearing (disabling) indication via Rx, the PCRF shall, while considering other services simultaneously requiring priority treatment of the IM CN signalling bearer as described in clause 6.1.11.2:

-	modify the ARP of the default bearer to the IMS APN to an appropriate value according to PCRF decision; and

-	modify the ARP and QCI in all PCC/QoS Rules that describe the IM CN signalling traffic to appropriate values according to PCRF decision.

At reception of service authorization from the P-CSCF (AF) for MPS priority for IMS Session-based Messaging, including the Priority indicator over Rx, the PCRF shall generate the corresponding PCC/QoS rule for the media flow(s), and the IM CN signalling bearer, with the ARP/QCI parameters as appropriate for the MPS for Messaging Service.

The AF responsible for on-demand changes in MPS for Messaging indication may also request an SDF for priority signalling between the UE and the AF, where the AF includes the Priority indicator over Rx, in order to enable the PCRF to set appropriate QoS values for the PCC/QoS rules that describe the signalling bearer.

# ***** End of changes *****