3GPP TSG-SA2 Meeting #164	S2-2408029

# Maastricht, Netherlands, August 19 – 23, 2024							   (revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5508 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization of MWAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on onclusion for KI#2: Authorization of a MWAB and configuration of MWAB Conclusion in TR 23.700-06, Authorization of a MWAB needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.x.3 (new) - Add description about Authorization of a MWAB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Authorization of a MWAB is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.x.3	Authorization of MWAB

5.x.3.1	General

MWAB-UE authorization and MWAB-gNB authorization are two different authorizations and managed by different entities in the networks. The BH 5GC controls the slice-based MWAB-UE authorization and the OAM that the MWAB-gNB connects to controls the MWAB-gNB authorization.

Details on authorization of MWAB-UE are described in clause 5.x.3.2. Details on authorization of MWAB-gNB are described in clause 5.x.3.3.

5.x.3.2	Authorization of MWAB-UE

Authorization of MWAB-UE is to allow the MWAB-UE to establish BH PDU Session(s) in the BH 5GC.

Dedicated S-NSSAI(s)/DNN for MWAB operation may be used for a MWAB-UE to indicate the MWAB operation to BH 5GC. Such S-NSSAI(s)/DNN for MWAB operation are stored in the MWAB-UE's subscription data.

The MWAB-UE authorization supports time-based and/or location-based control. For time-based control, existing slice subscription control defined in clause 5.15.16 can be used. For location-based control, existing mechanism, e.g. service area restriction, DNN/LADN based control can be used.	In addition, per operator/PLMN based location restriction on S-NSSAI defined in clause 5.15.8 can also affect the MWAB-UE authorization.

When the MWAB-UE's authorization status changes from "authorized" to "not authorized", the AMF performs as below:

-	If the AMF chooses to release the BH PDU Session(s), the AMF releases the BH PDU Session(s).

NOTE:	If the AMF recognizes the dedicated S-NSSAI/DNN used by the MWAB-UE for MWAB operation, e.g. a S-NSSAI for BH PDU Session support, the AMF can send a UE Configuration Update Command to the MWAB-UE and determine to delay the corresponding PDU Session release based on an operator configuration (e.g. local configured timer).

-	If the AMF chooses to de-register the MWAB-UE, the AMF de-registers the MWAB-UE.

5.x.3.3	Authorization of MWAB-gNB

Authorization of MWAB-gNB is to allow the MWAB-gNB to opearate as a gNB.

The OAM that the MWAB-gNB connects to is assumed to be aware of the MWAB-UE authorization limitations (e.g. pre-determined authorization limitations) and may (pre-)configure or trigger the MWAB-gNB to turn on or shut down accordingly.

When the MWAB-gNB is no longer authorized to operate by the OAM, the MWAB-gNB should hand over the UE(s) it serves to other cells. For the case that the BH PDU Sessions are released by the MWAB-UE, the MWAB-gNB also performs shut-down.

NOTE:	The coordination of the MWAB-gNB authorization/configuration from OAM and the update of subscription data (e.g. slice/DNN) of the MWAB-UE is handled by OAM/management system.

* * * * End of Changes * * * *