SA WG2 Meeting #164	S2-2408037

19th August – 23rd August, 2024, Maastricht, Netherland

Source:	Lenovo

Title:	Key Issue #1, Conclusions on AL-FEC

Document for:	Approval

Agenda Item: 	19.3.1

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This paper proposes conclusion principles for KI#1 on FEC part.

1	Discussion

At SA2#163 meeting, SA2 received the corresponding LS from SA4, RAN2 and RAN3, and some questions related to Application Layer FEC are answered, which give guidance on how to move forward.

SA4 (S2-2406850/S4-241368) LS states the following words on supported FEC schems:

Both static and dynamic redundancy ratios are used by the industry and may be supported. There are AL-FEC schemes in which the redundancy ratio is fixed based on code construction. Furthermore, dynamic redundancy ratios may be more beneficial in practice since applications often appeal to dynamic AL-FEC control in combination with retransmissions and congestion control mechanisms to ensure robust and efficient adaptation to network conditions.

Observation 1: Both static and dynamic FEC can be supported, and dynamic redundacny ratios are more practical. Here, for dynamic FEC, User Plane approach should be preferred since it is easier to notify the FEC ratio change via user plane (from AF=>UPF=>RAN).

What’s more, regarding the need to distinguish RAN intentionlly dropped packest, SA4 gives the feedback:

In this uncongested case SA4 sees that there is a need for the RAN to inform the application if packets are intentionally discarded.

Observation 2: NG-RAN should send the notification to Application to inform about the intentionaly discarded FEC packets.

Taking the above mentioned parts into consideration, we propose the following conclusion proposals for KI#1 regarding FEC.

2	Proposal

It is proposed to include the following changes in TR 23.700-70.

* * * * Start of Changes – All New Text * * * *

8.X	Conclusions for Key Issue #1 on FEC

The following conclusion principles are agreed for key issue #1.

- Both Control Plane and User Plane AL-FEC mechanism can be supported for 5GS. When enabled by the AF, NG-RAN can perform active discarding based on the provided FEC ratio parameter.

- For Control Plane mechanism, the FEC ratio parameter (the ratio of the PDUs within a PDU Set that are needed at the UE to reconstruct the original content) can be provided from AF to the NG-RAN using the (AF=>NEF/PCF=>SMF=>NG-RAN) path. The PCF may include the FEC ratio parameter in PCC rules and send to the SMF. The same FEC ratio applies to all the PDU Sets within the QoS flow and can be updated by AF when necessary.

- For User Plane mechanism, the dynamic FEC ratio parameter is provided per PDU set and sent to the PSA-UPF from AS via N6. It will be marked by the UPF into the N3 GTP-U header upon instructions from SMF and sent to the NG-RAN.

- NG-RAN sends the notification of intentional dropping of FEC packets to the AF for distinguishing from congestion lead packets dropping.

NOTE: Coordination with SA4 and RAN2 is needed.

* * * * End of Changes * * * *