SA WG2 Meeting #164	S2-2408055

Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-240xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5516 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | I-SMF insertion in EAS discovery for local service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-07-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in conclusion, I-SMF based approach is considered to reduce the impact on central 5GC NFs. The following contents are agreed in TR conclusion and are related to this CR changes: - I-SMF selection shall be performed by AMF. I-SMF selection mechanism shall be enhanced to support I-SMF for local offloading management, e.g. I-SMF selection for local offloading management is based on the subscription data. - EASDF configuration and DNS message handling is performed by I-SMF (instead of SMF). - EASDF discovery and selection is performed by I-SMF (instead of SMF). When the I-SMF approach is used the SMF does not discover and select an EASDF. - I-SMF retrieves EDI from NEF to configure EASDF. EDI management in the I-SMF shall be supported. I-SMF can also be preconfigured with the EAS deployment information. - I-SMF can be configured with or receive the local offloading policy from PCF via SMF which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN and shall select local UPF(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In this paper, following contents are introduced: - Add function enhancement in I-SMF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to support the local area network configuration is not clear, and how to I-SMF to reduce the impact to 5GC NFs is FFS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.x(all new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

5.34.x	Support EAS discovery for local service

This clause defines how 5GS supports to perform the EAS discovery in local area to reduce impact on central 5GC NFs, when UE establishes the PDU session.

In the SM subscription data, it may indicate possible local offloading for the UE to reduce the central 5GC NFs load and the I-SMF is selected by AMF according to the subscription data.

During the SM policy association procedure, the PCF generates the local offloading policy and provide the local offloading policy to SMF. The SMF further provides the local offloading policy to I-SMF transparently.

The I-SMF, replacing the SMF, is responsible for selecting EASDF and configuring DNS handling rules to EASDF. The I-SMF reuses the procedure of EDI management procedure to retrieve the EDI from UDR.

When receiving the DNS response from EASDF, if the EAS IP address or FQDN is in the scope of local offloading policy, the I-SMF inserts the local PSA and configures the local PSA with the traffic rule. The I-SMF, replace the SMF, to perform the UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

* * * End of Change * * *