3GPP TSG-SA WG2#164	S2-2408066

Maastricht, NL, 19-23 August 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated, Ericsson

Title:	Conclusions on KI#1

Document for:	Discussion/Approval

Agenda Item:	19.8.1

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This paper proposes principal conclusions on KI#1.

# 1.	Discussion

This paper proposes updated principles for the conclusion on KI#1: Identifying the Human User of a Subscription. The rationale of these changes is explained in this clause.

The only way that the human user can be identified in a UE is through its interaction with a user interface that belongs to the HLOS (for example in Android: https://support.google.com/android/answer/2865483?hl=en-GB or MS Windows: https://support.microsoft.com/en-gb/windows/how-to-switch-users-accounts-in-windows-660d4dcd-fa8d-7467-10b3-fee0e70e11d4 ). The “modem part” of the UE that is under the control of 3GPP and compliant to 3GPP specs, does not have a UI itself. This aspect is important to consider to achieve a realistic solution in this KI.

Considering this aspect, any possible security credentials or biometrics that need to be provided to differentiate between the different human users can only be provided using the HLOS' UI. Based on that assumption, the “modem part” of the UE has no means to determine whether the credentials or biometrics that are passed from the HLOS to the “modem part” of the UE correspond to an appropriate human user. For this reason, the use of 3GPP-level authentication mechanisms, e.g., secondary PDU session authentication or slice authentication do not provide any extra value in our view since the operator has no control of the layers that store and supply these credentials to the “modem part” of the UE.

For the user identity to “take effect”, the 3GPP system would need to support user authentication natively, which requires defining user identifiers and associated credentials (e.g., biometrics or other types) instead of relying on 3rd party authentication and authorization as in NSSAA or secondary PDU session authentication and authorization over NAS signaling. Furthermore, if user identifiers and credentials are managed by an HLOS , then such authentication would further require a trusted interface with the HLOS in the UE (which is currently out of the 3GPP scope).

If the goal of this key issue is merely to enable QoS differentiation, then one existing option for operator is to configure URSP rules to use different PDU sessions, e.g., different DNN/S-NSSAI and possibly QoS rules when the HLOS changes the user profile. Figure 1-1 shows an example of different URSP rules applied for different human users. Any of the existing Traffic Descriptors can be used in URSP rules to determine the change of human users and be agreed upon between the operators and HLOS vendors.

Figure 1-1: Example of URSP rules applied for different human users

# 2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-32:

# >>>>BEGINNING OF CHANGES<<<<

## 8.1	Key Issue #1: Identifying the Human User of a Subscription

The following bullets are the interim conclusion principles for KI#1:

Editor’s note: 	The above conclusion principles are only tentatively agreed.

-	The User Identifier format is NAI.

-	The User Identity Profile may contain:

-	one or more User Identifiers;

-	a list of linked subscriptions.

Editor's note:	Whether there can be more than one User identifier in the User identity Profile is FFS. The IE in the User Identity Profile is FFS.

-	The User Identifier is unique within PLMN and can be linked/unlinked with a 5GS subscription identified by a  GPSI.

-	The User Identifier and User Identity Profile are assigned and managed via application layer based on the interworking between UE and AF. The AF can be deployed by third party or operator. How to manage the user profile (creating, modifying, and deleting) is out of SA2's scope.

-	The linking/unlinking of the user identity profile with a subscription is supported via application layer based on the interworking between UE and AF.

-	The activation/deactivation of a user identity is verified and enforced from UE (e.g., HLOS and applications via username/password, fingerprint scan, facial recognition) and notifies the AF of the activation/deactivation of user identity.

-	The AF may leverage the existing 5GS capabilities (e.g., URSP guidance and QoS request as specified in clause 4.15.6 of TS 23.502 [5]) to support the service differentiation for the activated user identity.

NOTE:	Whether further information is required for e.g. billing is to be determined during normative phase based on SA5 input.

# >>>>END OF CHANGES<<<<