3GPP SA WG2 Meeting #164	S2-2408071

# Maastricht, NL, 19-23 August 2024											                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1426 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting of IMS Avatar communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#8 on " supporting IMS Avatar communication " are agreed. This CR implements the approved conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for Avatar communication, including architecture and procedures for both UE centeric and network centric based models. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS Avatar communication is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

# AC.X	Support of Avatar Communication

## AC.X.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication. Two modes for Avatar communication rendering are supported: UE centric and network centric.

The rendering of Avatar animation is performed based on Avatar assets, animation metadata and scene description. The Avatar assets are retrieved from the Digital Assets Repository, which includes the base Avatar model and any personalized accessories. The animation metadata can be generated by UE or the network from cameras, microphones, specialized motion capturing devices or capatured audio/videos streams. The scene description is an Avatar communication service control instruction generated by DC AS.

## AC.X.2	Architecture

### AC.X.2.1	General

The following figure depicts the enhanced architecture for AR calls with Avatars.

Figure AC.X.2.1-1: Enhanced architecture for AR calls with Avatars

To support Avatar communication, the data channel architecture supporting AR communication is enhanced as follows:

The Digital Assets Repository (DAR):

-	The DAR stores Avatar assets, including the base Avatar model and any personalized accessories. The assets are associated with a particular user and access to them is restricted. The owner can update the assets that are stored in the DAR. Participants in an AR call are authorized to access the base Avatar and a selected subset of accessories during the lifetime of the call.

AR Application Server (AR AS):

-	The AR Application Server is enhanced to support Avatar service control, including Avatar session media control, Avatar media capability negotiation with the UE.

MF:

-	The MF supports Avatar media processing. It receives and stores Avatar service media handling logic from the AR AS, and provides Avatar MF capability, including the generation of animation metadata, animation and rendering Avatar representation.

-	The MF supports retrieval of Avatar assets from DAR.

The following reference points are used for Avatar communication related signalling.

-	DAR1: Reference point between the UE and the DAR. It is used by the UE to upload and update the base Avatar model and its associated accessories.

-	DAR2: Reference point for MF to access the base Avatar model of another participant from DAR at the start or during an AR call

NOTE:	The DAR1 and DAR2 reference points are not specified in the specification.

### AC.X.2.2 UE Centric Avatar Communication

#### AC. X.2.2.1	Sending UE based rendering

For the UE centric case with sending UE based rendering, the sending UE animates the Avatar locally based on the Avatar base model and the animation streams. The sending UE uses its input data, e.g. the camera feeds and the user voice to generate the animation metadata. The rendered Avatar traffic is transparent to the IMS network and is sent to the peer UEs via the deployed media functions (e.g., IMS AGW/TrGW).

#### AC. X.2.2.2	Receiving UE based rendering

For the UE centric case with receiving UE based rendering, The receiving UE animates the Avatar locally based on the received Avatar base model and the animation metadata. The animation metadata can be generated by the sending UE using its input data or generated by MF from received audio/video streams.

### AC.X.2.3 Network Centric Avatar Communication

Network assisted Avatar communication is available depending on device capability and the requirements of the Avatar application on the media transport protocol used between the UE and the MF. Avatar services can be delivered via the following mechanisms:

-	An IMS Media Function (MF) having the Avatar Media Function (AMF) capability provides required network assisted Avatar media processing.

## AC.X.3	Procedures

### AC.X.3.1 UE Centric Avatar Communication

#### AC.X.3.1.1	Sending UE based rendering

Figure AC.X.3.1.1-1: Establishment of Sending UE rendered Avatar communication session

Figure AC.X.3.1.1-1 depicts a typical call flow to establish a Sending UE (UE-A) rendered Avatar communication. The main steps in the call flow are as follows:

0a.	UE-A generates or updates a conformant Avatar representation of the user and uploads it to the DAR. The representation may include the Avatar base model as well as other accessories, such as garments. UE-A may upgrade each of the components independently.

0b.	UE-A and UE-B establish an IMS session that includes audio, video, and bootstrap data channels that is used to distribute the initial scene description to both participants.

1.	The user of UE-A upgrades the IMS session with Avatar communication. UE-A initiates a re-INVITE adding media descriptors required for Avatar communication, e.g. RTP and/or application data channels.

2.	The AR Application Server generates the Avatar communication service control instruction in the scene description for the AR session and sends it over the data channel to the MF. The scene may include an Avatar ID of UE-A. The AR Application Server may update the scene during the lifetime of the IMS session by sending scene updates to the receivers.

3.	The MF/MRF forwards the scene description to the UE-A and UE-B over the established data channel.

4.	UE-A discovers the presence of Avatar ID in the scene. If UE-A does not have the Avatar base model of the Avtar ID, it sends a request to the DAR to access the Avatar base model.

5.	The DAR authorizes UE-A’s access to the Avatar base model.

6.	If successfully authorized, the DAR shares the selected Avatar base model and assets with UE-A.

7.  UE-A uses its input data, e.g., the camera feeds and the user voice to generate the animation metadata, then animates and renders it with the Avatar base model and scene description.

#### 8. UE-A sends the rendered Avatar media to UE-B via RTP.

#### AC. X.3.1.2	Receiving UE based rendering

Figure AC.X.3.1.2-1: Establishment of Receiving UE rendered Avatar communication session

Figure AC.X.3.1.2-1 depicts a typical call flow to establish a Receiving UE (UE-B) rendered Avatar communication. The main steps in the call flow are as follows:

0a.	UE-A generates or updates a conformant Avatar representation of the user and uploads it to the DAR. The representation may include the Avatar base model as well as other accessories, such as garments. UE-A may upgrade each of the components independently.

0b.	UE-A and UE-B establish an IMS session that includes audio, video, and bootstrap data channels that is used to distribute the initial scene description to both participants.

- The user of UE-A upgrades the IMS session with Avatar communication.

UE-A initiates media negotiation with AR AS to perform receiving UE rendered Avatar communication. After successful media negotiation, UE-A initiates a re-INVITE adding media descriptors required for Avatar communication, e.g. RTP and/or application data channels. The application data channel may be anchored in the MF.

2.	The AR Application Server generates the Avatar communication service control instruction in the scene description for the AR session and sends it over the data channel to the MF. The scene may include an Avatar ID of UE-A. The AR Application Server may update the scene during the lifetime of the IMS session by sending scene updates to the receivers.

3.	The MF/MRF forwards the scene description to the UE-A and UE-B over the established data channel.

4.	UE-B discovers the presence of the Avatar ID in the scene. UE-B sends a request to the DAR to access UE-A’s Avatar base model.

5.	The DAR authorizes UE-B’s access to the Avatar base model for the duration of the call. This step may involve the checking of the IMS session details and the authentication of UE-B. It may also include the checking of which assets and which level of details are to be shared.

6.	If successfully authorized, the DAR shares the selected subset of UE-A’s Avatar base model and assets with UE-B.

7a.	UE-A uses its input data, e.g., the camera feeds and the user voice to generate the animation metadata, then sends the animation metadata stream to MF via the established data channel.

7b.	UE-A sends the media streams that are used to generate the animation streams to the MF. These streams may include video streams from the user’s cameras and/or the user’s captured audio streams. The MF generates the animation streams from the received media streams.

NOTE:	Whether alternative 7a or 7b is performed is determined by media negotiation with AR AS and instructed by scene description.

8.	 The MF then sends the animation streams, which it received from UE-A or generated by itself, to UE-B.

9.	UE-B animates UE-A’s Avatar based on the downloaded Avatar base model and the received animation streams and then renders it as part of the scene.

### AC.X.3.2 Network Centric Avatar Communication

Figure AC.X.3.2-1: Establishment of Network Centric Avatar communication session

Figure AC.X.3.2-1 depicts a typical call flow to establish a network rendered Avatar communication. The main steps in the call flow are as follows:

0a.	UE-A generates or updates a conformant Avatar representation of the user and uploads it to the DAR. The representation may include the Avatar base model as well as other accessories, such as garments. UE-A may upgrade each of the components independently.

0b.	UE-A and UE-B establish an IMS session that includes audio, video, and bootstrap data channels that is used to distribute the initial scene description to both participants.

- The user of UE-A upgrades the IMS session with Avatar communication.

UE-A initiates media negotiation with AR AS to perform network rendered Avatar communication. After successful media negotiation, UE-A initiates a re-INVITE adding media descriptors required for Avatar communication, e.g., RTP and/or application data channels. The application data channel may be anchored in the MF.

2.	The AR Application Server generates the Avatar communication service control instruction in the scene description for the AR session and sends it over the data channel to the MF. The scene may include an Avatar ID of UE-A and UE-B. The AR Application Server may update the scene during the lifetime of the IMS session by sending scene updates to the receivers.

3.	The MF/MRF forwards the scene description to UE-A and UE-B over the established data channel.

4.	The MF discovers the presence of the Avatar ID in the scene. The MF sends a request to the DAR to access UE-A’s Avatar base model.

5.	The DAR authorizes MF access to the Avatar base model for the duration of the call.

6.	If successfully authorized, the DAR shares the selected subset of UE-A’s Avatar base model and assets with MF.

7a.	UE-A uses its input data, e.g., the camera feeds and the user's voice to generate the animation metadata, then sends the animation metadata stream to MF via established data channel.

7b.	UE-A sends the media streams that are used to generate the animation streams to the MF. These streams may include video streams from the user’s cameras and/or the user’s captured audio streams. The MF generates the animation streams from the received media streams.

NOTE:	Whether alternative 7a or 7b is performed is determined by media negotiation with AR AS and instructed by scene description.

8.	 The MF animates UE-A’s Avatar based on the downloaded Avatar base model and the received animation streams and then renders it as part of the scene.

9.	 The MF sends the rendered animation media to UE-B via RTP.

# >>>>END OF CHANGES<<<<