SA WG2 Meeting #164	S2-2408121

19 - 23 August, 2024, Maastricht, Netherlands	(revision of S2-2406087)

Source: 	KDDI, TOYOTA MOTOR CORPORATION

Title: 	KI#3, Conclusion for NWDAF-Based Energy Analytics

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: It is proposed to update the conclusion on KI#3.

1.	Discussion

For the aspect of NWDAF-Based Energy Analytics in KI#3, the following solutions are identified:

- Solution #3 proposes that NWDAF collects energy related information (e.g. Power, Energy and Environmental (PEE) measurements per cell, CQI related measurements per cell) from OAM, and provides energy related analytics to AF via NEF. Based on the analytics, the AF can decide to influence the service and select a suitable network configuration for network energy saving.

- Solution #5 proposes that NWDAF collects energy related information from other NFs (e.g. OAM, MDAF), and provides energy related analytics to AF via NEF (the details of the energy related information and energy related analytics are FFS). Based on the analytics, the AF may decide whether to use communication related application parameters or decide to use a different application parameter (e.g. assume a smaller bit rate in order to save energy) for a UE or group of UEs.

- Solution #11 proposes that NWDAF collects energy related information (e.g. carbon intensity in time and location per slice, ratio of renewable energy in time and location per slice) from OAM/MDAF and provides energy related analytics to PCF/AMF. Based on the analytics, the PCF can generate the URSP rules with validity conditions (e.g. time period of using solar supply during daytime as criteria for time window, the location where the solar power is supplied as criteria for location criteria) to guide a UE to transmit to a PDU Session supporting a greener network slice. The AMF can also generate NSSAI inforomation for a UE to allow the UE to potentially use the renewable energy in a specific time and/or location.

- Solution #12 proposes that NWDAF collects energy related information (e.g. Energy Consumption, Energy efficiency, NF energy usage, Datacenter energy efficiency ratio, Traffic load trend of cells, Energy Saving recommendation, Energy saving level, NF energy state, NF energy consumption estimate, renewable energy related information) from PCF, AMF, SMF, UPF, NRF, AF, OAM and/or  MDAF and provides energy related analytics (e.g. energy consumption, energy consumption of renewable energy, Carbon emission, energy efficiency, energy credit, NF energy state history, traffic volume, traffic/packet rate, traffic latency/ packet delay, Datacenter Carbon Emission Effectiveness KPI, Datacenter energy efficiency ratio, Analytics energy consumption, Spatial Validity, Energy Saving recommendation for NG-RAN/5GC, Traffic load trend for cells, NF energy saving level) at required granularities (e.g. network slice level, NF Instance/NF set level, PDU session level, QoS flow level, UE level) to consumer NF (e.g. NEF, PCF, AMF, SMF, EECF, OAM, CHF, AF (via NEF)). Based on the analytics, the consumer NF may trigger appropriate operation such as deciding the energy related limits for a service, making policy decision and adjustments, configuring the NF to enter an energy-saving mode, performing cell-based paging and providing a list of recommended cells, selecting a suitable UPF for the UE and recording the energy consumption information for charging.

- Solution #13 and Solution #33 propose UP path adjustment with energy related information (In detail, please see the description in the aspect of UP path adjustment).

- Solution #15 proposes that existing MM and SM procedures is enhanced based on the requirements of network energy saving and energy efficiency. In the enhanced MM procedures, AMF requests "UE Mobility" and "Abnormal behaviour" analytics from NWDAF. In the enhanced SM procedures, SMF requests "UE Communication" and "Abnormal behaviour" analytics from NWDAF. The energy related information is provided by a new NF (NEMF). Thus, there is no impact of existing NWDAF architecture because these analytics are defined in TS 23.288 [14].

- Solution #20 proposes that NWDAF collects energy related information for certain UEs, classifies the UEs in UE Energy Category and provides per UE analytics based on the received information at UE, UE/network slice and UE/PDU session level to PCF and AF (via NEF). Based on the analytics and the received information from SMF, the PCF may provide a modified QoS requirement for specific SDFs, PDU sessions or at UE level.

- Solution #29 proposes that NWDAF collects energy related information of the NF profile(s) of the target NF type and provides them to a consumer NF for energy-aware NF selection. Based on the energy related information, the consumer NF performs energy-aware NF selection.

- Solution #30 proposes that NWDAF provides network slice energy consumption analytics for a S-NSSAI to NSACF. Based on the analytics, the NSACF provides admission decision for updating maximum number of UEs and maximum number of PDU sessions per S-NSSAI to limit the energy consumption in the network slice.

- Solution #32 proposes that NWDAF collects energy related information from OAM, AF and other NFs and provides energy related analytics defined in Solution #12 corresponding to the historical energy consumption or prediction for the energy consumption over a certain period, NF load analytics and NF performance analytics to EECF. Based on the analytics and energy evaluation request from other NFs, the EECF provides session modifications recommendations in PDU session parameters to SMF for energy usage optimization.

- For KI#1 and KI#2, the following solutions are related to the NWDAF-Based Energy Analytics analytics:

  - Solution #4, #10, #18, #21, #25, #26, #27, #28 and #38

Based on the solutions above, we propose to update the interim conclusions on KI#3 for the aspect of the NWDAF-Based Energy Analytics.

2. 	Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-66 "Study on Energy Efficiency and Energy Saving".

# ***1st Changes ***

8.3	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

It is proposed to procced to normative work based on the following principles:

1)	For Enhancements on NF discovery and (re-)selection based on energy related information:

-	NF profile may be extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.

NOTE 1:	Which energy related information is included in the NF profile is to be further discussed in the normative phase. Only parameters captured in existing solutions will be considered. Coordinated with SA WG5 may be needed.

NOTE 2:	The information within NF profile is expected not to change frequently, e.g. constantly reflecting changes in energy consumption. The procedure for updating the information will be addressed in normative phase.

-	NF discovery and (re-)selection is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

2)	For enhancements on existing operations and procedures for energy saving and energy efficiency:

-	UP path of PDU session may be adjusted.

3)	For NWDAF-Based Energy Analytics for network energy saving and efficiency:

-	Network analytics from NWDAF is enhanced to assist NF in making a network energy saving related decision.

NOTE 3:	The energy related decision will also consider operator’s policy.

# *** End of Changes ***