3GPP TSG-SA WG2#164	S2-2408127

19-23 August 2024, Maastricht, Netherlands	(was S2-2406823)

Source:	Apple

Title:	Conclusion for KI#1.1

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#1.1.

# 1.	Introduction

This contribution proposes interim conclusion for KI#1.1 based on NWM discussions report in S2-2408126 and S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v1.0.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

### 8.1.x 	Conclusions for KI#1.1

The following principles are concluded for the normative work:

-	The two SUPIs for the DualSteer Device should be correlated in the UDM via subscription information, each SUPI subscription should contain a link to the associated SUPI.

-	The DualSteer Device can choose either of the SUPIs to be the first one to register with the network.

-	Based on operator's policy, a Primary SUPI can be assigned statically in the subscription. The HPLMN can inform the role of the SUPI to the DualSteer Device during the registration procedure if the DualSteer has been authorized.

-	If the operator policy requires to allow the secondary SUPI's registration only after the primary SUPI is registered, the network can reject the registration for the secondary SUPI if the primary SUPI is not registered.

-	The way how each of the SUPI is used will be determined by network policies defined by the operator.

# >>>>END OF CHANGES<<<<