3GPP TSG-WG SA2 Meeting #164	S2-2408150

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5534 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of CAG ID used for 5G Femto |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During the study phase of FS_5G_Femto, it is concluded that 5GS shall be enhanced to support deployment of 5G Femto. Therefore, this CR aims to add a new clause to describe how to support deployment of 5G Femto. The CAG concept of PNI-NPN is re-used for access control of Femto. The External Parameter Provisioning procedure is enhanced to support 5G Femto CAG provisioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause to describe the general enhancement of 5GS to support 5G Femto - Add a new clause to describe the enhancement of External Parameter Provisioning - Add a new annex to clarify the CAG provisioning of 5G Femto |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not capture the new objectives of 5G_Femto |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new), 5.X.Y (new), 5.X.Z (new), Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.X	Support for 5G Femto

5.X.Y	General

This clause describes the enhancement of 5GS to support deployment of 5G Femto.

NOTE:	The architecture enhancements for supporting 5G NR Femto deployments depends on the outcome of RAN WG3 study

The CAG concept defined in clause 5.30.3 is re-used for 5G Femto access control with the enhancement described in the following:

-	thbe External Parameter Provisioning procedure is enhanced to support 5G Femto CAG provisioning, see clause 5.X.Z.

-	The 5G Femto is not supported for roaming scenario in this release, i.e. both the UE and the 5G Femto belong to the HPLMN.

-	The Emergency Services are supported in 5G Femto as defined in clause 5.30.3.5.

The Annex X provides further information on the usage of CAG for 5G femto.

# * * * * Second change * * * *

5.X.Z	Support of 5G Femto CAG provisioning

The CAG mechanism specified in clause 5.30.3.4 is re-used for 5G Femto to perform access control. The 5G Femto CAG provisioning enables the owner of 5G Femto or an Authorized Administrator to manage the access control of Femto cell(s).

The External Parameters Provisionging procedure described in clause 4.15.6 of TS 23.502 [3] is used by AF to update the subscription data of a UE. The 5G Femto access control parameters including the list of CAG ID of the 5G Femto, the GPSI of the UE and optionally the validity condition (see TS 23.502 clause 4.15.6.3x) to authorize the UE to access the 5G Femto advertising the CAG ID included in the list of CAG ID. It is assumed that the owner of 5G Femto or Authorized Administrator provides CAG ID of the 5G Femto to the AF. The mechanism of binding the UE (GPSI) who is allowed to connect to the 5G Femto with a specific CAG ID is outside the scope of this specification. When the 5G Femto access control parameters are added in the UE subscription data the PLMN verify whether the CAG ID is related to a 5G Femto.

NOTE 1: It is assumed that the operator has provided the list of CAG ID associated to 5G femto to be used by AF based on SLA in case that the AF is deployed by a 3rd party. However, the AF may provide an erroneous CAG ID for any reason, since the CAG mechanism is used also for other scenarios, such as for PNI-NPN, the operator should verify whether the correct CAG ID is provided by AF before adding to the Subscription data. How the operator performs such control is outside the scope of the 3GPP.

In case of 5G Femto deployed by operator, the configuration of the 5G Femto access control parameters in UDM may be performed via OAM.

When the List of CAG ID is updated in UE subscription, the UDM may update the CAG information to the UE using the UE Configuration Update procedure for access and mobility management related parameters described in clause 4.2.4.2 of TS 23.502 [3].

When the UE receives the list of CAG ID , the UE may select the 5G Femto advertising the CAG ID which is included in the list of CAG ID received considering the validity condition, if present, and perform the Registration procedure via the selcted 5G Femto.

# * * * * Third change * * * *

Annex X (informative):
CAG used in 5G Femto

CAG concept is used in 5G Femto for access control. In order to ensure that the CAG ID is unique in the PLMN, the CAG ID of each 5G Femto cell is allocated by the Operator. The Operator provides the CAG ID of 5G Femto cell to the owner of the 5G Femto, if they are deployed by a 3rd party (e.g. via specific application which is outside the scope of this specification). The 5G Femto may be belong to a service provider which deployes the 5G Femto in different location or that re-sell as a service to private customers (e.g. to private owner for Home Femto or Small Office scenario).

The AF may perform the External Parameter Provisioning procedure in order to send to the PLMN the Femto access control information:

-	CAG ID of Femto cell;

-	GPSI of visitor UE of Femto cell;

-	Optionally, validity conditions of CAG (e.g. time duration).

The AF may be deployed by the owner of the 5G Femto or by the operator. The AF may be triggered by the owner of the 5G Femto by event not in the scope of this specification, for example an applicaton is running in the UE to enable the UE who is wishing to connects to a specific 5G Femto perform the request of authorisaton to the Owner of 5G Femto. The Owner of 5G Femto may provide the Femto access control information to the AF via the application. An other example can be the owner of the Home where the 5G Femto is deployed which interact via procedure not in the scope of this specification (e.g. via a phone call or web application to request that a give UE can access to the 5G Femto deployed in its location). In case that the AF is deployed by the operator the same example may apply and in addition the AF may be directly configured with the required information.

In case of AF deployed by 3rd party service provider, the AF may also store the relasionship between the CAG ID and the owner of 5G Femto, such as a Corporate, a commercial center, etc.. When the AF receives the request from the owner of 5G Femto, the AF can determine whether the CAG ID provided by the owner of 5G Femto is the corresponding CAG ID. If the CAG ID is the corresponding CAG ID of Femto owner, the AF triggers the External Parameter Provisioning procedure. Otherwise the AF rejects the request from the Femto owner.

In case of 5G Femto deployed by operator, the configuration of the binding of allowed CAG ID and the UE may be performed via OAM.

# * * * * End of changes * * * *