SA WG2 Meeting #164 	S2-2408209

# 19 - 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0549 |  | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support Direct AI/ML based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the Direct AI/ML based Positioning based on the conclusion in clause 8.1 in TR 23.700-84 as following: Principle #1: The LMF is enhanced to perform location calculation based on ML model. The interaction between LMF and MTLF is only for model provisioning and possibly, based on coordination with RAN WGs, for data collection for training. NOTE 1: When receiving the request for a UE location, the LMF selects an appropriate method to determine the UE location and may select AI/ML positioning as method. Principle #2: LMF is enhanced to perform model training for AI/ML based Positioning, the trigger for data collection and for model training in LMF is up to implementation. Principle #3: MTLF is enhanced to perform model training for AI/ML based Positioning. - Principle #3.1: To retrieve a model to perform location calculation, the LMF discovers a suitable MTLF via NRF. - Principle #3.2: NWDAF containing MTLF trains ML model for AI/ML based Positioning based on request from the LMF or internal trigger. - Principle #3.3: MTLF collects training data (e.g. data as defined by RAN for AI/ML based Positioning and historical data stored in ADRF) from data sources. NOTE 2: Whether LMF can also retrieve a trained model from OAM or not will be discussed in normative phase and need to coordinate with SA5. Principle #4: LMF or MTLF performs model performance monitoring for AI/ML based Positioning. - Principle #4.1 The result of model performance monitoring may trigger the LMF to change the positioning method, e.g. from AI/ML based positioning to the legacy positioning, or vice versa. - Principle #4.2 The result of model performance monitoring may trigger ML model retraining in the training entity. - Principle #4.3 The detailed procedure and service for this model performance monitoring will be decided in normative phase. Principle #5: Data used for model training, inference and model performance monitoring for AI/ML based positioning will be decided by RAN WGs, and SA WG2 will align with RAN WGs. The related procedures for data collection will be coordinated with RAN WGs in the normative phase. NOTE 3: The user's authorization/consent of collecting UE related training data is needed as specified in TS 23.288 [5] and TS 23.273 [7]. NOTE 4: Whether and how the NF profile of the LMF at the NRF will be extended to support selection of an LMF that supports AI/ML based Positioning will be discussed in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for the new description of the feature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIML_CN feature is not completed in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2b; 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First of Change * * *

4.2b	Positioning methods

The LCS feature utilises one or more positioning methods in order to determine the location of user equipment (UE). Determining the position of a UE involves two main steps:

-	Radio signal measurements or non-RAT measurements; and

-	Position estimate computation based on the measurements.

The positioning methods for 3GPP access are described in clause 5.2.

The positioning methods for non-3GPP access are described in clause 5.3.1.

The positioning method for Direct AI/ML based Positioning is described in clause 5.X.

* * * Second of Change * * *

5.X	Support Direct AI/ML based Positioning

When receiving the request for a UE location, the LMF selects an appropriate method to determine the UE location and may select Direct AI/ML positioning as method. In this method, the LMF is enhanced to discover a suitable NWDAF containing MTLF via NRF to perform location calculation based on AI/ML model that is provisioned via interaction between LMF and MTLF.

Editor’s note1: Based on coordination with RAN WGs, it will determine whether the interaction between LMF and MTLF is used for data collection for AI/ML model training.

The following two options are supported to perform AI/ML model training:

- LMF is enhanced to perform AI/ML model training for Direct AI/ML based Positioning, the trigger for AI/ML model training and for data collection for AI/ML model training in LMF is up to implementation.

- NWDAF containing MTLF is enhanced to perform AI/ML model training for Direct AI/ML based Positioning based on request from LMF or internal trigger. NWDAF containing MTLF collects the AI/ML model training data (e.g. data as defined by RAN for Direct AI/ML based Positioning and historical data stored in ADRF) from data sources.

NOTE 3:	The user's authorization/consent of collecting UE related AI/ML training data is needed as specified in TS 23.288 [5] and TS 23.273 [7].

Editor’s note2:	Whether LMF can also retrieve a trained AI/ML model from OAM or not will be discussed in normative phase and need to coordinate with SA5.

LMF or NWDAF containing MTLF performs AI/ML model performance monitoring for AI/ML based Positioning. The result of AI/ML model performance monitoring may trigger the LMF to change the positioning method, e.g., from Direct AI/ML based Positioning to legacy positioning method. And the result may trigger AI/ML model retraining in the training entity.

Editor’s note3: The reference of the procedure for AI/ML model performance monitoring for AI/ML based Positioning will be added when it is agreed.

Editor’s note4: Data used for AI/ML model training, AI/ML model inference and AI/ML model performance monitoring for Direct AI/ML based Positioning will be decided by RAN WGs, and SA WG2 will align with RAN WGs. The related procedures for data collection will be coordinated with RAN WGs in the normative phase.

Editor’s note5: Whether and how the NF profile of the LMF at the NRF will be extended to support selection of an LMF that supports Direct AI/ML based Positioning will be discussed in normative phase

* * * End of Changes * * *