SA WG2 Meeting #164 	S2- 2408227

# 19 - 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1154 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GC Supports for Vertical Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Vertical federated learning has been studied in TR 23.700-84, and some conclusion has been approved to capture the VFL as a new feature from R19 on. According the conclusion, this CR gives a high-level description to support VFL feature in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause to describe the VFL feature from high level. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Vertical Federated Learning is not supported in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (all new text)* * *

5.X	5GC supports for Vertical Federated Learning (VFL)

Vertical Federated Learning in 5GC, or between 5GC and AF, enables joint but distributed ML model training and model inference without any local data sharing. Different from Horizontal Federated Learning described in clause 5.3, VFL is applicable to the cases that local data sets in different parties share the same sample space but differ in feature space.

To support model training and model inference using Vertical Federated Learning, the following definitions are used:

VFL Server: An NWDAF or AF that integrates local training results for the local ML model update in VFL training process. It also coordinates the VFL training process by discovering and selecting VFL clients. In VFL inference process, The VFL server aggregates local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer. Only one VFL server exists for each VFL process.

VFL Client: An NWDAF or AF that holds the local dataset and performs local training and inference as asked by VFL Server. There can be multiple VFL Clients in VFL training and inference.

Note 1: In this Release of the specification, the NWDAF contains both MTLF and AnLF in it.

5GC supports the following three scenarios of Vertical Federated Learning:

- Scenario 1: VFL among NWDAFs in a single PLMN.

- Scenario 2: VFL between NWDAF as VFL server and AF(s) as VFL client(s)

- Scenario 3: VFL between AF as VFL server and NWDAF(s) as VFL client(s)

Note 2: For scenario 3 that AF as VFL server, only one AF is involved, but for Scenario 2 that AF as VFL client, multiple AFs can be involved.

When VFL server is the NWDAF (i.e. for scenario 1 and 2), VFL process is associated with an analytics ID (i.e. VFL training of models for analytics ID, VFL inference for an analytics ID). In this case, NF consumer subscribes or requests NWDAF for analytics ID, the NWDAF as VFL server starts VFL training of models for this analytics ID, and then VFL inference for this analytics ID. The NWDAF as VFL server will provide output based on the VFL inference process to the NF consumer as the required analytics.

When VFL server is AF (i.e. for scenario 3), VFL process is associated with AF internal process (i.e. VFL training of models for AF internal process, VFL inference for AF internal process). In this case, the AF as VFL server starts VFL training of models with NWDAF(s) as VFL clients, and then VFL inference process. If AF is untrusted, the interaction between AF and NWDAF should be via NEF.

Since the model training via VFL is distributed in VFL server and VFL client(s), no one will hold the whole ML model after VFL training, hence an ML model identifier is allocated by a VFL server, which is associated with the distributed ML Models in the VFL training process, and this ML model identifier is also used to correlate the participants (i.e. VFL server and VFL client(s)) during the VFL training and subsequent VFL inference process. When performing the VFL inference, the VFL server determines corresponding VFL client(s) for the inference process based on the same ML model identifier used in the VFL training process.

To support each VFL scenario above, some detailed procedures are defined in corresponding clauses:

- Registration and discovery of VFL entities (see clause 6.2X);

- Sample alignment and optional feature alignment for VFL (see clause 6.2X);

- VFL training process (see clause 6.2X);

- VFL inference process (see clause 6.2X);

* * * End of Changes * * *