3GPP TSG-SA WG2 Meeting #164	S2- 2408230

# Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1157 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New procedures for VFL training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R19 FS_AIML_CN has concluded KI#2 for supporting Vertical Federated Learning in TR 23.700-84. As for VFL training procedures, either the NWDAF or the AF can act as VFL server and initiate VFL training process with the VFL client(s). If an untrusted AF is involved in the VFL training process, the interactions between the NWDAF and the AF are via NEF. During the training procedure, the VFL client needs to compute the intermediate result for the local model and transfer to the VFL server. After that, the VFL server aggregates the intermediate result and computes gradient information and loss information. Then, VFL server sends the gradient information and loss information to the VFL client to update the local model. After several rounds of interaction, the VFL training process can be terminated by VFL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper proposes to capture VFL training process when NWDAF acts as VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training process is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2x(new), 6.2x.1(new), 6.2x.2(new), 6.2x.2.x(new), 6.2x.2.x.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * * (All new text)

6.2X	5GC supports Vertical Federated Learning

6.2X.1	General

This clause specifies how 5GC supports model training and model inference using Vertical Federated Learning (VFL). Either the NWDAF or the AF can act as VFL server and initiate VFL process with the VFL client(s).

6.2X.2	Procedures

6.2X.2.x	VFL training process

6.2X.2.x.x	Procedure for VFL training when NWDAF acts as VFL server

Figure 6.2X.2.x.x: VFL training procedure when NWDAF acts as the VFL server

0.	[Optional] The consumer NF may send analytics request to NWDAF by using Nnwdaf_AnalyticsSubscription_Subscribe service or Nnwdaf_AnalyticsInfo_Request service with analytics ID, Target of Analytics Reporting and analytics filter information (e.g. AOI).

1.	Based on the request in step 0 or the internal logic, the NWDAF as the VFL server may determine to use VFL mechanism if the ML model needs to be trained on local data set(s) but some data cannot be obtained from data source AF(s) (e.g. due to data privacy, data security). NWDAF as the VFL server allocates an ML model identifier, which is used to associate with the distributed ML Models during the VFL training process and used to to correlate the participants (i.e. VFL server and VFL client(s)) during the VFL training and subsequent VFL inference process.

2.	The NWDAF as VFL server selects VFL client(s) (i.e., NWDAF, AF) to participate the VFL training procedure by querying NRF as described in clause 6.2X.2.y.

3.	The NWDAF as VFL server and selected VFL clients perfrom the samples alignment procedure as described in clause 6.2X.2.z, and optional feature alignment procedure as described in clause 6.2X.y.y.

4a.	[Conditional] The NWDAF as the VFL server sends Nnwdaf_MLModelTraining_Subscribe to the selected NWDAF(s) as VFL client. The service operation includes the analytics ID (i.e., VFL training of models for analytics ID), the allocated ML model identifier and optional the analytic filter information (e.g., AOI).

4b.	[Conditional] The NWDAF sends the VFL training request to the AF(s) as VFL clients by invoking Naf_VFLTranining_Subscribe service. The Naf_VFLTraninging_Subscribe message includes the analytics ID (i.e., VFL training of models for analytics ID), the allocated ML model identifier and optional the analytic filter information (e.g., AOI). If the VFL client is an untrusted AF, NWDAF sends the VFL training request to the AF(s) via NEF.

Editor’s Note: How the NEF assists the VFL training process, and which NEF service should be invoked are FFS.

5a-5b. The NWDAF as VFL client and/or AF as VFL client perform local model training for the VFL and generates the intermediate result per sample base on it.

6a. [Conditional] The NWDAF as VFL client sends Nnwdaf_VFLTraining_Notify to the NWDAF as VFL server to provide the ML model identifier, the intermediate result and sample ID obtained in step 3.

6b.	[Conditional] The AF as VFL client sends the VFL training response to the NWDAF as VFL server by invoking Naf_VFLTraining_Notify service to provide the ML model identifier, the intermediate result and sample ID obtained in step 3.

7.	The NWDAF as VFL server aggregates intermediate results from VFL clients and computes intermediate training information (e.g. gradient information, loss information) per sample for updating its own local model and the models of VFL clients. The NWDAF as VFL server also determines whether the VFL training is converged, e.g. maximum number of iterations, or the result of loss function is lower than a threshold.

8a.	[Conditional] The NWDAF as the VFL server sends Nnwdaf_VFLTraining_Subscribe to the NWDAF as VFL client. The service operation includes the analytics ID, the allocated ML model identifier, the intermediate training information and sample ID. Each NWDAF as VFL client updates its own ML Model based on the intermediate traning information.

8b.	[Conditional] The NWDAF as the VFL server sends Naf_VFLTraining_Subscribe to the AF as VFL client AF. The Naf_VFLTraining_Subscribe message includes the analytics ID, the allocated ML model identifier, the intermediate training information and sample ID. Each AF as VFL client updates its own ML Model based on the intermediate traning information.

The steps 5-8 should be repeated until VFL training procedure is finished. The NWDAF as the VFL server sends a notification to VFL client(s) indicating the VFL training has been finshed.

# * * * * End of changes * * * *