SA WG2 Meeting #S2-164	S2-2408263

19 - 23 August, Maastricht, Netherlands	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5553 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility support by MWAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR2 |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the FS_VMR2 conclusion (clause 8.4 of 23.700-06), this CR proposes to add the Mobility support by MWAB to 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Mobility support by MWAB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Mobility support by MWAB is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.X.Y	Mobility support of UEs served by MWAB

#### 5.X.Y.1	UE mobility between a fixed cell and MWAB cell

When a UE is served by a MWAB-gNB, the existing mobility (IDLE and Connected mode) procedures is reused to handle UE's mobility between a MWAB-gNB and fixed cell

When the MWAB goes out-of-service (due to e.g. MWAB moves to an area where the MWAB is not allowed to operate as MWAB), the handling is as following.

-	For UEs in RRC_IDLE and RRC_INACTIVE state, procedure for cell (re-)selection as specified in TS 38.304 [50] for RRC_IDLE and RRC_INACTIVE is used between MWAB cell and other cells.

-	MWAB-gNB triggers the handover procedures to the neighbouring cells for UEs in RRC_CONNECTED state served by the MWAB.

#### 5.X.Y.2	UE mobility between MWAB cells

Similar to the behaviours described in clause 5.X.Y.1, UEs and NG-RAN use existing procedures as specified in TS 23.502 [3], or TS 38.304 [50] to handle the mobility between MBSR cells.

#### 5.X.Y.3	UE mobility when moving together with a MWAB cell

When a UE served by a MWAB cell and moved with MWAB,

-	for UEs in RRC_IDLE and RRC_INACTIVE state, it may observe change of TAC and/or cell IDs. This can trigger mobility registrations, as defined in TS 23.502 [3], if the new TAC is not in the TAI list in the RA.

-	for UEs in RRC_CONNECTED state, the HO procedure may be used if the new TAC is not in the TAI list in the RA.

Editor's note:	It needs to be confirmed by RAN WG3.

#### 5.X.Y.4	MWAB-UE mobility

When MWAB-UE moves in the Backhaul PLMN/SNPN, the existing mobility procedure is applied.

The MWAB-UE may re-establish a new backhaul PDU session for OAM access, N2, Xn and N3 interface if the original backhaul PDU cannot be used.

NOTE:	It assumed the SSC mode 1 is used for backhaul PDU session.

Next CHANGE

END OF CHANGES