3GPP TSG-WG SA2 Meeting #164 	S2-2408286

Aug. 19 – 23, 2024, Maastricht, NL	(revision of S2-240xxxx)

Source:	ZTE

Title:	KI#1: Conclusion for DualSteer

Document for:	Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract: This paper proposes conclusion for KI#1 DualSteer.

1. Discussion

Two types of DualSteer Device should be considered in this release. The following are examples for the two types of DualSteer Device:

DualSteer Type 1 device: One USIM/SUPI + two AS/NAS

DualSteer Type 2 device: two USIM/SUPI + two AS/NAS

NOTE 1: NAS1 and NAS2 can be combined into one NAS for some RATs combination, e.g. 5G NR and 5G Satellite

NOTE 2: The Dual Steer layer is used for DualSteer PDU Session only

The following table is our understanding how the DualSteer feature can work for both non-simultaneous transmission only and simultaneous transmission and for both DualSteer Type 1 device and DualSteer Type 2 device.

|  | DualSteer Type 1 Device(One USIM) | DualSteer Type 2 Device(Two USIM) |
| --- | --- | --- |
| support of non-simultaneous transmission only | The Device should support Dual Registration in both Access/RAT type. Dual Registration: The Device performs dual registration for the USIM/SUPI in both Access/RAT types DualSteer PDU session: The UE establishes DualSteer PDU session in one Access/RAT Type per URSP rule. If the device is registered in both Access/RAT Type, the device shall select one Access/RAT type to establish the DualSteer PDU session. How to select the Access/RAT type is based on local configuration. The SMF provides the DualSteer rule to the UE for the DualSteer PDU Session. The lower layer can notify the DualSteer layer on the Access/RAT type availability when the device is registerd/deregisted for the USIM/SUPI in the Access/RAT type. Based on the availability information and the DualSteer rule, the DualSteer layer in the UE determines whether to handover the DualSteer PDU Session towards the target Access/RAT type and trigger the device to perform handover. If the device decides to handover, it will transfer all PDU sessions to target Access/RAT Type. | For each USIM/SUPI, the Device should support Dual Registration in both Access/RAT type. Dual Registration: Each USIM/SUPI can be registered in both Access/RAT type if the Access/RAT type is available. DualSteer PDU session: The UE establishes DualSteer PDU session in one Access/RAT Type per URSP rule. If the device is registered in both Access/RAT Type for both USIM/SUPI, the device shall select one USIM/SUPI and one Access/RAT type to establish the DualSteer PDU session based on local configuration. The SMF provides the DualSteer rule to the UE for the DualSteer PDU Session. The lower layer can notify the DualSteer layer on the Access/RAT type availability when the device is registerd/deregisted in the Access/RAT type for both USIM/SUPI Based on the availability information and the DualSteer rule, the DualSteer layer in the UE determines whether to handover the DualSteer PDU Session towards the target Access/RAT type and trigger the device to perform handover. When both USIM/SUPIs are registered in the target Access/RAT type, the device needs to determine the target USIM/SUPI based on the local configuration If the device decides to handover, it will transfer all PDU sessions to target Access/RAT Type |
| support of simultaneous transmission | Not supported | For each USIM/SUPI, the Device should support Dual Registration in both Access/RAT type. Dual Registration: Each USIM/SUPI can be registered in both Access/RAT type if the Access/RAT type is available. DualSteer PDU session: The Device establish DualSteer PDU session in one access per URSP rule. If both Access/RAT types are available, the Device establishes DualSteer PDU Session over two Access/RAT types. The SMF provides the DualSteer rule to the UE for the DualSteer PDU Session. The lower layer can notify the DualSteer layer on the Access/RAT type availability when each USIM/SUPI is registered/deregister in the Access/RAT type. Based on the availability information and the DualSteer rule, the DualSteer layer in the UE determine whether to switch the traffic flow towards the target Access/RAT type |

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-54. The changes can be complementary to other proposals.

# * * * * First change * * * *

8	Conclusions

8.1	Conclusions for DualSteer

In this release, two types of DualSteer device should be considered.

-	DualSteer Type 1 device: One USIM/SUPI + two AS/NAS

-	DualSteer Type 2 device: two USIM/SUPI + two AS/NAS.

The two NAS can be combined into one NAS for some RATs combination, e.g. 5G NR and Satellite NR

For both types of DualSteer device, Dual registration for each USIM/SUPI shall be supported.

In case of non-simultaneous transmission, when both Access/RAT types are available, the DualSteer device needs to determine based on local configuration which Access/RAT type is used to establish the DualSteer PDU session. For Type 2 device the configuration also indicates which USIM/SUPI is preferred for the Access/RAT type.

There is an DualSteer layer over the NAS. The DualSteer layer is used for DualSteer PDU Session. The lower layer can notify the DualSteer layer on the Access/RAT type availability information when the USIM/SUPI is registered/deregistered in the Access/RAT type.

Based on the Access/RAT type availability information and the DuaSteer rule received from the network, the DualSteer layer determines the target Access/RAT type for the application traffic and request the lower layer to

-	handover the PDU sessions to target Access/RAT type for non-support of simultaneous transmission case, or

-	switch the application traffic to target Access/RAT type for simultaneous transmission case

In case of simultaneous transmission, when the target Access/RAT types are available for both USIM/SUPIs, the DualSteer uses the local configuration to determine which USIM/SUPI is preferred for the target Access/RAT type.

# * * * * End of changes * * * *