3GPP SA WG2 Meeting #164	S2-2408291

Maastricht, NL, 19-23 August 2024	(revision of S2-240xxxx)

# Source:		Lenovo

Title:	Energy Consumption estimation per specific UE / QoS flow using input from OAM and CHF

Document for:	Endorsement

Agenda Item:	19.4.1

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This paper proposes a way forward to estimate the network EC for specific UE (and/or per QoS flow, application) in the 5GC by using the input from OAM and CHF for KI#1 and KI#2.

# 1. Introduction/Discussion

According to the SA2/SA5 joint call minutes, SA2 is requested to prepare consolidate solutions on how to collect/calculate and what to do with energy consumption information in 5GC. This paper proposes a hopefully simplified way forward for conclusion in SA2 and the split of work between SA2 and SA5.

The calculation (or rather estimation) of the Energy Consumption (EC) in the network for a specific UE (and further finer granularity for specific UE) has been discussed within the FS_EnergySys study item. Various approaches have been proposed, e.g. considering the EC in the each gNB and UPF which have served the UE, or reporting of EC from the gNB to the 5GC, etc.  During the conclusion phase, it appeared difficult to reach a consensus among the companies in SA2.

One complexity of taking into account the EC of the gNBs and UPFs where the UE was served is that there needs to be a correlation of the data volume transmitted over each entity for any possible granularity of EC.  In order to avoid this complexity, this paper proposes to use the EC per Network Slice which is an available value in the OAM system. In this case the EC is a function of the 1) the total EC for a network slice (e.g. for a time unit like per day), 2) the used the data volume (DV) in the network slice (e.g. for a time unit like per day) and 3) the used data volume (DV) of the specific granularity.  By dividing the EC for a network slice by the DV of the network slice, the result indicates the EC for data unit. Then, by multiplying the EC for data unit with the data volume for a target granularity we get the EC for a target granularity.

The EC can be calculated on different target granularities as follows:

- On per general application level for a specific customer, i.e. the whole EC consumed to serve the traffic of an application for all UEs:

EC per application (in total) = (Total average EC for slice / DV slice) * DV total application.

- On per specific UE per application level, i.e. the EC consumed by a specific UE for the traffic of specific application:

EC per specific UE per application = (Total average EC for slice / Data slice) * DV of UE for App ID.

- On per specific UE level:

EC per specific UE = SUM for “i” [(Total average EC for slice#i / DV slice#i) * DV of UE slice#i].

where the following definition of the parameters apply:

- "Total average EC for slice" means the whole EC consumed for a network slice. This parameter can be obtained from the OAM system.

- "DV slice" means the whole data volume transmitted over a network slice. This parameter can be obtained from the CHF.

- "DV total application" means the whole data volume transmitted via the given network slice to/from the specific application or group of applications. This parameter can be obtained from the CHF.

- “DV of UE for App ID” means the data volume transmitted by the UE for the specific application or group of applications where the DV may be further provided in finer granularity like the data volume UE for the specific application via the given network slice. Such DV can be measure as the DV for the QoS flow serving the application data, where it is assumed that a distinct QoS flow is setup for the application service data flow, if the EC consumption should be estimated. This parameter can be obtained from the CHF.

- "DV of UE slice" means the data volume transmitted by the UE over the specific network slice. This parameter can be obtained from the CHF by adding the data volume transmitted by the UE over all PDU Sessions associated with a network slice.

The calculation of the EC can be performed in NF in the 5GC, e.g. ECF, which collects the parameter "Total average EC for slice" from the OAM system and all required DV parameters from the CHF.

It is assumed that the network establishes one or more dedicated service data flows and corresponding QoS flows for the data traffic of the one or more application associated with a service or 3rd party for which the energy-related data should be calculated (or estimated). For example, even if there is a data flow with the same QoS level, the network establishes a separate QoS data flow, or a sub-flow for the purpose of data reporting collection.

For KI#1, when the AF requests reporting for EC on any granularity, such information can be store in the UDR (as application or UE subscription data) and/or PCF in order to trigger the DV collection for the required granularity. Once the ECF is configured to estimate the EC on any granularity (e.g. by request from AF via NEF), the ECF subscribes with the OAM and CHF to receive the corresponding parameter values.  The ECF exposes the EC reports to the AF.

For KI#2, the PCF may subscribe with the ECF to monitor the EC for a specific granularity against a threshold. The ECF calculates the EC for the required granularity as described for KI#1. The ECF notifies the PCF when a subscribe EC event (e.g. a threshold) is reached.

The solution for KI#3 does not depend directly on the input from ECF of CHF. The 5GC can use information provided by OAM system as described in the current conclusions for KI#3.

Proposal 1: The EC estimation on various granularities is performed in an NF in the 5GC, e.g. ECF. The ECF collects the required parameters from the OAM system and from the CHF. After the ECF estimates the EC, the ECF can provide the result to AF (e.g. for KI#1) or to PCF (e.g. for KI#2 and KI#3).

Proposal 2: It is proposed that the OAM system provides the EC for a network slice to the 5GC.

Proposal 3: It is proposed that the CHF provides the DV for a required granularity to the 5GC. The granularities for DV are:

- DV slice;

- DV of UE slice;

- DV of UE for App ID

- DV total application

# 2. Conclusion

It is proposed that SA2 agrees the work split between SA2 and SA5 is according to Proposal 1, Proposal 2 and Proposal 3 above.

The conclusions clauses in the TR 23.700-66 may be updated according to the Proposal 1, Proposal 2 and Proposal 3.