3GPP TSG-SA2 Meeting #164	S2-2408293

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0470 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This paper includes the procedure for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B to support 5G ProSe Multi-hop Relay based on the agreed conclusions of the TR 23.700-03. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the procedure for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.3A.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

6.3.2.3A.2	5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B

Figure 6.3.2.3A.2-1 illustrates the procedure for 5G ProSe muti-hop UE-to-Network Discovery with Model B.

Figure 6.3.2.3A.2-1: 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B

1.	The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message which includes, additional to the parameters described in clause 6.3.2.3.3 of TS 23.304 [4], an indication that multi-hop relay is supported for a RSC and an initialized multi-hop number counter (e.g. determined according to QoS requirement or (pre-)configuration).

The Source Layer-2 ID is self-assigned by the Remote UE and the Destination Layer-2 ID is set to a Default Destination Layer-2 ID.

2.	The Solicitation message is propagated by the 5G ProSe Intermediate Relay(s) with (pre)configured RSC(s) matching the RSC in the message while updating the multi-hop counter until the maximum number of hops is reached, in which case the message is dropped. To avoid loops and allow the routing of the Discovery Response, each Intermediate Relay includes its User Info ID in the message.

The 5G ProSe UE-to-Network Discovery Solicitation message contains the Type of Discovery Message, Discoverer Info, the maximum number of hops or TTL and optionally Target Info. The Target Info may contain the User Info of UE-to-Network Relay and Intermediate Relay(s). The maximum number of hops can be a constant value or a variable similar to TTL which will be decreased by 1 per hop.

Per-hop QoS information or cumulative QoS information (e.g. cumulative delay between Remote UE up to the Intermediate Relay) may be added or updated in the 5G ProSe UE-to-Network Discovery Solicitation message sent by Intermediated Relay(s).

The Source Layer-2 ID is self-assigned by the Intermediate Relay and the Destination Layer-2 ID is set to a Default Destination Layer-2 ID.

3.	If the information contained in the Solicitation message matches the 5G ProSe UE-to-Network Relay configuration, the 5G ProSe UE-to-Network Relay(s) that receives the Solicitation message send a Response which is relayed back to the Remote UE by the Intermediate Relays. Additional information related to the Intermediate Relays and links of the path from which the Response message transits is added at each hop to facilitate path selection by the Remote UE, for example, per-hop QoS information or cumulative QoS information (e.g. cumulative delay between Remote UE up to the Intermediate Relay). The information can also be used to limit the number of propagated Response messages based on criteria such as the end-to-end QoS.

The Source Layer-2 ID is self-assigned by the UE-to-Network Relay and the Destination Layer-2 ID is set to the unicast Source Layer-2 ID of the received Relay Discovery Solicitation message from the Intermediate Relay.

If the number of User Info IDs of intermediate Relays included in the message has reached the maximum number of hops (or the TTL is 0), the Relay should drop the message. If the Target Info included in the received Solicitation does not contain the User Info ID of itself, the Relay may drop the message.

A 5G ProSe intermediate Relay sends a Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the User Info IDs of all intermediate Relays in the path.

The UE-to-Network send Discovery response with additional Aggregation indication per RSC. Optionally, the Aggregation level per RSC is also included.

4.	If one or more discovery responses are received, the 5G ProSe Remote UE selects the 5G ProSe UE-to-Network Relay and multi-hop path to reach it based on the information received. For example, the UE-to-Network Relay may choose the path based on e.g. the PC5 signal strength of each message received, hops to the Remote UE, the User Info IDs of intermediate Relays in the path, per-hop QoS information or cumulative QoS information, etc.

The Response message additionally contains the User Info IDs of intermediate Relays in the path.

The Source Layer-2 ID is self-assigned by the Intermediate Relay and the Destination Layer-2 ID is set to the unicast Source Layer-2 ID of the received Relay Discovery Solicitation message from the Remote UE.

* * * * End of Changes * * * *