3GPP TSG-SA2 Meeting #164	S2-2408298

# Maastricht, NL,  19th - 23rd Auguest, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1438 | rev | - | Current version: | V18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to support the conclusion of KI#1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA2 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.1 in 3GPP TS 23.700-77 concludes KI#1 on "Extensible IMS mechanism supporting IMS events in the context of DC communication". The corresponding normative work need to be implementd. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhance the IMS AS and HSS service to support IMS event subscriptoin and notification. - Provide the procedures of IMS event subscription and notification. - Update the IMS data channel architecture and reference point descriptoin. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No normative specification for conclusions of KI#1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.1.1, AA.1.2, AA.2.1.1, AA.2.1.X (new), AA.2.4.1, AA.2.4.2.1, AA.2.4.2.3 (new), AA.2.4.2.4 (new), AA.4.X (new), AC.2.1, AC.2.2.4, AC.2.2.X (new), AC.2.3, AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AA.1.1	Architectural Support

Figure AA.1.1-1 shows the architecture to support SBA interactions between IMS entities.

Figure AA.1.1-1: System Architecture to support SBA in IMS

NOTE 1:	The DCSF and MRF supports NF registration and discovery, but they do not provide services in this release of the specification.

NOTE 2:	Nimsas services are defined in the context of IMS Data Channel, see clause AA.2.4.

Figure AA.1.1-2 shows the architecture using the reference point representation.

Figure AA.1.1-2: System Architecture to support SBA in IMS in reference point representation

NOTE 3:	In this Release of the specification the SBI capable IMS AS supporting data channel services is collocated with the TAS.

NOTE 4:	In Figure AA.1.1-2, if an MRF supporting DC functions is used instead of a MF, then existing IMS interfaces (Mr' and Cr) are used for control of the DC media.

# * * * * Second change * * * *

## AA.1.2	Reference point to support SBA in IMS

Following reference points are realized by service-based interfaces in IMS:

N5:	Reference point between the PCF and an AF.

NOTE:	P-CSCF acts as an AF from PCF point of view. N5 Reference point is defined in TS 23.501 [93].

N70:	Reference point between an SBI capable I/S-CSCF and an SBI capable HSS.

N71:	Reference point between an SBI capable IMS AS and an SBI capable HSS.

DC1:	Reference point between an SBI capable IMS AS and DCSF.

DC2:	Reference point between an SBI capable IMS AS and MF.

N72:	Reference point between DCSF and an SBI capable HSS.

N73:	Reference point between an SBI capable HSS and NEF.

Nxx:	Reference point between an SBI capable IMS AS and NEF.

# * * * * Third change * * * *

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| imsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| imsUE Context | Registration | Request/Response | S-CSCF |
| Management | DeregistrationNotification | Subscribe/Notify | S-CSCF |
| (_ImsUECM) | Deregistration | Request/Response | S-CSCF |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
| ImsUE Authentication | Get | Request/Response | S-CSCF |
| ImsEventExposure (_ImsEE) | Subscribe | Subscribe/Notify | NEF, AS |
|  | Unsubscribe | Subscribe/Notify | NEF, AS |
|  | Notify | Subscribe/Notify | NEF, AS |

# * * * * Forth change * * * *

### AA.2.1.X	Nhss_ImsEventExposure (ImsEE) service

#### AA.2.1.X.1	Nhss_ImsEventExposure_Subscribe service operation

Service operation name: Nhss_ImsEventExposure_Subscribe

Description: This service operation enables the consumer NF to receive the subscribed event(s), or if the event(s) are already defined in HSS, then the subscription is updated.

Inputs, Required: List of Event Category IDs, Target of Event Reporting (Public Identity(s) of IMS subscriber(s)), Notification endpoint.

Event Category IDs include the following values:

-	IMS_Registration_Event_Category: the events in this category is reported by the HSS to the consumer NFs that the specific subscriber is registered or unregistered. The events include SUB_REGISTERED and SUB_DEREGISTERED.

-	IMS_Session_Event_Category: the events in this category is reported by the IMS AS when the session status and media of a specific IMS session change and has been defined in 3GPP Rel-18 TS 29.175 [12].

-	IMS_DC_Application_Event_Category: the events in this category is reported by the DCSF when a specific subscriber downloads some IMS data channel application from the DCSF.

If Target of Event Reporting is null, it indicates any IMS subscriber.

Inputs, Optional: Event Filter, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time.

Outputs, Required: Result indication and when the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available.

#### AA.2.1.X.2	Nhss_ImsEventExposure_Unsubscribe service operation

Service operation name: Nhss_ImsEventExposure_Unsubscribe

Description: This service operation enables the consumer NF to delete the subscription of an event(s) if already defined in the HSS.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Result indication.

#### AA.2.1.X.3	Nhss_ImsEventExposure_Notify service operation

Service operation name: Nhss_ImsEventExposure_Notify

Description: This service operation enables HSS to report the event(s) to the consumer NF.

Inputs, Required: Event ID, Notification Correlation Information, time stamp.

Inputs, Optional: Event information (defined on a per Event ID basis).

Outputs, Required: Result indication.

# * * * * Fifth change * * * *

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF, HSS, NEF |
|  | Unsubscribe | Subscribe/Notify | DCSF, HSS, NEF |
|  | Notify | Subscribe/Notify | DCSF, HSS, NEF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

# * * * * Sixth change * * * *

#### AA.2.4.2.1	General

Service description: This service enables the consumer to be notified about session events when served IMS subscribers take part in IMS sessions.

The following operations are available for this service:

-	Explicit subscription and unsubscription to receive session events.

NOTE:	The implicit subscription to receive session events used by the DCSF in Release-18 is also supported in further Releases.

-	Notifying IMS session control events of a specific IMS subscriber to NFs, e.g. DCSF, based on local configuration in the IMS AS.

IMS AS processing of the session event is paused until the consumer acknowledges reception of the session event notification. The consumer is given the possibility to apply session media control and policies by invoking the Nimsas_MediaControl service before giving IMS AS a session event notification acknowledgement.

If the consumer does not apply media control/policies to a session event, IMS AS continues processing and forwards the event as is (including the media state) to calling/called party.

# * * * * Seventh change * * * *

#### AA.2.4.2.3	Nimsas_SessionEventControl_Subscribe service operation

Service operation name: Nimsas_SessionEventControl_Subscribe

Description: This service operation enables the consumers NF to subscribe the IMS session events related to a specific subscriber, a group of subscrbers or any subscriber explicitly, or if the event(s) are already defined in IMS AS, then the subscription is updated.

Inputs, Required: List of Event Category IDs, Target of Event Reporting (Public Identity(s) of IMS subscriber(s)), Notification endpoint.

Event Category IDs include the following values:

-	IMS_Session_Event_Category: the events in this category is reported by the IMS AS when the session status and media of a specific IMS session change and has been defined in 3GPP Rel-18 TS 29.175 [12].

If Target of Event Reporting is null, it indicates any IMS subscriber.

Inputs, Optional: Event Filter, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time.

Event Filter contains the parameters which filter a specific subscriber or a group of IMS subscribers whose events need to be monitored, including:

-	Serving Number: it is the subscriber served by the DC AS.

-	Destination Number: it is the called party number and used to subscribe non-subscriber events e.g. an event like "a user has called number 800-123456" where 800-123456 is a DestionationNumber.

Outputs, Required: Result indication and when the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available.

# * * * * Eighth change * * * *

#### AA.2.4.2.4	Nimsas_SessionEventControl_Unsubscribe service operation

Service operation name: Nimsas_SessionEventControl_Unsubscribe

Description: This service operation enables the consumers NF to remove the subscription of an event(s) if already defined in the IMS AS.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Result indication.

# * * * * Ninth change * * * *

## AA.4.X	Service Based IMS AS Discovery and Selection

### AA.4.X.1	General

When an IMS event consumer NF selects an IMS AS, it shall ensure that the selected IMS AS instance supports the required IMS event categories. Optionally, it can discover the event categories supported by the specific IMS AS instances through the NRF.

### AA.4.X.2	IMS AS Registration

If an IMS AS supports NRF based discovery and selection, it shall register with the NRF using the Nnrf_NFManagement_NFRegister Request message as defined in 3GPP TS 23.502 [94].

The NF profile for the IMS AS includes all common information for SBI capable IMS entities plus the IMS event category it supported.

### AA.4.X.3	IMS AS Discovery and Selection

During the IMS procedure, an SBI capable IMS entity sends a Nnrf_NFDiscovery Request to the NRF as defined in 3GPP TS 23.502 [94] to discover the IMS event categories supported by the specific IMS AS instance.

# * * * * Tenth change * * * *

## AC.2.1	Architecture

Figure AC.2.1-1 shows the data channel architecture when using the service-based DC media function.

Figure AC.2.1-1: Architecture option of IMS supporting DC usage with MF

Figure AC.2.1-2 shows the data channel architecture when using the MRF.

Figure AC.2.1-2: Architecture option of IMS supporting DC usage with MRF

NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this Release.

# * * * * Eleventh change * * * *

### AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 or wit MRF via Mr'/Cr for data channel media resource management;

-	The IMS AS interacts with HSS for retrieving and storing DC enhanced IMS AS service specific data via N71/Sh.

-	The IMS AS interacts with the NEF for exposing data channel capabilities to the 3rd party AF/AS and notifying the 3rd party AF/AS of IMS events.

# * * * * Twelfth change * * * *

### AC.2.2.X	NEF

The NEF is enhanced to support the following functionalities:

-	The NEF exposes IMS events and IMS network capabilities to the 3rd party AF/AS.

IMS events and IMS network capabilities exposure can be provided an individual NEF instance, or by the NEF instance serving the 5GC network exposure simultaneously.

# * * * * Thirteenth change * * * *

## AC.2.3	Reference points

The following reference points are defined to support data channel service in IMS:

-	DC1:	Reference point between the DCSF and the IMS AS.

-	DC2:	Reference point between the IMS AS and MF.

-	DC3:	Reference point between the DCSF and NEF.

-	DC4:	Reference point between the DCSF and DC Application Server.

-	DC5:	Reference point between the DCSF and DCAR.

-	N72/Sc:	Reference point between the DCSF and HSS.

The following reference points are updated to support data channel signalling control in IMS:

-	N70/Cx/Dx:	Reference point between the CSCF and HSS.

-	N71/Sh:	Reference point between the IMS AS and HSS.

The following reference points are updated to support IMS events and IMS network capabilities exposure:

-	N33:	Reference point between the NEF and the 3rd party AF/AS.

The following reference points are defined for IMS events and IMS network capabilities exposure:

-	N73:	Reference point between the NEF and the HSS.

-	Nxx:	Reference point between the IMS AS and NEF.

The following reference points are defined for data channel media handling:

-	MDC1:	Reference point for transport of data channel media between data channel media function (either MF or MRF) and DCSF.

-	MDC2:	Reference point for transport of data channel media between data channel media function (either MF or MRF) and DC Application Server.

-	MDC3:	Reference point for transport of data channel media between DCSF and DC Application Server.

The following reference point is updated to support data channel media handling:

-	Mr'/Cr:	SIP based reference point between IMS AS and MRF.

# * * * * Forteenth change * * * *

# AC.X	Support of IMS Network Capabilities Exposure

## AC.X.1	General

This clause describes the enhancements to support IMS network capabilities exposure, including IMS event subscription/notification.

## AC.X.2	IMS event subcription/notification mechanism

The IMS events can be classified into the following categories:

-	IMS registration event category: it is reported by the HSS to the consumer NFs that the specific subscriber is registered or unregistered.

-	IMS session event category: it is reported by the IMS AS when the session status and media of a specific IMS session change and has been defined in 3GPP Rel-18 TS 29.175 [12].

-	IMS data channel application event category: it is reported by the DCSF when a specific subscriber downloads some IMS data channel application from the DCSF.

NOTE:	IMS data channel application event category is not supported in Release 19.

The target of Event Reproting may indicate a specific subscriber identified by the subscriber ID, a group of subsribers filtered by the parameters contained in the subscribe request, or any subscriber.

These IMS events can be subscribed by either the NF inside IMS network, or the NF outside IMS network. The latter shall subscribe and be notified of IMS events via the NEF.

## AC.X.3	IMS event subscription/notification procedures

### AC.X.3.1	IMS AS registration information storage and retrieval

When the third-party Registration for a specific subscriber is successful, the IMS AS shall store it address into the subscriber data of HSS. Figure AC.X.3.2-1 depicts this procedure.

Figure AC.X.3.2-1: IMS AS Registration Information storage procedure

1.	The IMS AS instance sends a Nhss_ImsSDM_Update request to the HSS serving the specific UE to create or update the IMS AS registration information. The request contains the UE's identity and IMS AS registration information (address of the IMS AS instance).

2.	On success, the HSS updates the IMS AS registration information resource and returns a successful response. Otherwise, the HSS returns an error response.

The IMS event consumer NF can retrieve the address of the IMS AS instance serving the specific UE from the HSS. Figure AC.X.3.2-2 depicts this procedure.

Figure AC.X.3.2-2: IMS AS Registration Information retrieval procedure

1.	The NF sends a Nhss_ImsSDM_Get request to the HSS serving the specific UE to retrieve the IMS AS registration information. The request contains the UE's identity and the data type of IMS AS registration information (address of the IMS AS instance).

2.	On success, the HSS returns the IMS AS registration information in a successful response. Otherwise, the HSS returns an error response.

### AC.X.3.2	Subscription/Notification procedure of IMS registration events

The procedure depicted in Figure AC.X.3.2-1 is used by the NF outside IMS network to subscribe or unsubscribe to the IMS registration events of a specific subscriber. The NEF need to repeat the subscription steps 3-4 to send Nhss_ImsEventExposure_Subscribe/Unsubscribe Request message to to the HSS instances flitered by the parameters contained in the Request message or all the HSS instances in the network, if the NF subscribes or unsubscribes to the IMS registration events of a group of subscribers or any subscriber.

Cancelling is done by sending Nnef_ImsEventExposure_Unsubscribe request identifying the subscription to cancel. The notification steps 6-9 are not applicable in cancellation case.

Figure AC.X.3.2-1: Subscription/Notification procedure of IMS registration events

1.	The NF sends Nnef_ImsEventExposure_Subscribe/Unsubscribe Request message to the NEF for subscribing or unsubscribing to IMS registration events of the specific subscriber(s). The NF identifies the events by Event Category IDs and provides the associated notification endpoint of the NF.

2.	The NEF may query the NRF to find the HSS serving the specific subscriber(s).

3.	The NEF sends Nhss_ImsEventExposure_Subscribe/Unsubscribe Request message to the HSS. The NEF indicates the IMS registration events by Event Category IDs and provides the associated notification endpoint of the NEF.

4.	The HSS confirms with Nhss_ImsEventExposure_SubScribe/Unsubscribe Response message to the NEF.

5.	The NEF confirms with Nnef_ImsEventExposure_Subscribe/Unsubscribe Response message to the NF.

6.	The HSS detects the monitored event occurs and sends the event report, by sending Nhss_ImsEventExposure_Notify Request message to the associated notification endpoint of the NEF.

7.	The NEF sends the event report to the NF by sending Nnef_ImsEventExposure_Notify Request message to the associated notification endpoint of the NF.

8.	The NF confirms with Nnef_ImsEventExposure_Notify Response message to the NEF.

9.	The NEF confirms with Nhss_ImsEventExposure_Notify Response message to the HSS.

### AC.X.3.3	Subscription/Notification procedure of IMS session events

The procedure depicted in Figure AC.X.3.3-1 is used by the the NF outside the IMS network to subscribe or unsubscribe to the IMS session events of a specific subscriber. The NEF need to repeat the subscription steps 3 and 7 to send Nhss_ImsEventExposure_Subscribe/Unsubscribe Request message to the HSS instances flitered by the parameters contained in the Request message or all the HSS instances in the network, if the NF subscribes or unsubscribes to the IMS session events of a group of subscribers or any subscriber.

Cancelling is done by sending Nnef_ImsEventExposure_Unsubscribe request identifying the subscription to cancel. The notification steps 9-12 are not applicable in cancellation case.

Figure AC.X.3.3-1: Subscription/Notification procedure of IMS session events

0.	The IMS AS instance has registred the IMS events it supports to the NRF as defined in clause AA.4.X. The IMS AS instance has stored its address in the serving subscriber data in the HSS as defined in clause AC.X.3.1 after the 3rd party registration is completed.

1.	The AF sends Nnef_ImsEventExposure_Subscribe/Unsubscribe Request message to the NEF for subscribing or unsubscribing to IMS session events. The AF identifies the events by Event Category IDs and provides the associated notification endpoint of the AF.

2.	The NEF may query the NRF to find the HSS serving the specific subscriber.

3.	The NEF sends Nhss_ImsEventExposure_Subscribe/Unsubscribe Request message to the HSS. The NEF indicates the IMS session events by Event Category IDs and provides the associated notification endpoint the NEF.

4.	The HSS finds the address of the IMS AS instance serving the specific subscriber in the subscriber data and may query the NRF to check whether the IMS AS instance supports the required IMS events.

The HSS will store the subscription information in Nhss_ImsEventExposure_Subscribe Request message and skip the subscription steps 5-6, if the specific subscriber is unregistered yet and there is no record of the serving IMS AS instance. After the specific subscriber is registered, the HSS will execute the subscription steps 5-6.

If the HSS detects that the IMS AS instance serving the specific subscriber is changed, it will unsubscribe the old IMS AS instance and subscribe to the new IMS AS instace.

If the IMS AS instance does not support the required IMS events, the steps 5-6 will be skipped and an error code will be returned in the steps 7-8.

5.	The HSS sends Nimsas_SessionEventControl_Subscribe/Unsubscribe Request message to the IMS AS instance serving the specific subscriber for subscribing or unsubscribing to IMS session events of the specific subscriber. The HSS identifies the events by Event Category IDs and provides the associated notification endpoint of the NEF so the NEF can receive the notification from the IMS AS instance directly.

6.	The IMS AS instance confirms with Nimsas_SesionEventControl_SubScribe/Unsubscribe Response message to the HSS.

7.	The HSS confirms with Nhss_ImsEventExposure_Subscribe/Unsubscribe Response message to the NEF.

8.	The NEF confirms with Nnef_ImsEventExposure_Subscribe/Unsubscribe Response message to the NF.

9.	The IMS AS instance detects the monitored event occurs and sends the event report, by sending Nimsas_SessionEventControl_Notify Request message to the associated notification endpoint of the NEF.

10.	The NEF sends the event report to the NF by sending Nnef_ImsEventExposure_Notify Request message to the associated notification endpoint of the NF.

11.	The NF confirms with Nnef_ImsEventExposure_Notify Response message to the NEF.

12.	The NEF confirms with Nimsas_SessionEventControl_Notify Response message to the IMS AS.

# * * * * End of changes * * * *