3GPP TSG-WG SA2 Meeting #164	S2-2408333

# Maastricht, Netherlands, Aug 19 – 23, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0556 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF enhancements for Direct AI/ML based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in clause 8.1 of TR 23.700-84: Principle #1: The LMF is enhanced to perform location calculation based on ML model. The interaction between LMF and MTLF is only for model provisioning and possibly, based on coordination with RAN WGs, for data collection for training. NOTE 1: When receiving the request for a UE location, the LMF selects an appropriate method to determine the UE location and may select AI/ML positioning as method. Principle #2: LMF is enhanced to perform model training for AI/ML based Positioning, the trigger for data collection and for model training in LMF is up to implementation. Principle #4: LMF or MTLF performs model performance monitoring for AI/ML based Positioning. - Principle #4.1 The result of model performance monitoring may trigger the LMF to change the positioning method, e.g. from AI/ML based positioning to the legacy positioning, or vice versa. - Principle #4.2 The result of model performance monitoring may trigger ML model retraining in the training entity. - Principle #4.3 The detailed procedure and service for this model performance monitoring will be decided in normative phase. Based on the above conclusions for KI#1, the LMF related functionalities and service operations enhancements are proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions that LMF performs location calculation based on ML model, ML model training and ML model performance monitoring for Direct AI/ML based Positioning. Add new LMF service operations and procedures to support providing the Location Measurement data to the consumer NF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancements to LMF to perform location calculation based on ML model, ML model training and ML model performance monitoring for Direct AI/ML based Positioning, and Location Measurement data exposure are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.X (new), 4.4.13, 5.X (new), 6.21.1, 6.21.X (new), 6.X (new), 8.3.1, 8.3.2.1, 8.3.2.X (new), 8.3.2.Y (new), 8.3.2.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR 1170 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.3.X	Network Data Analytics Function, NWDAF

NWDAF may collect data from LMF to perform ML model training or ML model performance monitoring for Direct AI/ML based positioning.

NWDAF may provide ML model to LMF for performing location calculation.

NWDAF may provide Location Accuracy analytics to LMF.

# * * * * Next Change * * * *

4.4.13	NL8 Reference Point

The NL8 reference point supports LMF to receive location related analytics from NWDAF, data collection request sent by NWDAF to an LMF, or ML model provisioning from NWDAF to an LMF, as defined in TS 23.288 [37].

# * * * * Next Change * * * *

5.X	Support of Direct AI/ML based Positioning

LMF may support Direct AI/ML based Positioning. When receiving the request for a UE location, the LMF selects an appropriate method to determine the UE location and may select Direct AI/ML based Positioning as method. The location services need to be enhanced as following for the Direct AI/ML based Positioning:

-	Support of location calculation based on ML model, the ML model may be provided by the NWDAF containing MTLF as described in clause 6.21.X or available at the LMF itself.

NOTE 1:	The LMF can support ML model training for Direct AI/ML based Positioning. The data collection and model training in LMF is up to implementation.

NOTE 2:	The LMF can support ML model performance monitoring for Direct AI/ML based Positioning if the ML model is trained by the LMF itself. The result of ML model performance monitoring may trigger the LMF to change the positioning method, e.g., from AI/ML based positioning to the legacy positioning, or vice versa. The result of ML model performance monitoring may trigger the LMF to perform ML model retraining based on implementation.

# * * * * Next Change * * * *

6.21.1	General

This clause specifies procedures for NF in the LCS system to interact with NWDAF for data analytics. General framework for NF in the LCS system to interact with NWDAF for data analytics refers to the clauses 6.1.1 and 6.1.2 of TS 23.288 [37].

Clause 6.21.2 describes the procedures of Location Accuracy Analytics retrieved by LMF.

Clause 6.21.3 describes the procedures of UE Mobility Analytics retrieved by AMF.

Clause 6.21.X describes the procedures of ML model for Direct AI/ML based Positioning retrieved by LMF.

NOTE:	In this release of specification, only AMF and LMF can be NF service consumers to NWDAF.

The procedure in Figure 6.21-1 can be used by a 5GC NF involved in 5GC location services to get optional assistance from NWDAF as shown below.

Figure 6.21-1: Procedure for NWDAF assistance to location services

1.	The AMF may invoke Nlmf_Location_DeterminationLocation service operation towards the LMF to request the current location of the UE.

2.	Assistance from NWDAF for location services may be requested from the LMF and/or AMF.

3.	The LMF may request ML model from NWDAF by invoking Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request service operations as defined in TS 23.288 [37].

4.	The NWDAF provides the ML model to the LMF by invoking Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelInfo_Request Response service operations.

5.	The LMF may trigger UE positioning. The LMF may select Direct AI/ML based Positioning as method to perform location calculation.

6.	The LMF may return the location estimate to AMF as specified in clause 8.3.2.

# * * * * Next Change * * * *

6.21.X	ML model Retrieval by LMF

LMF may subscribe/request ML model from NWDAF to perform location calculation.

The procedure to subscribe/request ML model by LMF is referred to clause 6.2A of TS 23.288 [37].

# * * * * Next Change * * * *

6.X	Procedures of Location Measurement data exposure for Direct AI/ML based Positioning

6.X.1	General

This clause specifies procedures for NWDAF containing MTLF to collect Location Measurement data from LMF for ML model training or ML model performance monitoring for Direct AI/ML based Positioning.

6.X.2	Location Measurement data collection

Figure 6.X.3-1: Procedure of Location Measurement data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for Direct AI/ML based Positioning based on the request from LMF or internal trigger.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF, the service operation includes an AoI. If the NWDAF wants to collect the Location Measurement data of PRUs, the NWDAF also includes a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

3.	The NWDAF collects Location Measurement data from LMF by invoking Nlmf_Location_DataExposureSubscribe service operation, the NWDAF includes an AoI to request the Location Measurement data from LMF. The NWDAF may also include an PRU indication to indicate obtaining the PRU information if the NWDAF wants to collect the Location Measurement data of PRUs.

4.	The LMF collects UE IDs from AMF via the AMF event exposure service with AoI as filtering criteria.

5.	The LMF uses the procedure defined in clause 6.11 to obtain the Location Measurement data of the UEs. The LMF first filters out PRUs from the UE IDs collected from AMF if the PRU indication is indicated in step 3.

NOTE:	Location Measurement data is defined by RAN WGs for Direct AI/ML based Positioning, and SA WG2 will align with RAN WGs.

6.	The LMF sends the collected Location Measurement data to the NWDAF by invoking Nlmf_Location_DataExposureNotify service operation, then the NWDAF trains the ML model based on the Location Measurement data. The Location Measurement data includes the PRU location measurement(s) and associated PRU known location if the PRU indication is indicated in step 3.

# * * * * Next Change * * * *

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
|  | MeasurementData | Request/Response | LMF |
|  | UPConfig | Request/Response | AMF |
|  | UPSubcribe | Subscribe/Notify | AMF |
|  | UPNotify |  | AMF |
|  | DataExposureSubscribe | Subscribe/Notify | NWDAF |
|  | DataExposureNotify |  | NWDAF |
|  | DataExposureUnSubscribe |  | NWDAF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |

# * * * * Next Change * * * *

8.3.2.1	General

Service description: This service enables an NF to request location determination for a target UE. The following are the key functionalities of this NF service.

-	Allow the consumer NF to request the current geodetic and optionally local and/or civic location of a target UE.

-	Allow the consumer NF to subscribe/unsubscribe the geodetic and optionally local and/or civic location of a target UE for some certain events.

-	Allow the consumer NF to get notified about the geodetic and optionally local and/or civic location of a target UE when some certain events are detected.

-	Allows the consumer NF to cancel location event reporting for a target UE.

-	Allows the consumer NF to transfer location context information for location event reporting for a target UE.

-	Allows the consumer NF to request the PRU location measurement.

-	Allows the consumer NF to request to set up, modify or terminate a secure LCS-UP connection for a target UE.

-	Allows the consumer NF to get notified about status or modification of a secure LCS-UP connection for a target UE.

-	Allows the consumer NF to request the Location Measurement data used for ML model training or ML model performance monitoring for Direct AI/ML based Positioning.

The events to trigger location estimation notification are defined in clause 4.1a.5.1

# * * * * Next Change * * * *

8.3.2.X	Nlmf_Location_DataExposureSubscribe service operation

Service operation name: Nlmf_Location_DataExposureSubscribe

Description: Allow the consumer NF to subscribe the Location Measurement data used for ML model training or ML model performance monitoring for Direct AI/ML based Positioning.

Input, Required: Notification Target Address, Notification Correlation ID.

Input, Optional: AoI, PRU indication, Subscription Correlation ID (in the case of modification of the subscription).

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

# * * * * Next Change * * * *

8.3.2.Y	Nlmf_Location_DataExposureNotify service operation

Service operation name: Nlmf_Location_DataExposureNotify

Description: Allow the consumer NF to get notification about the Location Measurement data used for ML model training or ML model performance monitoring for Direct AI/ML based Positioning.

Input, Required: Notification Correlation ID.

Input, Optional: Location Measurement data (e.g., PRU location measurement(s) and associated PRU known location), Subscription Correlation ID (this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation ID by the LMF).

NOTE:	Location Measurement data is defined by RAN WGs for Direct AI/ML based Positioning, and SA WG2 will align with RAN WGs.

Output, Required: Operation execution result indication.

Output, Optional: None.

# * * * * Next Change * * * *

8.3.2.Z	Nlmf_Location_DataExposureUnSubscribe service operation

Service operation name: Nlmf_Location_DataExposureUnSubscribe

Description: Allow the consumer NF to cancel subscription to Location Measurement data used for ML model training or ML model performance monitoring for Direct AI/ML based Positioning.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

# * * * * End of changes * * * *