3GPP TSG-WG SA2 Meeting #164	S2-2408354

Maastricht, NL, 19th Aug – 23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2 Conclusion: Update of conclusion of KI#2

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: It is proposed to update KI#2 conclusion, and further clarify the subscription and policy control aspect.

# 1. Discussion

Several aspects to be clarified in this document.

## 1.1	Policy decision by PCF

To make policy decision, we think the following aspects need to be clarified by PCF:

1.	Input: PCF may take into account the following information for making policy decision. 1) subscription of the UE regarding EE (i.e., whether the UE is subject to energy related execution), 2) energy related information (i.e., Energy consumption, renewable energy per specific UE) from the functionalities defined by KI#1. Other possible inputs can be discussed in the normative phase, e.g., analytic information from NWDAF (this may also be relied on the discussion on KI#3).

2.	Output: There should be no enhancement on the output, i.e., no new policy regarding energy will be introduced and solutions only impact internal behaviour (of PCF). By now we see current parameters are sufficient for the control purpose. Nevertheless, it does not matter very much that which exact policy parameter (e.g., RFSP, UE-AMBR) will be determined based on the extra input on energy as mentioned in the 1st bullet.

3.	Scenario: As mentioned in 2, there will be no enhancement on AM/SM policy, even though there will be extra factors for PCF for making the decision. In other words, AMF/SMF will not be affected regarding the policies for EE. Note that the AM/SM policy control will not affect the non-best-effort traffic, and PCF performs the enforcement with almost no standardization efforts.

## 1.2	Subscription and energy control impact on charging subsystem

On the control of maximum energy consumption and the energy credit:

-	As explained in our submitted (but not handled) S2-2406501 in the last meeting, we consider the "Energy related limit control" for the best-effort traffic is needed to accommodate the requirement from SA1 in 22.261 and CHF is the better place to trigger the control in addition to charging, given that:

-	CHF is the centralized NF for controlling the energy-related limits of a UE, and

-	CHF can reuse the online-charging mechanism (and the existing procedures).

-	Sol #22 proposes EC requirements as part of energy related UE subscription information.

-	EC requirements specify the requirements of energy related limits (i.e., UE's maximum energy consumption in a specific period of time or space, or UE's maximum energy credit limit), and the EC requirements can be part of energy saving subscription information of the UE.

To control the energy-related limits of a UE, it is proposed that based on the energy related information (i.e., the granularity of PDU Session/QoS Flow) and the UE energy saving subscription data, CHF triggers the adjustment of the parameters used for session management.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66.

# * * * * First change * * * *

## 8.2	Conclusion for Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria

The following principles for enhancements on subscription and policy control are agreed for the normative work:

-	The following information is stored as part of the subscription data in the UDM/UDR:

-	Energy saving subscription information per UE to assist the network to perform energy saving strategies for the UE.

-	The PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions (reusing the existing parameters) based on operator policy.

NOTE 1:	Further details including the input parameters and the affected policies for policy control will be defined during the normative work.

-	The CHF, based on the energy related information in PDU Session/QoS Flow granularity and the energy saving subscription information of the UE, performs charging and triggers the adjustment of the parameters used for session management.

NOTE 2:	The interaction for PCF with CHF for policy control and charging (e.g. how the PCC rules are determined based on energy criteria) will be considered in the normative phase, based on collaboration with SA WG5.

# * * * * End of changes * * * *