3GPP TSG-WG SA2 Meeting #164	S2-2408359

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | XXXX | rev | X | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EC aware slice admission control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | dummy |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR is to implement EnC aware slice admission control as proposed in pCR…. (using solution #30 in TR 23.700-66 as the baseline) |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are: - Provide a solution for slice level EnC credit control |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without these changes, the Rel. 19 feature on slice level EnC credit control would not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * * (all new)

## 5.X	Support of Energy System

### 5.X.Y	Network slice Admission control based on slice energy consumption

#### 5.X.Y.1	General

The existing NSAC mechanism can be enhanced to support the following based on the energy consumption information per S-NSSAI, per PDU session:

-	Adjustment on maximum number of UEs/PDU sessions of the requested S-NSSAI if allowed by the SLA.

-	Determination on accept/reject a new PDU session of requested S-NSSAI according to operator policy.

NOTE:	Based on operator policy, there could be no EnC related admission rejection case but multiple EnC related admission acceptance case (e.g., with QoS constraints).

-    Notify the SMF/AMF whether a certain energy related threshold is exceeded.

NOTE:	The source of energy consumption information can be EECF/NWDAF/OAM, which will be updated per progress of the work.

Based on the feedback from the NSAC, SMF may determine to establish, modify, release a PDU session of requested S-NSSAI based on the related policies using existing procedures.

EECF collects energy related information used for slice level energy consumption control and provides the control of energy saving in network slice granularity.

For above methods, the following procedures apply:

-	The NSACF receives the current level of energy consumption analytics or measurements, performs admission control based on energy consumption threshold set according to agreement with the network slice customer, and notify the SMF/AMF whether a certain energy rlated threshold is exceeded.

-	Optionally, the EECF sends the adjustment information to NSACF based on energy consumption threshold and and energy consumption information, indicating NSACF to adjust the maximum number of UEs and maximum number of PDU Sessions.

NOTE:	Energy consumption threshold(s) can be configured at the EECF and NSACF.

NOTE:	In this release this feature only supports non roaming case with centralized NSACF architecture.

Editor's note:	The name and functionaility of EECF to be adjusted according to the results of KI#1 discussion.

#### 5.X.Y.2	Network slice Admission control based on energy threshold

The NSACF keeps track of the current energy consumption per network slice and check whether a certain energy consumption threshold is exceeded by the network slice. When an event related to a UE causes the current number of PDU sessions established within the network slice is to increase, the NSACF checks whether a certain energy consumption threshold for that network slice has already been reached and if it has, the NSACF applies admission control policies. The admission control policies can be one of the following:

- If the maximum energy credit limit is exceeded for the slice, do not update the number of PDU session and notify the SMF that the maximum energy credit limit is reached.

- If an energy credit threshold is reached but below the maximum energy credit limit, update the number of PDU session and notify SMF about the reached energy credit threshold.

Such energy credit threshold can be related to an admission with a reduced QoS (e.g., lower bandwidth, longer latency or degraded AQP level) according to the SLA.

The anchor SMF triggers a request to NSACF for energy consumption control per network slice control during PDU session establishment/release procedures in clauses 4.3.2 and 4.3.4 of TS 23.502 [3].

# * * * * End of changes * * * *