3GPP TSG-WG SA2 Meeting #164	S2-2408381

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3806 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Regenerative-based satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 5GSAT_Ph3 introduces support of Regenerative-based satellite access which has impacts to S1 connection management and CN functions determining it is use based on RAN node IDs (e.g. by the MME or PCRF). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the impact that regenerative-based satellite access brought to S1 connection management and note how CN functions can determine its use. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regenerative-based satellite access operation is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.1, 4.13.2, 4.13.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.13	Introduction of satellite support for Cellular IoT

### 4.13.1	General

This clause describes the functionality for supporting Cellular IoT over satellite access, including transparent satellite payloads and regenerative satellite payloads. Support for WB-E-UTRAN, NB-IoT and LTE-M satellite access is specified in TS 36.300 [5].

# * * * * Second change * * * *

### 4.13.2	Support of RAT types defined in EPC for satellite access

In the case of satellite access with WB-E-UTRAN, NB-IoT or LTE-M, the RAT Types values "WB-E-UTRAN(LEO)", "WB-E-UTRAN(MEO)", " WB-E-UTRAN(GEO)", " WB-E-UTRAN(OTHERSAT)", "NB-IoT(LEO)", "NB-IoT(MEO)", "NB-IoT(GEO)", "NB-IoT(OTHERSAT)", "LTE-M(LEO)", "LTE-M(MEO)", "LTE-M(GEO)" and "LTE-M(OTHERSAT)" are used in EPC to distinguish the different WB-E-UTRAN, NB-IoT and LTE-M satellite access types.

In order to enable efficient enforcement of mobility restrictions:

-	Cells of each NB-IoT satellite RAT type (NB-IoT(LEO), NB-IoT(MEO), NB-IoT(GEO) or NB-IoT(OTHERSAT)) need to be deployed in TAs that are:

-	different from TAs for other NB-IoT satellite RAT types; and

-	different from TAs supporting terrestrial NB-IoT RAT type; and

-	different from TAs for WB-E-UTRAN satellite RAT types; and

-	different from TAs for WB-E-UTRAN terrestrial RAT types.

The MME may initiate Detach of the UE when an S1 UE Context Release Request is received with Cause indicating the release is requested due to a UE using satellite access moved out of PLMN serving area, as specified in TS 36.413 [36].

NOTE X:	There is no differentiation of whether the RAT type is a transparent satellite payload or a regenerative satellite payload. The e.g. Global RAN Node IDs associated with satellite payload can be used to determine whether the UE is accessing via transparent or regenerative satellite payload, if needed, by e.g. the MME, PCRF, etc.

# * * * * Third change * * * *

#### 4.13.x	S1 connection management for regerative satellite payload

Editor's note:	When the eNB leaves the service area of an MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3.

# * * * * End of changes * * * *