SA WG2 Meeting #164	S2-2408410

August 19 – 23, 2024, Maastricht, NL

Source:	China Telecom, vivo, OPPO, CATT, Xiaomi

Title:	Conclusion for KI# 1.1

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS/Rel-19

Abstract of the contribution: This paper proposes a conclusion for key issue #1.1 of the FS_MASSS.

1	Discussion

This paper proposes the conclusion for Key Issue #1.1 of the FS_MASSS.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V1.0.0.

* * * * Start of Changes (all new text) * * * *

8.1	Conclusions for DualSteer

### 8.1.X	Conclusion for Key Issue #1.1

It is concluded that:

1)	The Subscription Data stored in UDM/UDR needs to be enhanced to include the following DualSteer related parameters:

-	The linked SUPI information

-	The DualSteer authorization information

- per SUPI level, e.g. whether DualSteer is activated or not. The DualSteer authorization information indicates DualSteer is activated after successful registration of two linked SUPIs separately and verification that they are in same DualSteer device.

-	 per session level, e.g. whether DualSteer traffic switching is allowed or not for each DNN/S-NSSAI.

2)	UE context in SMF subscription data of source SUPI is used to select the SMF of the target PDU Session for DualSteer established by target SUPI.

* * * * End of Changes * * * *