3GPP SA WG2 Meeting #164	S2-2408412

Maastrich, NL Aug 18 – 23, 2024           	(revision of S2-240xxxx)

Source: 	NTT DOCOMO

Title:	Resolving the Editor's Note for KI#4

Document for: 	Approval

Agenda Item: 	19.3.1

Work Item / Release:    FS_XRM_Ph2 / Rel-19

Abstract of the contribution: Conclusions for KI#4.

# Discussion

Clause 8.4 has one open Editor's Note regarding the issue whether the UE needs to indicate support for the new "additional packet filter" (and the SMF provides the additional packet filter only when the UE has indicated the support), or the legacy UE should ignore the additional packet filter if provided by the network.

Editor's note:	Whether there is a need for a new UE capability indication for UL is FFS (e.g. for legacy UE handling and network handling).

The background of the issue is that the legacy UE, i.e., a pre-R19 UE, is not able to support the additional new type of Packet Filter.

Figure 1: Possible QoS Flow scenarios for mixed traffic

Figure 1 shows the possible QoS Flow scenarios for a mixed traffic with 3 media flows (A, B, C) each having different QoS requirements. Media flow B has the strongest QoS requirements in terms of delay budget and error ratio, followed by media flow A, while media flow C has the weakest QoS requirements.

If the UE is not enhanced with the additional new type of Packet Filter, the UE is not able to separate the media flows in UL direction. Consequently, the UL traffic of all media flows will be transferred in a single QoS Flow and that QoS Flow needs to have a 5QI according to the most stringent QoS requirements of any of the media flows, as otherwise, the QoS requirements would not be fulfilled for that media flow (the other media flows will then be treated in a better way as well but this should not be a problem, at least not for the application).

Therefore, it’s proposed that the PCF selects the media flow with the most stringent QoS requirements (delay budget and error ratio) and sets the corresponding PCC rule with the highest precedence. As stated in NOTE 4 in clause 8.4, it is expected that CT1 designs the NAS protocol in a backward compatible manner, meaning that the legacy UE should ignore the additional packet filter of the QoS rule, and the legacy UE uses the legacy packet filter of the QoS rule with the highest precedence to map the mixed media flows to the corresponding QoS Flow.

Note that we assume that the CN is enhanced with the mixed traffic feature (i.e. PCF, SMF and UPF support the additional new type of Packet Filter) and therefore, the PCC will always generate at least one separate PCC rule for each media flow so that the policy instructions (e.g. for charging, policing, traffic handling, …) can be given on a per media flow basis, regardless of whether the media flows are transferred with the same QoS Flow or not (as the UPF can always separate the media flows).

The conclusion description should be updated accordingly so that the normative work can start. In addition, for better readability, the conclusion description is restructured a bit so that the UL and DL direction are documented separately (where necessary).

If seen necessary, SA2 can send an LS to CT1 to ask feedback on this question, and such capability indicator can be defined in normative phase based on the feedback by CT1. The LS out to CT1 is drafted in S2-2408416.

# Proposal

It is proposed to capture the following changes in XRM Ph2 TR 23.700-70.

*** First change ***

## 8.4	Agreements for Key Issue #4

The following aspects are concluded as principles for normative work:

1.	The AF may provide QoS requirements for media flows and additional packet filters as part of "AF session with required QoS". The detectable media flows that require different QoS treatment are mapped to distinct QoS Flows. If the AF does not provide any QoS requirement for the media flows or it provides the same QoS requirement, then the media flows can be mapped to the same QoS Flow. For the multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter, the AF may provide additional Packet Filter in the "AF session with required QoS" procedure to the NEF/PCF.

2.	In order to unique identify each media flow, the additional packet filter is used together with the legacy packet filter. The additional Packet Filter is used to differentiate the media flow among multiple media flows that share the same legacy packet filter both in uplink (in UE) and downlink (in UPF) in the following case:

a)	For the media flows that are transported in (S)RTP, (S)RTCP and other associated protocols that are multiplexed into a single UDP/IP traffic flow as specified in IETF RFCs 5761, 5764, 7983, 8872, 8843 and 9443. The additional Packet Filter SHALL be the Synchronization Source (SSRC), Payload Type (PT) and RTP-M header fields.

3.	The PCC rules are enhanced to support the additional new type of Packet Filter along with legacy packet filter.

-	The PCF generates a PCC rule per media flow using  the additional Packet Filter along with legacy packet filter (assuming here that the UPF supports the new type of Packet Filter).

-	PCF sets the QoS parameters according to the individual QoS requirements of the respective media flow. The PCF selects the media flow with the most stringent delay budget and error ratio requirements and sets the corresponding PCC rule with the highest precedence.

4.	The SMF binds the PCC rules to QoS Flows (based on existing principles) and provides N4 Rules to the UPF and QoS rules to the UE with the following enhancements:

-	For both directions, the received additional new type of Packet Filter along with legacy packet filter are provided to the UPF in the N4 rules. The PDI in PDR is extended to support the additional new type of Packet Filter.

-	For the uplink direction, the QoS rules are enhanced to include the additional new type of Packet Filter when it is received in the corresponding PCC rule (additional new type of packet filter is not expected to be interpreted by a legacy UE; in case of legacy UE, it will use legacy packet filter of the QoS rule with the highest precedence). If supported by the UE, the UE uses both the legacy packet filter and the additional new type of packet filter for QoS Flow mapping.

NOTE:	It is recommended that CT1 specifies additional packet filter in a backward compatibility manner so that legacy UE can match the XRM traffic to a QoS flow with the QoS requirement defined in r18 XRM.

*** END of changes ***