3GPP TSG-SA2 Meeting #164	S2-2408480

Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024	(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0476 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support 5G ProSe multihop U2N Relay Communication with Model B discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 23.700-03 has conclusion that multi-hop U2N relay will be supported.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause for Layer-2 link management over PC5 and 5G ProSe Communication for multi-hop Layer-3 UE-to-Network Relay after Model B Discovery without N3IWF support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-hop U2N relay will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.1a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.5.1.1a	5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay without N3IWF after Model B Discovery

Figure 6.5.1.1a-1: 5G ProSe Communication via multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF after Model B discovery

1.	Service authorization and provisioning are performed for the 5G ProSe Layer-3 UE-to-Network Relay (step 1a), Intermediate Relay(s) (step 1b), and 5G ProSe Layer-3 Remote UE (step 1c).

2.	The 5G ProSe Layer-3 UE-to-Network Relay may establish a PDU Session for relaying. (refer to step 2 in clause 6.5.1.1)

3.	The 5G ProSe Layer-3 Remote UE performs Model B discovery of a 5G ProSe Layer-3 UE-to-Network Relay and store the path information to the UE-to-Network Relay(s) received during discovery procedure.

4.	For multi-hop connection setup between Remote UE and 5G ProSe Layer-3 UE-to-Network Relay, the Remote UE sends a Direct Communication Request to the 5G ProSe UE-to-Network Relay, which includes path information, and (optional) the requested PC5 QoS information.

Each Intermediate Relay forwards the received multi-hop Communication Request to the next Intermediate Relay or the UE-to-Network Relay based on the selected path information. The Communication Request from the Intermediate Relay may include cumulative delay (delay between Remote UE up to the Intermediate Relay).

It is assumed an Intermediate Relay sends the DCR message to the next Intermediate Relay only if the security association is established with the previous Intermediate Relay.

If there is no PDU Session associated with the Relay Service Code or a new PDU Session for relaying is needed, the 5G ProSe Layer-3 UE-to-Network Relay initiates a new PDU Session establishment procedure for relaying before completing the PC5 connection establishment.

5.	For IP PDU Session Type and IP traffic over PC5 reference point, the UE-to-Network Relay assigns IPv6 prefix or IPv4 address to Intermediate Relay(s) and the Remote UE. The Intermediate Relay relays the IP allocation message (e.g. Router Solicitation, Advertisement).

After an IP address of Remote UE is assigned, the UE-to-Network Relay informs the assigned IP address of Remote UE to Intermediate Relay(s) at the multi-hop path for the connection between Remote UE and UE-to-NW Relay.

6.	The 5G ProSe Layer-3 Remote UE may provide PC5 QoS Info and PC5 QoS rule(s) to the 5G ProSe Layer-3 UE-to-Network Relay using Layer-2 link modification procedure via Intermediate Relay(s). The 5G ProSe Layer-3 UE-to-Network Relay generates the Packet Filters used over Uu interface based on the received PC5 QoS Info and QoS Rule(s) as described in clause 5.6.2.1 and may perform the UE requested PDU Session Modification as defined in TS 23.502 [5] clause 4.3.3 to setup a new QoS Flow or bind the traffic to an existing QoS Flow.

7.	The 5G ProSe Layer-3 UE-to-Network Relay shall send a Remote UE Report (Remote User ID, Remote UE info) message to the SMF for the PDU Session associated with the multihop relay connection (refer to step 7 in clause 6.5.1.1).

8.	Remote UE and UE-to-Network Relay may exchange data traffic via Intermetidate Relays. Intermediate Relays route IP packet to/from Remote UE based on IP address of Remote UE.

# * * * * End of changes * * * *