3GPP TSG-SA2 Meeting #164	S2-2408514

# Maastricht, NL,  19th - 23rd Auguest, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1439 | rev | - | Current version: | V18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to support the conclusion of KI#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA2 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.2 in 3GPP TS 23.700-77 concludes KI#2 on "Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session". The corresponding normative work need to be implementd. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures of IMS data channel capabilies exposure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No normative specification for conclusions of KI#2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.1, AA.2.4.X (new) and AC.X.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |
| Nimsas_SessionManagement | Create | Request/Response | DC AS, NEF |
|  | Update | Request/Response | DC AS, NEF |
|  | Delete | Request/Response | DC AS, NEF |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

# * * * * Second change * * * *

### AA.2.4.X	Nimsas_SessionManagement service

#### AA.2.4.X.1	General

Service description: This service enables the consumer NF to create, update and terminate an IMS session with audio, video and/or data channel media.

#### AA.2.4.X.2	Nimsas_SessionMangement_Create service operation

Service operation name: Nimsas_SessionManagement_Create

Description: This service operation enables the consumer NF to create an IMS session with audio, video and/or data channel media.

Inputs, Required: Serving Number, Destination Number, Media Information set.

Serving Number: it is the number served by the DC AS.

Destination Number: it is the called party number.

Media Inforamaiton set includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Type: it indicates the media type used in the created session, and can be Audio, Video or Data Channel.

-	Media Parameter: it indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it need indicate the codec type the consumer NF requires to use.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC).

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P) and the party with wich the P2A ADC is used.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: DC Info.

DC Info indicates the attributes of data channel to be created when initiating the call.

Outputs, Required: Result indication and when the operation is successful: Session ID.

Outputs, Optional: None.

#### AA.2.4.X.3	Nimsas_SessionMangement_Update service operation

Service operation name: Nimsas_SessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as creating or releasing bootstrap data channel(s) or application data channel(s).

Inputs, Required: Session ID, Media operation set.

Media operation set includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	operation type: it indicates adding, updating or removing the corresponding media.

-	media type: it indicates the operated media is audio, video or data channel.

-	media info: it indicates the related media parameters. For different media, it contains different parameters.

-	When the media type is audio or video, it need indicate the codec type the consumer NF requires to use.

-	When the media type is data channel, it need indicate the data channel type (ADC or BDC).

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P) and the party with wich the P2A ADC is used.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### AA.2.4.X.4	Nimsas_SessionMangement_Delete service operation

Service operation name: Nimsas_SessionManagement_Delete

Description: This service operation enables the consumer NF to release the specific IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

# * * * * First change * * * *

### AC.X.4		IMS data channel capability exposure procedures

#### AC.X.4.1	General Description

##### AC.X.4.1.1	Network model of P2A session

Figure AC.X.4.1.1-1 illustrates a typical network model of P2A session where UE A initiates an audio/video session with the service number of an enterprise. After the audio/video session is connected, the DC AS in the enterprise domain can invoke the IMS APIs exposure by the NEF to add data channel(s) into the session for better interaction with UE A.

Figure AC.X.4.1.1-1: Network model of P2A session

To enable the DC AS in the enterprise domain to invoke the IMS APIs exposure by the NEF, the DC AS shall subscribe the IMS session events of all the subscribers who call its service number firstly and then the IMS AS will report to the DC AS when the session status of the sessions calling its service number are changed.

##### AC.X.4.1.2	Network model of P2P session

Figure AC.X.4.1.2-1 illustrates a typical network model of P2P session where UE A initiates an audio/video session with UE B. The DC AS in either the originating network or the terminating network can invoke the IMS APIs exposure by the NEF to add or remove data channel(s) into the session if its data channel channel application(s) is used in the current IMS session. Figure AC.X.4.1.2-1 shows the DC AS in the terminating network as an example.

Figure AC.X.4.1.2-1: Network model of P2P session

#### AC.X.4.2	Establishment procedure of bootstrap data channel

In a P2A session, the DC AS can invoke the IMS APIs exposure by the NEF to create a bootstrap data channel between the DCSF-2 and UE A. Figure AC.X.4.2-1 shows the procedure.

Figure AC.X.4.2-1: Establishment procedure of bootstrap data channel in a P2A session

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to create a bootstrap data channel with UE A in the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving the service number of the enterprise if needed. If the service number is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to create a bootstrap data channel with UE A in the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of adding a bootstrap data channel with UE A. Then the DCSF-2 instructs the IMS AS-2 to set up the bootstrap data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource for the bootstrap data channel. The detailed procedure is depicted in step 3~10 in Figure AC.7.1-1.

5.	The IMS AS-2 generates a re-INVITE request in which the SDP offer contains the media information of the bootstrap data channel required by the DC AS with the other existing media descriptions.

6.	The IMS AS-2 sends the re-INVITE request to UE A to negotiate the bootstrap data channel media. Then the UE A returns a 200 OK response with the SDP answer.

7.	The IMS AS-2 interacts with the MF-2 to update the bootstrap data channel resource according to the SDP answer in the received 200 OK response.

8.	The MF-2 establishes the bootstrap data channel with UE A and the DCSF-2 respectively.

9.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

10.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

In a P2P session, the DC AS in either the originating network or the terminating network can invoke the IMS APIs exposure by the NEF to create two bootstrap data channels with UE A and UE B respectively. Figure AC.X.4.2-2 shows the procedure of the DC AS in the terminating network.

Figure AC.X.4.2-2: Establishment procedure of bootstrap data channel in a P2P session

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to create two bootstrap data channels with UE A and UE B respectively in the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving UE B if needed. If UE B is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to create two bootstrap data channels with UE A and UE B respectively in the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of adding two bootstrap data channels with UE A and UE B respectively. Then the DCSF-2 instructs the IMS AS-2 to set up two bootstrap data channels by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource for the two bootstrap data channels. The detailed procedure is depicted in step 3~10 in Figure AC.7.1-1.

5.	The IMS AS-2 generates two re-INVITE requests in which the SDP offer contains the media information of the bootstrap data channel with UE A and UE B respectively with the other existing media descriptions.

6.	The IMS AS-2 sends one re-INVITE request to UE A to negotiate the bootstrap data channel media. Then the UE A returns a 200 OK response with the SDP answer.

7.	The IMS AS-2 interacts with the MF-2 to update the bootstrap data channel resource according to the SDP answer in the received 200 OK response from UE A.

8.	The IMS AS-2 sends another re-INVITE request to UE B to negotiate the bootstrap data channel media. Then the UE B returns a 200 OK response with the SDP answer.

9.	The IMS AS-2 interacts with the MF-2 to update the bootstrap data channel resource according to the SDP answer in the received 200 OK response from UE B.

10.	The MF-2 establishes the bootstrap data channel between the DCSF-2 and UE A and UE B respectively.

11.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

12.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

#### AC.X.4.3	Termination procedure of bootstrap data channel

The DC AS can invoke the IMS APIs exposure by the NEF to terminate bootstrap data channel(s) between the DCSF-2 and UE A. Figure AC.X.4.3-1 shows the procedure in a P2A session.

Figure AC.X.4.3-1: Termination procedure of bootstrap data channel

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to terminate the bootstrap data channel with UE A in the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving the specific UE if needed. If the UE is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to terminate the bootstrap data channel with UE A in the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of removing the bootstrap data channel with UE A. Then the DCSF-2 instructs the IMS AS-2 to terminate the bootstrap data channel by sending a Nimsas_MediaControl request.

5.	The IMS AS-2 generates a re-INVITE request in which the port of "m=application" line for the bootstrap data channel is set to zero.

6.	The IMS AS-2 sends the re-INVITE request to UE A to terminate the bootstrap data channel. Then the UE A returns a 200 OK response with the SDP answer.

If in a P2P session, the IMS AS-2 also sends a re-INVITE request to UE B to terminate the bootstrap data channel.

7.	The IMS AS-2 interacts with the MF-2 to release the bootstrap data channel resource according to the SDP answer in the received 200 OK response.

8.	The MF-2 terminates the bootstrap data channel with UE A and the DCSF-2 respectively.

9.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

10.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

#### AC.X.4.4	Establishment procedure of P2A application data channel

The DC AS can invoke the IMS APIs exposure by the NEF to create a P2A application data channel between UE A and itself. Figure AC.X.4.4-1 shows the procedure in a P2A session.

Figure AC.X.4.4-1: Establishment procedure of P2A application data channel

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to create a P2A application data channel with UE A in the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving the specific UE if needed. If the UE is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to create a P2A application data channel with UE A in the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of adding a P2A application data channel with UE A. Then the DCSF-2 instructs the IMS AS-2 to set up the P2A application data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource for the P2A application data channel. The detailed procedure is depicted in step 3~10 in Figure AC.7.1-1.

5.	The IMS AS-2 generates a re-INVITE request in which the SDP offer contains the media information of the P2A application data channel required by the DC AS with the other existing media descriptions.

6.	The IMS AS-2 sends the re-INVITE request to UE A to negotiate the P2A application data channel media. Then the UE A returns a 200 OK response with the SDP answer.

7.	The IMS AS-2 interacts with the MF-2 to update the P2A application data channel resource according to the SDP answer in the received 200 OK response.

8.	The MF-2 establishes the application data channel with UE A and the DC AS respectively.

9.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

10.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

#### AC.X.4.5	Establishment procedure of P2P application data channel

The DC AS in either the originating network or the terminating network can invoke the IMS APIs exposure by the NEF to create a P2P data channel between UE A and UE B if its data channel channel application is used in the current IMS session. Figure AC.X.4.4-1 shows the procedure of the DC AS in the terminating network as an example.

Figure AC.X.4.5-1: Establishment procedure of P2P application data channel

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to create a P2P application data channel between UE A and UE B in the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving the specific UE if needed. If the UE is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to create a P2P application data channel between UE A and UE B in the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of adding a P2P application data channel between UE A and UE B. Then the DCSF-2 instructs the IMS AS-2 to set up the P2P application data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource for the P2P application data channel. The detailed procedure is depicted in step 3~10 in Figure AC.7.1-1.

NOTE:	In this case, the P2P application data channel is anchored to the MF-2. It is not mandatary and depends on the operator policy.

5.	The IMS AS-2 generates two re-INVITE requests in which the SDP offer contains the UE A side and the UE B side media information of the P2P application data channel respectively with the other existing media descriptions.

6.	The IMS AS-2 sends one re-INVITE request to UE A to negotiate the P2P application data channel media. Then the UE A returns a 200 OK response with the SDP answer.

7.	The IMS AS-2 interacts with the MF-2 to update the P2P application data channel resource of UE A side according to the SDP answer in the received 200 OK response from UE A.

8.	The IMS AS-2 sends another re-INVITE request to UE B to negotiate the P2P application data channel media. Then the UE B returns a 200 OK response with the SDP answer.

9.	The IMS AS-2 interacts with the MF-2 to update the P2P application data channel resource of UE B side according to the SDP answer in the received 200 OK response from UE B.

10.	The MF-2 establishes the application data channel with UE A and UE B respectively.

11.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

12.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

#### AC.X.4.6	Establishment procedure of P2A2P application data channel

As specified in clause AC.7.2.3, the P2A2P application data channel means that both the originating UE and the terminating UE establish an application data channel for the same application to communicate with the same DC AS and the DC AS correlates the data exchanged with both UEs through the two application data channels. That is, when a P2A2P application data channel is needed, the DC AS will invoke the IMS APIs exposure by the NEF to create two P2A application data channels with UE A and UE B respectively. Figure AC.X.4.6-1 shows the establishment procedure of a P2A2P application data channel.

Figure AC.X.4.6-1: Establishment procedure of P2P application data channel

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to create two P2A application data channels with UE A and UE B respectively in the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving the specific UE if needed. If the UE is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to create two P2A application data channels with UE A and UE B respectively in the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of adding two P2A application data channel with UE A and UE B respectively. Then the DCSF-2 instructs the IMS AS-2 to set up two P2P application data channels by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource for two P2A application data channels with UE A and UE B respectively. The detailed procedure is depicted in step 3~10 in Figure AC.7.1-1.

5.	The IMS AS-2 generates two re-INVITE requests in which the SDP offer contains the media information of two P2A application data channels respectively with the other existing media descriptions.

6.	The IMS AS-2 sends one re-INVITE request to UE A to negotiate the P2A application data channel media. Then the UE A returns a 200 OK response with the SDP answer.

7.	The IMS AS-2 interacts with the MF-2 to update the P2A application data channel resource of UE A according to the SDP answer in the received 200 OK response from UE A.

8.	The IMS AS-2 sends another re-INVITE request to UE B to negotiate the P2A application data channel media. Then the UE B returns a 200 OK response with the SDP answer.

9.	The IMS AS-2 interacts with the MF-2 to update the P2A application data channel resource of UE B according to the SDP answer in the received 200 OK response from UE B.

10.	The MF-2 establishes the P2A application data channel between the DC AS with UE A and the P2A application data channel between the DC AS and UE B respectively.

11.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

12.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

#### AC.X.4.7	Termination procedure of application data channel

The termination procedure of application data channel is same as that of bootstrap data channel specified in clause AC.X.4.3.

#### AC.X.4.8	Termination procedure of all the data channels

The DC AS can invoke the IMS APIs exposure by the NEF to terminate the current P2A or P2P session. It will also terminate all the bootstrap data channels and the application data channels. Figure AC.X.4.8-1 show the termination procedure of all the data channels in a P2A session.

Figure AC.X.4.8-1: Termination procedure of all the data channels

1.	The DC AS sends a Nnef_ImsSessionManagement_Delete request to the NEF requiring to release the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving the specific UE if needed. If the UE is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Delete request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Delete request to the IMS AS-2 requiring to release the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of releasing the specific session. Then the DCSF-2 instructs the IMS AS-2 to release the all the data channels by sending a Nimsas_MediaControl request.

5.	The IMS AS-2 generates a BYE request.

6.	The IMS AS-2 sends the BYE request to UE A to release the specific session. Then the UE A returns a 200 OK response.

7.	The IMS AS-2 interacts with the MF-2 to release all the data channel resource.

8.	The MF-2 establishes the application data channel with UE A and the DC AS respectively.

9.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Delete response to the NEF.

9.	The NEF returns a successful Nnef_ImsSessionManagement_Delete response to the DC AS.

# * * * * End of changes * * * *