3GPP TSG|WG-SA2 Meeting #164 	S2-2408520

Maastricht, Netherlands, August 19 – 23, 2024	(revision of S2-240xxx)

# Source:	China Telecom

# Title:	New WID on Ranging/SL Positioning Information Usage Reporting

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Ranging/SL Positioning Usage Information Reporting

Acronym:	TEI19_Ranging_SL_UIR

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940069 | Study on Ranging based services and Sidelink Positioning | SA2 R18 Study for Ranging based services and Sidelink Positioning (TR 23.700-86) |
| 980015 | (Stage 2 of Ranging_SL) Architecture Enhancement to support Ranging based services and Sidelink Positioning | SA2 R18 Work Item for Architecture Enhancement to support Ranging based services and Sidelink Positioning (TS 23.586) |
| 1000009 | Study on Charging Aspects of Ranging and Sidelink Positioning | SA5 R18 Study for Charging Aspects of Ranging and Sidelink Positioning (TR 28.845) |
| 1040013 | Charging Aspects of Ranging and Sidelink Positioning | SA2 R19 Work Item for Charging Aspects of Ranging and Sidelink Positioning |

Dependency on non-3GPP (draft) specification: None

3	Justification

SA5 has completed the study towards the charging aspects of Ranging and Sidelink Positioning in TR 28.845 and the WID is agreed in SA #104 meeting. However, two work tasks which are no-5GC NF(s) involved scenario (i.e. the UE-only operation, UE Discovery) need to depend on SA2. It is because UE should send Ranging/SL Positioning usage information to the GMLC (embedding CTF) with respect to the conclusion in SA5, but SA2 has not defined how to report such information to the network in no-5GC NF(s) involved scenario.

Given that the no-5GC NF(s) involved scenario (i.e. the UE-only operation, UE Discovery) in Ranging/SL Positioning service uses the similar mechanism as the scenario in ProSe (i.e. ProSe Direct Communication and ProSe UE-to-Network Relay Discovery), the mechanism of usage information reporting for charging purpose in ProSe can be also partly applied in Ranging/SL Positioning service. In ProSe, the DDNMF is used to provide the necessary charging functionality or charging information for the usage of 5G ProSe Direct Discovery and/or ProSe Direct Communication as described in TS 23.304, for example, receiving the usage report from UE over the user plane and using the PC3a protocol. Considering most charging users are 5G ProSe capable UEs in Ranging/SL service, the DDNMF may also be reused in Ranging/SL service for charging purpose. Specifically, the UE may report Ranging/SL Positioning Information to DDNMF as the method defined in ProSe, and then the DDNMF further can deliver Ranging/SL Positioning Usage Information to GMLC (embedding CTF) for charging purpose. In this case, how DDNMF delivering Ranging/SL Positioning Usage Information to GMLC is required to be defined in SA2.

Thus, to ensure charging for Ranging/SL service properly and SA5 charging group can continue their normative work, it proposes to reuse part of DDNMF mechanism in ProSe for Ranging/SL Positioning charging purpose and define how DDNMF to deliver Ranging/SL Positioning Information to GMLC in Rel-19.

4	Objective

In order to charge in no-5GC NF(s) involved scenario of Ranging/Sidelink Positioning service (i.e.UE-only operation and  UE Discovery) properly, the Ranging/SL Positioning Usage Information for charging purpose is required to be transferred to GMLC (embedding CTF) from UE.

Considering no-5GC NF(s) involved scenario of Ranging/Sidelink Positioning service has similar mechanism as the scenario in ProSe (i.e. ProSe Direct Communication and ProSe UE-to-Network Relay Discovery), the DDNMF is reused to receive the Ranging/SL Positioning Usage Information from UEs over the user plane and using the PC3a protocol as described in TS 23.304. Furthermore, the DDNMF is required to be enhanced to deliver the Ranging/SL Positioning Usage Information to GMLC for Ranging/SL Positioning charging purpose.

Specifically, the following enhancements are needed in no-5GC involved scenario of Ranging/Sidelink Positioning service (i.e.UE-only operation and  UE Discovery):

- Enhancement of DDNMF to receive Ranging/SL Positioning Usage Information from UE.

- Support of DDNMF to deliver Ranging/SL Positioning Usage Information to GMLC.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.586 | Architectural enhancement to support Ranging/SL Positioning Usage Information | SA#106 (December 2024) |  |
| 23.273 | GMLC discovery and service enhancements to support Ranging/SL Positioning Usage Information reporting | SA#106 (December 2024) |  |
| 23.502 | NEF service enhancement to support Ranging/SL Positioning Usage Information reporting | SA#106 (December 2024) |  |

This TEI19_WID requires 0.5 TU.

6	Work item Rapporteur(s)

Jiahui LI, China Telecom, lijh36@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

SA5 - Charging, if any identified.

9	Supporting Individual Members

Supporting IM name

China Telecom

CATT

Oracle

vivo

OPPO