3GPP TSG-WG SA2 Meeting #164	S2-2408523

# Maastricht, Netherlands, August 19 – 23, 2024	 (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | draft | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement on support of Ranging/SL Positioning Usage Information Reporting to GMLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19-xxx |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To address the scenario described in TEI19-xxx where the Ranging/SL Positioning Usage Information for charging purpose requires to be reported to GMLC in the UE-olny operation and Ranging/Sidelink Positioning UE Discovery procedure. The Ranging/SL Positioning Usage Information can be transferred via DDNMF when the UE is in coverge and DDNMF is used to report such information to the GMLC. Thus, the functions of GMLC and corresponding procedures need enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance GMLC for charging purpose. Add procedures of Ranging/SL Positioning Usage Information reporting in the UE-olny operation and UE Discovery procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.9, 5.2.1. 5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.3.9	GMLC

In addition to the functions defined in TS 23.273 [8], the GMLC supports the following:

-	enable trusted AFs and NFs to perform MT-LR by accessing GMLC directly with additional service parameters for Ranging/SL Positioning.

-	enable AFs and NFs to perform MT-LR by accessing GMLC through NEF with additional service parameters for Ranging/SL Positioning.

-	determine the serving AMF instances for the UEs involved in the Ranging/SL Positioning request and forward the request to respective serving AMF.

-	receive response from AMF and event reports from and return Ranging/SL Positioning result to NFs and AFs.

-	perform Application Layer ID to GPSI or GPSI to Application Layer ID resolution by querying NEF.

-	use UE's Ranging/SL Positioning privacy profile received from UDM for verifying UE privacy.

-	request Home GMLC of the UE(s) that belong to different PLMN(s) to retrieve the mapping information between GPSI and Application Layer ID (if needed) and to check those UE's Ranging/SL Positioning privacy profile.

-	provide the necessary charging functionality or charging information for the usage of Ranging/SL Positioning service to interact with CHF.

NOTE:	When receiving location request by using SUPI of a UE from LCS client after translating SUPI to GPSI by using NUDM_sdm get service, GMLC performs Application Layer ID to GPSI or GPSI to Application Layer ID resolution by querying NEF with Nnef_UEid service operation defined in clause 5.2.6.27 of TS 23.502 [3].

# * * * * Next change * * * *

### 5.2.1	General

Both Model A and Model B discovery as defined in clause 6.3.2 of TS 23.304 [7] are supported for the 5G ProSe capable UEs discovery (including commercial and public safety use cases). During Ranging/SL Positioning UE direct discovery, the discovery message indicates this discovery message is for "Ranging/Sidelink Positioning"; During Ranging/SL Positioning Group Member Discovery, the Application Layer Group ID indicates a Ranging/Sidelink Positioning group that the UE belongs to.

NOTE:	The content type value indicates the discovery is for Ranging/Sidelink Positioning UE discovery. The details for Ranging/Sidelink Positioning UE discovery depends on stage 3.

After procedures of UE discovery, the usage information of Ranging/SL Positioning UE discovery is reported to DDNMF over the user plane as the same mechanism in ProSe UE-to-Network Relay Discovery, and DDNMF further provides the information to GMLC for charging purpose.

Procedures for V2X communication over PC5 reference point as defined in TS 23.287 [6] are supported for the V2X capable UEs discovery. When unicast mode V2X communication procedure as defined in clause 6.3.3 of TS 23.287 [6] is used, the Service Type in Layer-2 link establishment procedure indicates "Ranging/Sidelink Positioning"; when broadcast mode or groupcast mode V2X communication procedure as defined in clause 6.3.1 or clause 6.3.2 of TS 23.287 [6] is used, Service Type indicates "Ranging/Sidelink Positioning".

The role(s) of the discovered UE (e.g. Target UE, SL Reference UE, SL Positioning Server UE, Located UE) is (are) included in discovery message (for 5G ProSe capable UE) and unicast link establishment messages (for V2X capable UE) as RSPP specific meta data, which contents are defined in clause 6.11 of TS 38.355 [12]. More details on this and other information conveyed during discovery can be found in clause 6.4.

The procedures for Ranging/Sidelink Positioning UE Discovery & Selection are defined in clause 6.4.

# * * * * Next change * * * *

## 5.11	UE-only Operation for SL Positioning using Located UE

When LMF is not involved for SL Positioning/Ranging, e.g. the LMF in the serving network does not support SL Positioning, UE-only Operation SL Positioning is used, including Target UE as SL Positioning Server UE and Target UE not as SL Positioning Server UE.

When the Target UE acts as a SL Positioning Server UE, the following principles applies:

-	The Target UE performs the Located UE's discovery and selection when absolute location is requested.

-	For absolute location, the Target UE obtains the location of the Located UE(s), and the Located UE(s) may trigger a 5GC-MO-LR to retrieve its location. The Target UE uses the location of Located UE(s) together with the Ranging/SL positioning measurement data or result to estimate its own location.

-	The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same scheduled location time (as per TS 23.273 [8]) to improve the Target UE positioning accuracy.

-  The Ranging/SL Positioning Usage Information is reported to DDNMF by Target UE over the user plane as the same mechanism in ProSe Direct Communication and DDNMF further provides the information to GMLC.

NOTE:	Security and privacy aspects require confirmation from SA WG3.

When the Target UE does not act as SL Positioning Server UE, the following principles applies:

-	The Target UE performs the SL Positioning Server UE's discovery and selection.

-	The Target UE requests its absolute location from the SL Positioning Server UE.

-	The SL Positioning Server UE interacts with the Target UE to exchange UE capabilities, assistance data, Ranging/SL positioning measurement data, location results and location of Located UE(s).

-	The SL Positioning Server UE can optionally determine to use the location of Located UE(s) together with the Ranging/SL positioning measurement data or result to estimate the absolute location of Target UE.

-	The Target UE performs the Located UE's discovery and selection, and obtains the location of the Located UE(s).

-	The SL Positioning Server UE returns the Ranging/SL positioning result to the Target UE.

-	The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same scheduled location time (as per TS 23.273 [8]) to improve the Target UE positioning accuracy.

-  The Ranging/SL Positioning Usage Information is reported to DDNMF by Target UE over the user plane as the same mechanism in ProSe Direct Communication and DDNMF further provides the information to GMLC.

When UE-only operation is triggered for the service request from a SL Positioning Client UE, the SL positioning client UE should send the service request to a discovered Target UE or SL Reference UE, the UE receiving the service request will work as the Target UE.

# * * * * End of changes * * * *