3GPP TSG-WG SA2 Meeting #164	S2-2408554

Maastricht, NL, 19th Aug – 23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion on key issue 1 for topology 1 architecture

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusion principles are proposed on key issue 1 for topology 1 architecture.

1. Introduction/Discussion

Key issue 1 includes the aspect “system architecture identified along with the solutions for KI#2 and KI#3”. The system architecture should be concluded with the architecture figure showing entities, NF functions, interfaces and in addition, general description of the features listed in key issue#2 and key issue#3 (e.g. supported by which entity/NF function).

This contribution proposed evaluation and conclusion principles for topology 1 system architecture.

Support of AIoT service:

AIoTMF(other name like AIoTF, AIoT controller, etc) is introduced to support AIoT service. AIoTMF is either a standalone NF in the core network or supported by enhanced AMF.

Summary of the “how to support AIoTMF” from all TR solutions:

- AIoTMF as standalone NF: solution #3, #6, #8, #17, #19, #20

- AIoTMF is supported by enhanced AMF: #7, #25, #29

- Either option is fine: solution #4, #5, #9, #10, #18, #28, #31, #33

NOTE that other solutions do not describe topology 1 system architecture.

Observation 1: majority solutions prefer AIoTMF as a standalone NF

Interface between BS reader and core network:

Two solution options are proposed in the TR:

- R-AP protocol, AIoT control protocol, using SBI as underlying transport

- Simplified NG-AP, AIoT control protocol and using (existing) SCTP as underlying transport

Observation 2: AIoT control protocol is commonly used in all SA2 solutions for interface between BS reader and core network.

It is proposed to name the interface between BS reader and core network “AIoT-NGAP”, and SA2 only focus on the AIoT control protocol. The underlying transport protocol will be coordinated with RAN3.

Interface between AIoT Device and the core network:

Two solution options are proposed in the TR:

- NAS-like protocol (named AIoT NAS): solution #4, #5, #9, #11, #12, #15, #17, #20, #33

- No logic interface between AIoT Device and the core network, but to use AS interface and AIoT NGAP for transmission between AIoT device and core network: Solution #3

Observation 3: NAS-like protocol/interface between AIoT Device and the core network has overwhelming support in the TR.

AIoT Device subscription management:

Depending on the AIoT Device ID assignment mode, two solution options are proposed in the TR:

- UDM is responsible for AIoT Device subscription management

- AAA server is responsible for AIoT Device subscription management

AIoT service exposure:

All TR solutions indicate NEF is responsible for AIoT service exposure, except for solution#3.

Solution #3 proposed CAPIF framework can be used to register and discover APIs of the AIoT Controller.

Observation 4: all solutions except solution#3 proposed NEF is responsible for AIoT service exposure

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

8.b	Conclusion on Key Issue #1

8.b.1	Architecture to Support Topology 1

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on “Ambient IoT Device subscription management” and key issue#3 aspect on “Ambient IoT service exposure” is considered in this section.

The following principles are agreed for the architecture to support topology 1:

-	A new core network function, AIoTMF, is introduced to support Ambient IoT

-	The AIoTMF communicates with a BS reader via a direct interface called AIoT-NGAP

Figure 8.b.1-1: illustrative architecture for topology 1

NOTE: SA2 only focus on the AIoT control functionality offered by AIoT-NGAP interface. The underlying transport protocol will be coordinated with RAN3.

-	AIoT-NAS protocol is used between an Ambient IoT Device and the AIoTMF

-	UDM or AAA Server is responsible for management of Ambient IoT Device subscription-like information

-	NEF is responsible for Ambient IoT service exposure to third-party AF

# * * * * End of changes * * * *