3GPP TSG SA WG2#164	            S2-2408571

Maastricht, 19-23 August 2024 	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0252 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF provisioning N6 delay measurement information via EDI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To address the outcome of KI#2 in approved eEDGE_5GC_Ph3 WID SP-240996 on provisioning of N6 delay measurement information, N6 delay measurement assistance and requirement information in EDI is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce N6 delay measurement assistance and requirement information in EDI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 6.2.3.4.1	General

EAS Deployment Information management refers to the capability to create, update or remove EAS Deployment Information from AF and the distribution to the SMF. The NEF is in charge of the management of EAS Deployment Information which may be stored in UDR.

The EAS Deployment Information indicates how edge services are deployed in each Local part of the DN, the description of EAS Deployment Information is shown in Table 6.2.3.4-1.

Table 6.2.3.4-1 Description of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| AF ID | Addressing information of Application Function responsible for the DNAI in the record. [Optional]. See NOTE 1. |
| DNN | DNN for the EAS Deployment Information. [optional] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| External Group Identifier/Internal Group Identifier | Group ID for the EAS Deployment information. [optional]. See NOTE 2. |
| Application ID | Identifies the application for which the EAS Deployment Information corresponds to. [optional] |
| FQDN(s) | Supported FQDN(s) for application(s) deployed in the Local part of the DN. |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [optional] |
| DNS Server Information | list of DNS server identifier (consisting of IP address and port) for each DNAI. [optional] |
| EAS IP address range Information | IP address(es) of the EASs in the Local part of the DN or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es) of the Local part of the DN where the EAS is deployed for each DNAI. [optional] |
| N6 traffic routing information | Information about how to forward edge traffic in the local part of DN corresponding to DNAI. [optional] |
| N6 delay measurement required indication | Indicates that N6 delay monitoring should be supported for a given EAS IP address(es) or designated IP range(s). See NOTE X. [optional] |
| N6 delay measurement assistance information | Includes the following parameters per DNAI and/or per EAS to enable N6 delay monitoring: - measurement endpoint address information (consisting of IP address and optionally port number). - measurement protocol(s) supported by the application measurement endpoint, - protocol-specific configuration parameters, - supported maximum measurement session frequency. NOTE Y. [optional] |
| NOTE 1: When an AF ID is provided, all DNAI(s) correspond to the same EHE provider. NOTE 2: The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. For HR-SBO roaming scenario, External Group Identifier and Internal Group Identifier, cannot be used by AF in VPLMN. NOTE 3: AF ID can be used in case of AF(s) involving different EHE providers, and the source EHE is unaware of other/target EHE specific deployment details. NOTE X: A measurement endpoint can be used for delay measurements representing a single EAS IP address or a range (IPv4 subnet or IPv6 prefix) of EAS IP addresses NOTE Y: The supported maximum measurement session frequency represents the measurement endpoint’s capability and/or preference on receiving and reflecting the N6 delay measurement sessions. |  |

Editor’s Note: The exact measurement protocol(s) and its specific configuration parameters should be decided in collaboration with SA3.

The EAS Deployment Information management procedures are described in this clause, the procedures are independent of any PDU Session, including:

-	The procedure for EAS Deployment Information management from AF via the NEF.

-	The procedure for EAS Deployment Information management in the SMF.

-	The procedure for BaselineDNSPattern management in the EASDF.

NOTE:	In order to support EAS discovery when the Edge Hosting Environment is provided by a partner, an SLA is needed between current operator and the partner to provide e.g. the Address(es) and credentials for the DNS servers if the partner hosts the DNS server(s) for the related DNS resolution.

End of CHANGES