3GPP TSG-WG SA2 Meeting #164	S2-2408578

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1454 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility support of UE-satellite-UE communicaitons |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Thales, CATT, CSCN, Honor, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of features to support UE-satellite-UE communications for satellite communications based on the conclusions for KI#3 in TR 23.700-29. Service continuity is essential to minimize voice drop rates, and adjustments to the Access Gateway (AGW) location may be necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause to describe the high level feature of mobility support - Add a new clause to describe the high level feature of IMS-AGW relocation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service continuity cannot be ensured when serving satellite changes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XY.3 (new), XY.3.1 (new), XY.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# XY.3	Network function functional enhancements

## XY.3.1	P-CSCF

The P-CSCF supports the following supports functionality:

- Session continuity control for UE-satellite-UE communications when one of UE’s serving satellite changes

- Reselection of IMS-AGW during an IMS session when the satellite changes

# * * * * Second change (new) * * * *

# XY.3	High level features

## XY.3.x		Mobility support with AGW onboarding

This clause applies when one of the UE’s serving satellite changes (e.g. UE’s movement, or satellite’s movement ). To ensure the IMS service continuity instead of directly disabling the IMS session, the UE-SAT-UE communication link can be either decided to be continued, or to fall back to the ground. The determination of mobility support is executed separately at MO/MT side. When the serving satellite changes, the serving P-CSCF is informed of the new satellite ID and access information via PCF,

- if the UE’s new serving satellite (constellation) that has UPF and IMS-AGW on board, and the UE’s new serving satellite can communicate with the other UE’s serving satellite, the P-CSCF may decide to continue the UE-SAT-UE communication and select a new IMS-AGW that onboarding the new serving satellite

- if the new serving satellite has no connection with the other UE’s serving satellite, the P-CSCF may decide to deactivate the UE-SAT-UE communication and select a new IMS-AGW on the ground

Figure XY.3-2: IMS signalling and media path after deactivating the UE-SAT-UE

Figure XY.3-2 depicts the media path between UE 1 and UE 2 after deactivating the UE-SAT-UE:

-	UE-1 sends the voice media packets (e.g. RTP packets) to the onboarding gNB and ground PSA, and the ground PSA forwards the voice packet to the ground IMS-AGW (via N6).

-	UE-1’s ground IMS-AGW forwards the voice packet to the UE-2’s ground IMS-AGW based on the address information exchanges during the call setup period.

# * * * * Third change (new) * * * *

## XY.3.y	IMS-AGW relocation

To enable the service continuity, a new IMS-AGW may be selected during an IMS voice/video session. P-CSCF will first interact with the newly selected IMS-AGW to obtain the IP address for both the MO side and the MT side and send the newly allocated IP addresses to UE and the other P-CSCF (IMS-ALG) via re-invite messages.

Editor's note:	It's FFS whether the UE or the P-CSCF initiates the re-Invite procedure.

# * * * * End of changes * * * *