3GPP TSG-SA2 Meeting #164	S2-2408585

# Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1182 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: new Analytics ID for signalling storm caused by abnormal NF behavior |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TR 23.700-84 it was concluded for key issue 4 to provide Analytics for signalling storm caused by abnormal NF behavior. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new Analytics ID for signalling storm caused by abnormal NF behavior is introduced. Necessary input parameters for NWDAF to generate the new analytics are derived from key issue 4 conclusions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Analytics for signalling storm caused by abnormal NF behavior |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 6:X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

### 6.X	Abnormal NF behaviour related network data analytics

#### 6.X.1	General

This clause defines an Analytics to discover abnormal NF behaviour, i.e. signalling storms, that can be used by analytics consumers for the prevention of a signalling storm or for mitigation the impact of it.

The consumer of this analytics could be a 5GC NF.

By means of statistics and predictions, NWDAF supports assistance to signalling storm mitigation and prevention by statistically learning the expected normal level of signalling and identifying a significant deviation indicating a signalling storm caused by NFs signalling.

The consumer of this analytics shall indicate in the request:

-	Analytics ID = "Abnormal NF behaviour";

-	Target of Analytics Reporting as defined in clause 6.1.3;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Analytics Filter Information optionally including:

-	expected NF behaviour parameters;

-	analytics thresholds identifying a signalling storm caused by NFs signalling;

-	Area of interest;

-	Application ID;

-	DNN;

-	S-NSSAI.

-	Optionally, maximum number of objects and maximum number of SUPIs;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

If the Target of Analytics Reporting is any NF, then the Analytics Filter should at least include:

-	Area of Interest, application ID, DNN or S-NSSAI.

When the NWDAF detects those NFs that deviate from the expected UE behaviour, the NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.X.3.

#### 6.X.2	Input Data

Table 6.7.5.2-3: Input data from 5GC related to Control Plane Signalling Abnormality

| Information (NOTE 1) | Source | Description |
| --- | --- | --- |
| NFs ID | Service Producer and Consumer entities via OAM or SCP | The NF instance ID of the service producer and consumer entities. |
| Service Operation information | Service Producer and Consumer entities via OAM or SCP | The information of the reference point and corresponding service operation. |
| Service Operation Request Counters | Service Producer entity via OAM or SCP (NOTE 2) | The counters related to the requests received by the service producer entity |
| > Number of requests |  | The number of request received in a time window. |
| > Rate of requests |  | The rate of the service requests. |
| > Unhandled requests |  | The number of request received but not responded. |
| > Timing of request |  | Max/min/variance of inter-request periods. |
| > Rejected requests |  | The number of rejected/timeout requests. |
| > Redundant requests |  | The number of redundant request. |
| Service Operation Response Counters | Service Consumer entity via OAM or SCP (NOTE 2) | The counters related to the requests sent by the service consumer entity. |
| > Number of requests |  | The number of request sent in a time window. |
| > Rate of requests |  | The rate of the service requests. |
| > Unhandled requests |  | The number of request sent but not replied. |
| > Timing of request |  | Max/min/variance of inter-request periods. |
| > Rejected requests |  | The number of timeout requests. |
| > Redundant requests |  | The number of redundant request. |
| NF Context data | Service Producer and Consumer entities via OAM | The information about the current UE context, or network context and NF’s load. |
| > Number of UEs |  | Total number of UEs that is served by the NF. |
| > Number of network context |  | Total number of network context (e.g. interface context) that is maintained by the NF. |
| > NF load |  | The load of specific NF instance(s) in their NF profile. |
| > NF status & resource usage |  | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk) as defined in clause 5.7 of TS 28.552 [11]. |
| Unexpected operational event (Note 3) | Service Producer and Consumer entities |  |
| NOTE 1: All input data are optional. NOTE 2: The already supported counters/KPIs by OAM defined per NF in TS 28.552 [11] are collected from OAM; other remaining counters/KPIs are collected from the corresponding NFs. NOTE 3: Whether and how service producer/consumer NFs provides these events are left to implementation. |  |  |

#### 6.X.3	Output Analytics

The output analytics of NF signalling storm statistics of NWDAF is defined in Table 6.X.3-1.

Table 6.X.3-1: Signalling storm statistics

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm. |
| > Target NF ID | The target of signalling storm detected by NWDAF. |
| > Abnormality ID | The potential cause of NF Abnormality (e.g. massive UE/frequent access, NF abnormal signalling, etc.). |
| > Source SUPI/NF list | SUPI(s) of the UE(s) or NF(s) which cause the signalling storm. |

The output analytics of signalling storm prediction of NWDAF is defined in Table 6.X.3-2.

Table 6.X.3-2: Signalling storm predictions

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm. |
| > Target NF ID | The target of signalling storm predicted by NWDAF. |
| > Abnormality ID | The potential cause of NF Abnormality (e.g. massive UE/frequent access, NF abnormal signalling, etc.). |
| > Source SUPI/NF list | SUPI(s) of the UE(s) or NF(s) which cause the signalling storm. |

The predictions are provided with a Validity Period, as defined in clause 6.1.3.

The number of abnormalities and the length of the NF list shall respectively be lower than the parameters maximum number of objects and Maximum number of NFs provided as part of Analytics Reporting Information.

End of Changes