3GPP TSG-WG SA2 Meeting #164	S2-2408589

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1364 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Set handling in non-3GPP accesses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Charter Communications, Tencent, Tencent Cloud, Interdigital Inc., Lenovo, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-zz |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following R19 TR 23.700-70 conclusions for Key Issue #7 (i.e., clause 8.7) were agreed as principles for normative work: - For wireline access: - W-AGF: - In addition to PDU QoS parameters, uses the PDU Set QoS parameter(s) sent by SMF over N2 to determine W-UP resources. - Uses PDU Set Information received in the GTP-U header over N3 to identify PDU Sets. - 5G-RG: - 5G-RG may receive the UL Protocol Description associated with the QoS rule over N1 from SMF to identify PDU Sets. - How the 5G-RG may identify PDU Sets and determine W-UP resources to use is left up to 5G-RG implementation. - For untrusted/trusted access: - N3IWF/TNGF: - In addition to PDU QoS parameters, uses the PDU Set QoS parameter(s) sent by SMF over N2 to determine IPsec Child SAs. - Uses PDU Set Information received in the GTP-U header over N3 to identify PDU Sets. - Based on operator local configuration, optionally, uses the PDU Set Importance received in the GTP-U header to determine the drop precedence of DSCP value for the DL packets. - UE: - As specified in clause 5.37.5.1 of TS 23.501 [2], UE may receive the UL Protocol Description associated with the QoS rule over N1 from SMF to identify PDU Sets. Whether and how the UE may identify UL PDU Sets is left up to UE implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying conclusions on PDU Set handling for non-3GPP accesses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not implemented in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.1.3.27.4	PDU Set based handling

PDU Set based handling support is defined in clause 5.37.5 of TS 23.501 [2]. The PCF may, based on local configuration or on information received from the AF, generate PDU Set Control Information and the PCF includes the UL and/or DL Protocol Description (see clause 5.37.5 of TS 23.501 [2] and TS 26.522 [40]) received from AF and the PDU Set Control Information in the PCC Rule. The PDU Set Control Information includes the PDU Set QoS Parameters (see clause 5.7.7 of TS 23.501 [2]). Based on the received the PCC rule or a pre-configured PCC rule, the SMF provides (R)AN the PDU Set QoS Parameters (as described in clause 5.37.5 of TS 23.501 [2]). If the PCC rule includes the Protocol Description, it is used for PDU Set identification (as described in clause 5.37.5 of TS 23.501 [2]).

When the AF requests that a data session to a UE/5G-RG is set up with a specific QoS (as described in clause 6.1.3.22), the AF can also provide the individual QoS parameters Requested PDU Set Delay Budget (UL and/or DL), Requested PDU Set Error Rate (UL and/or DL) and Requested PDU Set Integrated Handling Information (UL and/or DL) to describe the requirements for the PDU Set based handling and the Protocol Description (UL and/or DL).

# * * *  End of changes * * * *