3GPP TSG-SA2 Meeting #164	S2-2408614

Maastricht, Netherlands, 19 August – 23 August, 2024 	(revision of S2-24xxxxx)

Source:	CATT

Title:	Discussion on TEI19 WID of PRU Usage Extension in Core Network and One Transmission Path for Messages of one Positioning Procedure

Document for:	Discussion

Agenda Item:	30.2

Work Item / Release:	TEI19_PRUUaOTP / Rel-19

Abstract of the contribution: this discussion paper is proposed to introduce TEI19 WID of PRU Usage Extension in Core Network and One Transmission Path for Messages of one Positioning Procedure.

1. Introduction

In SA2#159 meeting, the received RAN1 LS (S2-2310113) includes RAN1 agreements of NR carrier phase positioning in Rel-18 which introduces more scenarios to use PRU. The first scenario is to support UE based positioning using PRU measurements, and the second scenario is that LMF determines UE location based on UE measurements and PRU measurements in indicated time windows when network assisted positioning or UE assisted positioning is used. Based on the existing SA2 specification, the first scenario above is not supported. Furthermore, for the second scenario above, when the serving LMF of target UE and the serving LMF of PRU are different LMFs, the impacts to Nlmf interface are different in the case that network assisted positioning or UE assisted positioning is selected by the serving LMF of target UE. So it is proposed to extend PRU usage in core network to align SA2 specification and RAN specifications.

Furthermore, after user plane positioning is introduced in Rel-18 5G_eLCS_Ph3, for the messages (LPP messages, supplementary services messages) belonging to one positioning procedure, there is no common understanding on transferring the messages via user plane and control plane. Some companies believe that for the messages belonging to one LCS session, either control plane or user plane (i.e. one transmission path) should be used by UE and LMF. While some companies think it is flexible for LMF to choose control plane or user plane to transfer the messages above, e.g. some messages above are transferred via user plane and the others can be transferred via control plane.

Besides the above issue for user plane positioning, after the LMF selects the user plane or control plane positioning for one positioning procedure, during the positioning procedure, the transmission path may fail caused by control plane congestion in the case that the control plane positioning is selected or user plane positioning dedicated PDU Session release in the case that the user plane positioning is used. In this case, the LMF behaviour is also not clear. Especially, the PDU Session for user plane positioning may be released by different causes which can lead to different LMF behaviour. For example, if the PDU Session release is cause by handover or S-NSSAI congestion, it is impossible to re-establish the PDU Session. In this case, if the LMF re-triggers user plane connection establishment for multiple times, all the requests will be rejected by UE. To avoid the delay, the LMF needs to re-select control plane to re-trigger the positioning procedure if the release cause is handover or S-NSSAI congestion. Another example, the PDU Session may also be released to support service continuity (SSC mode 2), in this case, the PDU Session can be re-established, so the LMF can re-trigger the user plane connection establishment. So the LMF behaviour to support different PDU Session release cause also needs to be considered.

Furthermore, the user plane positioning causes different charging information compared with the control plane positioning. The charging difference leads to different user preference on user plane positioning and control plane positioning. To improve the user experience, the user preference above needs to be considered by LMF to select user plane or control plane positioning. However, this is not supported by existing specification.

Considering the issues related to user plane positioning are not in the scope of Rel-18 5G_eLCS_Ph3 WID, it is proposed to introduce the mechanism to solve the issues in Rel-19.

2. Discussion

2.1	Support of more scenarios to use PRU in core network

2.1.1	Network assisted positioning based on target UE measurements and PRU measurements in indicated time window(s)

The RAN1’s agreements related to network assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) is shown in Table 1.

Table 1: The RAN1’s agreements related to network assisted positioning based on target UE measurements and PRU measurements in indicated time window(s)

| RAN1 meeting | RAN1’s agreements |
| --- | --- |
| RAN1#112-bis | To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window |
| RAN1#113 | To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows |
| RAN1#114 | - When a LMF requests the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), - the duration of a time window can be configured by one of the following values: - {1, 2, 4, 8, 12} OFDM symbols - {1, 2, 4, 6, 8, 12, 16} slots - FFS: additional values - the number of the time windows can be configured as: - {1, 2, …, 16} When a LMF requests the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s): - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2, …, 16} |

To support the RAN1 agreements above, two IEs (Time Window Information SRS List and Time Window Information Measurement List) are introduced in TS 38.455.

If the serving LMF of target UE and the serving LMF of PRU are the same, the LMF is responsible to trigger positioning procedures for target UE and PRU separately to obtain the simultaneous measurements of target UE and PRU in the indicated time window(s). In this scenario, the SA2 specification is not impacted.

If the serving LMF of target UE and the serving LMF of PRU are different, the scenario is shown in Figure 1.

Figure 1: Two LMFs scenario for network assisted positioning

As shown Figure 1, the LMF1 is the serving LMF of target UE and the LMF2 is the serving LMF of PRU. gNB1, gNB2 and gNB3 are selected by LMF1 to take part in the measurement procedure. The serving gNB could also be one gNB taken part in the measurement procedure. For all the gNBs taken part in the measurement, they are routable for both LMF1 and LMF2. It is assumed that the OAM configures the LMF with the information of TRPs located in the border area of the neighbour LMF(s).

When the LMF1 determines to obtain simultaneous measurements of target UE and PRU in the indicated time window(s), the LMF1 determines the positioning method and the time window(s), selects TRPs which are responsible to measure the UL SRS from target UE and PRU in the indicated time window(s). The LMF1 provides the positioning method, time window(s) (i.e. Time Window Information SRS List and Time Window Information Measurement List specified in TS 38.455) and the identifiers of the selected TRPs to the LMF2. Based on the received information, the LMF2 selects PRU(s) and triggers network assisted positioning for the selected PRU(s) towards the selected TRPs and returns the PRU measurements to the LMF1. Based on measurements from target UE and PRU in the indicated time window(s), the LMF1 calculates the location of target UE.

Based on the analyses above, to support RAN1’s agreements in Table 1, the proposal is as follows:

Proposal#1: to support network assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) when the serving LMF of target UE and the serving LMF of PRU are different LMFs,

-	The serving LMF of target UE determines the positioning method and the time window(s), selects TRPs which are responsible to measure the UL SRS from target UE and PRU in the indicated time window(s).

-	The serving LMF of target UE provides the positioning method, time window(s) (i.e. Time Window Information SRS List and Time Window Information Measurement List specified in TS 38.455), and the identifier of the selected TRPs to the serving LMF of PRU.

-	Based on the received information, the serving LMF of PRU selects PRU(s), triggers network assisted positioning for the selected PRU(s) towards the selected TRPs, and returns the measurements related to PRU(s) in the indicated time window(s) to the serving LMF of target UE.

-	The serving LMF of target UE is responsible to calculate the location of target UE based on the measurements related to target UE and measurements related to the PRU.

2.1.2	UE assisted positioning based on target UE measurements and PRU measurements in indicated time window(s)

The RAN1’s agreements related to UE assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) are shown in Table 2.

Table 2: The RAN1’s agreements related to UE assisted positioning based on target UE measurements and PRU measurements in indicated time window(s)

| RAN1 meeting | RAN1’s agreements |
| --- | --- |
| RAN1#112-bis | To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. |
| RAN1#113 | To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.FFS: the maximum number of the windows |
| RAN1#114 | When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8} |

To support the RAN1 agreements above, a new IE (NR-DL-PRS-MeasurementTimeWindowsConfig) as follows is introduced in TS 37.355.

-- ASN1START

NR-DL-PRS-MeasurementTimeWindowsConfig-r18 ::=

SEQUENCE (SIZE(1..2)) OF

NR-DL-PRS-MeasurementTimeWindowsConfigElement-r18

NR-DL-PRS-MeasurementTimeWindowsConfigElement-r18 ::= SEQUENCE {

nr-StartSFN-TimeWindow-r18		INTEGER (0..1023),

nr-PeriodicOrOneShotTimeWindow-r18 CHOICE {

nr-PeriodicityAndSlotOffsetTimeWindow-r18

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

nr-OneShotSlotOffsetTimeWindow-r18 CHOICE {

scs15-r18						INTEGER (0..10239),

scs30-r18						INTEGER (0..20479),

scs60-r18						INTEGER (0..40959),

scs120-r18						INTEGER (0..81919)

}

}																		OPTIONAL, -- Need ON

nr-SymbolOffsetTimeWindow-r18	INTEGER (0..13)							OPTIONAL, -- Need ON

nr-DurationTimeWindow-r18		ENUMERATED { n1, n2, n4, n6, n8, n12, n16, ... },

nr-SelectedDL-PRS-FrequencyLayerIndex-r18	INTEGER (0..nrMaxFreqLayers-1-r16)

OPTIONAL, -- Need ON

nr-SelectedDL-PRS-IndexListPerFreq-r18

SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-SelectedDL-PRS-IndexPerTRP-r18	OPTIONAL, -- Need OP

nr-MeasurementsToPerformInTimeWindow-r18			BIT STRING { rstd (0),

ueRxTx (1),

rsrp (2),

rsrpp (3),

rscp (4),

rscpd (5)

} (SIZE(1..16)) OPTIONAL, -- Need ON

...

}

NR-SelectedDL-PRS-IndexPerTRP-r18 ::= SEQUENCE {

nr-SelectedTRP-Index-r18				INTEGER (0..nrMaxTRPsPerFreq-1-r16),

dl-SelectedPRS-ResourceSetIndexList-r18	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

INTEGER (0..nrMaxSetsPerTrpPerFreqLayer-1-r16)		OPTIONAL, --Need OP

...

}

-- ASN1STOP

If the serving LMF of target UE and the serving LMF of PRU are the same, the LMF is responsible to trigger positioning procedures for target UE and PRU separately to obtain the simultaneous measurements of target UE and PRU in the indicated time window(s). In this scenario, the SA2 specification is not impacted.

If the serving LMF of target UE and the serving LMF of PRU are different, the scenario is shown in Figure 2.

Figure 2: Two LMFs scenario for UE assisted positioning

As shown Figure 2, the LMF1 is the serving LMF of target UE and the LMF2 is the serving LMF of PRU. For all the gNBs (TRPs) taken part in the measurement, they are routable for both LMF1 and LMF2. It is assumed that the OAM configures the LMF with the information of TRPs located in the border area of the neighbour LMF(s).

When the LMF1 determines to obtain simultaneous measurements from target UE and PRU in the indicated time window(s), the LMF1 determines the positioning method and the time window(s). The LMF1 provides the positioning method and the time window(s) (i.e. NR-DL-PRS-MeasurementTimeWindowsConfig-r18 specified in TS 37.355) to the LMF2. Based on the received information, the LMF2 selects PRU(s) and triggers UE assisted positioning for the selected PRU(s) and returns the PRU measurements to the LMF1. Based on measurements from target UE and PRU in the indicated time window(s), the LMF1 calculates the location of target UE.

Based on the analyses above, to support RAN1’s agreements in Table 2, the proposal is as follows:

Proposal#2: to support UE assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) when the serving LMF of target UE and the serving LMF of PRU are different LMFs,

-	The serving LMF of target UE determines the positioning method and the time window(s) (i.e. NR-DL-PRS-MeasurementTimeWindowsConfig specified in TS 37.355).

-	The serving LMF of target UE provides the positioning method and the time window(s) (i.e. NR-DL-PRS-MeasurementTimeWindowsConfig specified in TS 37.355) to the serving LMF of PRU.

-	Based on the received information, the serving LMF of PRU selects PRU(s), triggers UE assisted positioning for the selected PRU(s), and returns the measurements related to PRU(s) in the indicated time window(s) to the serving LMF of target UE.

-	The serving LMF of target UE is responsible to calculate the location of target UE based on the measurements related to target UE and measurements related to the PRU.

2.1.3	UE-based positioning using PRU measurements

The RAN1’s agreements related to UE-based positioning using PRU measurements in Table 3.

Table 3: The RAN1’s agreements related to UE-based positioning using PRU measurements

| RAN1 meeting | RAN1’s agreements |
| --- | --- |
| RAN1#113 | For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability. - additional information of the same PRU includes at least PRU location. - FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc. |
| RAN1#114 | For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. |

When UE-based positioning is determined by LMF, if the LMF determines to use PRU, the LMF may provide the PRU measurements, the timestamp associated with the measurements and the PRU location to target UE. The information is included in the assistance data.

Proposal#3: When the LMF determines to use UE-based positioning and to use PRU to improve the positioning result, the LMF provides the assistance data including PRU measurements, the timestamp associated with the measurements and the PRU location to target UE. The target UE uses the assistance data to calculate location.

2.2	One Transmission Path for One Positioning Procedure

2.2.1	Background and Discussion

User plane positioning is introduced in Rel-18 5G_eLCS_Ph3 to transfer the LPP messages and supplementary service messages via user plane.

During the discussion in normative phase, for the messages (LPP messages, supplementary services messages) belonging to one LCS session, there is no common understanding on how to transfer the messages via user plane and control plane. Some companies believe that for the messages belonging to one LCS session, either control plane or user plane should be used by UE and LMF at the same time. While some companies think it is flexible for LMF to choose control plane or user plane to transfer the messages above, e.g. some messages above are transferred via user plane and the others could be transferred via control plane.

Besides the above issue for user plane positioning, after the LMF selects the user plane or control plane positioning for one positioning procedure, during the positioning procedure, the transmission path may fail caused by control plane congestion in the case that the control plane positioning is selected or user plane positioning dedicated PDU Session release in the case that the user plane positioning is used. In this case, the LMF behaviour is also not clear. Especially, the PDU Session for user plane positioning may be released by different causes which can lead to different LMF behaviour. For example, if the PDU Session release is cause by handover or S-NSSAI congestion, it is impossible to re-establish the PDU Session. In this case, if the LMF re-triggers user plane connection establishment for multiple times, all the requests will be rejected by UE. To avoid the delay, the LMF needs to re-select control plane to re-trigger the positioning procedure if the release cause is handover or S-NSSAI congestion. Another example, the PDU Session may also be released to support service continuity (SSC mode 2), in this case, the PDU Session can be re-established, so the LMF can re-trigger the user plane connection establishment. So the LMF behaviour to support different PDU Session release cause also needs to be considered.

Furthermore, the user plane positioning causes different charging information compared with the control plane positioning. The charging difference leads to different user preference on user plane positioning and control plane positioning. To improve the user experience, the user preference above needs to be considered by LMF to select user plane or control plane positioning. However, this is not supported by existing specification.

Considering the issues above are not in the scope of Rel-18 5G_eLCS_Ph3 WID, it is proposed to introduce the mechanism to ensure one transmission path used for one positioning procedure in Rel-19.

2.2.2	Proposal

If some messages belonging to one LCS session could be transferred via user plane and others could be transferred via control plane, the LMF and UE need to be enhanced to select user plane or control plane before sending a message. So this makes LMF and UE more complicated and increases positioning delay.

To reduce complicity and positioning delay, it is proposed that the messages belonging to one LCS session, either control plane or user plane should be used by UE and LMF to transfer the messages.

Considering the charging difference between user plane positioning and control plane positioning and the UE positioning capabilities, the UE provides UE preference on control plane and user plane positioning along with the user plane positioning capabilities. The LMF considers the UE preference to select a transmission path (i.e. control plane or user plane).

When LMF selects control plane or user plane for a positioning procedure, the mechanism to support one transmission path for the procedure is as follow:

-	When the LMF selects control plane for a positioning procedure,

-	The LMF sends the supplementary services messages and LPP messages over control plane to UE. If the UE receives the DL NAS TRANSPORT message including an LCS Corrleation ID, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the control plane.

-	During the positioning procedure, if the control plane transmission fails due to control plane congestion. The LMF determines to select user plane to re-trigger the positioning procedure if the user plane connection exists or re-trigger the positioning procedure over the control plane when control plane congestion ceases.

-	When the user plane is selected by LMF for a positioning procedure,

-	The LMF sends the supplementary services messages and LPP messages over user plane to UE. If the UE receives the user plane transport message including an LCS Corrleation ID from LMF, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the user plane.

-	During the postioning procedure, if the user plane positioning dedicated PDU Session is released based on the triggers described in clause 4.3.4.2 in TS 23.502[19], the SMF invokes the Nsmf_PDUSession_SMContextStatusNotify service operation to AMF to notify the PDU Session status(i.e. release) and the cause. The AMF notifies the PDU Session status(i.e. release) and the cause to LMF. Based on the notification, the LMF determines to select control plane to re-trigger the positioning procedure or to re-trigger the user plane connection establishment.

NOTE X:  As examples, if the PDU Session is released due to handover or S-NSSAI based congestion control as specified in clause 6.1.6.3.8 in TS 29.502[X], the LMF selects control plane to re-trigger the positioning procedure. If the PDU Session is released due to establish a new PDU Session with an identical PDU Session ID as specified in clause 6.1.6.3.8 in TS 29.502[X], the LMF re-triggers the user plane connection establishment.

The proposal is as follows.

Proposal#4:

The LMF is responsible to select control plane positioning or user plane positioning for a positioning procedure to transfer the supplementary services messages and LPP messages included in the procedure based on user plane positioning capability, control plane congestion status (e.g. AMF load status), UE preference on control plane and user plane positioning and other implementation factor. The UE preference on control plane and user plane positioning is determined by UE based on the charging information, cause value of the user plane positioning dedicated PDU Session release, user plane positioning capabilities and so on. The UE provides the preference along with the user plane positioning capabilities.

When the LMF selects control plane for a positioning procedure,

-	The LMF sends the supplementary services messages and LPP messages over control plane to UE. If the UE receives the DL NAS TRANSPORT message including an LCS Corrleation ID, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the control plane.

-	During the positioning procedure, if the control plane transmission fails due to control plane congestion. The LMF determines to select user plane to re-trigger the positioning procedure if the user plane connection exists or re-trigger the positioning procedure over the control plane when control plane congestion ceases.

When the user plane is selected by LMF for a positioning procedure,

-	The LMF sends the supplementary services messages and LPP messages over user plane to UE. If the UE receives the user plane transport message including an LCS Corrleation ID from LMF, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the user plane.

-	During the postioning procedure, if the user plane positioning dedicated PDU Session is released based on the triggers described in clause 4.3.4.2 in TS 23.502[19], the SMF invokes the Nsmf_PDUSession_SMContextStatusNotify service operation to AMF to notify the PDU Session status(i.e. release) and the cause. The AMF notifies the PDU Session status(i.e. release) and the cause to LMF. Based on the notification, the LMF determines to select control plane to re-trigger the positioning procedure or to re-trigger the user plane connection establishment.

NOTE X:	As examples, if the PDU Session is released due to handover or S-NSSAI based congestion control as specified in clause 6.1.6.3.8 in TS 29.502[X], the LMF selects control plane to re-trigger the positioning procedure. If the PDU Session is released due to establish a new PDU Session with an identical PDU Session ID as specified in clause 6.1.6.3.8 in TS 29.502[X], the LMF re-triggers the user plane connection establishment.

3. Proposal

Based on discussion in clause 2, the proposals are as follows.

- To support NR carrier phase positioning, the proposal includes:

Proposal#1: to support network assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) when the serving LMF of target UE and the serving LMF of PRU are different LMFs,

-	The serving LMF of target UE determines the positioning method and the time window(s), selects TRPs which are responsible to measure the UL SRS from target UE and PRU in the indicated time window(s).

-	The serving LMF of target UE provides the positioning method, time window(s) (i.e. Time Window Information SRS List and Time Window Information Measurement List specified in TS 38.455), and the identifier of the selected TRPs to the serving LMF of PRU.

-	Based on the received information, the serving LMF of PRU selects PRU(s), triggers network assisted positioning for the selected PRU(s) towards the selected TRPs, and returns the measurements related to PRU(s) in the indicated time window(s) to the serving LMF of target UE.

-	The serving LMF of target UE is responsible to calculate the location of target UE based on the measurements related to target UE and measurements related to the PRU.

Proposal#2: to support UE assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) when the serving LMF of target UE and the serving LMF of PRU are different LMFs,

-	The serving LMF of target UE determines the positioning method and the time window(s) (i.e. NR-DL-PRS-MeasurementTimeWindowsConfig specified in TS 37.355).

-	The serving LMF of target UE provides the positioning method and the time window(s) (i.e. NR-DL-PRS-MeasurementTimeWindowsConfig specified in TS 37.355) to the serving LMF of PRU.

-	Based on the received information, the serving LMF of PRU selects PRU(s), triggers UE assisted positioning for the selected PRU(s), and returns the measurements related to PRU(s) in the indicated time window(s) to the serving LMF of target UE.

-	The serving LMF of target UE is responsible to calculate the location of target UE based on the measurements related to target UE and measurements related to the PRU.

Proposal#3: When the LMF determines to use UE-based positioning and to use PRU to improve the positioning result, the LMF provides the PRU measurements, the timestamp associated with the measurements and the PRU location to target UE. The target UE uses the information to calculate location.

- To support one transmission path used for one positioning procedure, the proposal includes:

The LMF is responsible to select control plane positioning or user plane positioning for a positioning procedure to transfer the supplementary services messages and LPP messages included in the procedure based on user plane positioning capability, control plane congestion status (e.g. AMF load status), UE preference on control plane and user plane positioning and other implementation factor. The UE preference on control plane and user plane positioning is determined by UE based on the charging information, cause value of the user plane positioning dedicated PDU Session release, UE positioning capabilities and so on. The UE provides the preference along with the UE positioning capabilities.

When the LMF selects control plane for a positioning procedure,

-	The LMF sends the supplementary services messages and LPP messages over control plane to UE. If the UE receives the DL NAS TRANSPORT message including an LCS Corrleation ID, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the control plane.

-	During the positioning procedure, if the control plane transmission fails due to control plane congestion. The LMF determines to select user plane to re-trigger the positioning procedure if the user plane connection exists or re-trigger the positioning procedure over the control plane when control plane congestion ceases.

When the user plane is selected by LMF for a positioning procedure,

-	The LMF sends the supplementary services messages and LPP messages over user plane to UE. If the UE receives the user plane transport message including an LCS Corrleation ID from LMF, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the user plane.

-	During the postioning procedure, if the user plane positioning dedicated PDU Session is released based on the triggers described in clause 4.3.4.2 in TS 23.502[19], the SMF invokes the Nsmf_PDUSession_SMContextStatusNotify service operation to AMF to notify the PDU Session status(i.e. release) and the cause. The AMF notifies the PDU Session status(i.e. release) and the cause to LMF. Based on the notification, the LMF determines to select control plane to re-trigger the positioning procedure or to re-trigger the user plane connection establishment.