3GPP TSG-WG SA2 Meeting #164	S2-2408577

# Maastricht, NL, 19th Aug – 23rd Aug, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1453 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on UE-satellite-UE communicaitons |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of features to support UE-satellite-UE communications for satellite communications based on the conclusions for KI#3 in TR 23.700-29. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the references to include TS 23.228 and TS 29.224 - Update the definitions to include UE-satellite-UE communication - Add a new clause to show the architecture of UE-satellite-UE communication - Add a new clause to describe the feature of UE-satellite-UE communication - Add a new clause to describe the feature of when UPF and AGW onboard - Add a new clause to describe the feature of activation of UE-S-UE communication |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-satellite-UE communication will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 4.x(new), 5.4.x.1(new), 5.4.x.2(new), 5.4.x.2.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

XY.3	High level features

XY.3.1	Activation of UE-satellite-UE communications with AGW onboarding

The determination of activating UE-satellite-UE communication is executed separately at the MO/MT UE side. During the call setup period, the serving P-CSCF determines to activate the UE-Satellite-UE communication if

- the UE is accessing via a satellite (constellation) that has gNB, UPF and IMS-AGW onboard, and has subscribed to the UE-SAT-UE communication service;

- the other UE is also accessing via a satellite (constellation) that has gNB, UPF and IMS-AGW onboard, and has subscribed to the UE-SAT-UE communication service, and;

- both UEs’ serving satellite(s) (constellation) can communicate.

To support the determination of the UE-satellite-UE communication activation, the serving P-CSCF should obtain the satellite ID and access information from PCF, and UE’s subscription information from AS.

Editor’s Note: How to obtain the other UE’s access information, and subscription info is FFS.

NOTE: 		How the P-CSCF obtains the connection relationship between the MO UE’s serving satellite (constellation) and MT UE’s serving satellite (constellation) is out of 3GPP scope.

If the serving P-CSCF determines to activate the UE-satellite-UE communication, the P-CSCF should indicates an indication indicating that the UE-Satellite-UE call should be performed to the PCF and the PCF forward the indication to the SMF.

The serving P-CSCF (IMS-ALG) also selects the onboarding IMS-AGW as the media gateway and informs the onboarding IMS-AGW to allocate IP addresses to both the MO side and the MT side, sends the allocated IP addresses to UE and the other P-CSCF (IMS-ALG) as specified in clause G.4.3.

Figure XY.3-1: IMS signalling and media path before activating the onboarding UPF and IMS-AGW

Figure XY.3-1 depicts the IMS signalling path between UE 1 and UE 2 before activating the UE-satellite-UE communication link:

-	UE-1 sends the IMS signalling data (e.g. RTP packets) to the onboarding gNB and the IMS anchor UPF (via UE’s PDU session) to the serving P-CSCF.

Figure XY.3-2: IMS signalling and media path after activating the onboarding UPF and IMS-AGW

Figure XY.3-2 depicts the IMS signalling and media path between UE 1 and UE 2 after activating the UE-satellite-UE communication link:

-	UE sends IMS signalling data via the onboarding gNB and the inserted UL CL to the serving P-CSCF

-	UE-1 sends the voice media packets (e.g. RTP packets) to the onboarding gNB and local PSA (via UE-1’s PDU session), and the local PSA forwards the voice packet to the onboarding IMS-AGW (via N6).

-	UE-1’s onboarding IMS-AGW forwards the voice packet to the UE-2’s onboarding IMS-AGW based on the address information exchanges during the call setup period.

-	The onboarding IMS-AGW then forwards the voice packet to the onboarding local PSA ( via N6) and finally to the UE-2 (via UE-2’s PDU session).

# * * * * End of changes * * * *