3GPP TSG-WG SA2 Meeting #164	S2-2408694

Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-2408057)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4923 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription of UPF event via I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Nokia, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2024-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For certain UE(s) case, to get exposure data from UPF, NF consumer may subscribe to the UPF indirectly via SMF. But, if the UPF is inserted by I-SMF that SMF cannot serve the target DNAI for the traffic routing for local access to the DN, the SMF can’t directly subscribe the event in UPF. The SMF should send the subscription to UPF via I-SMF. For this case, the SMF should sent the subscription to I-SMF, and the I-SMF represents the SMF to subscribe to UPF. For any UE case, the I-SMF can be discovered by NF consumer and subscribed directly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For certain UE(s) case, if I-SMF is used for this PDU session and the UPFs controlled by the I-SMF, the SMF sends the the UPF event subscription related information to I-SMF. For any UE case, the I-SMF can be discovered by NF consumer and subscribed directly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to SMF subscribe the event to the UPF/DNAI that SMF not control is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.2, 4.15.4.5.3, 4.23.9.1, 5.2.8.1, 5.2.8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

##### 4.15.4.5.2	Information flow for subscription to UPF event exposure service for certain UE(s) via SMF

Figure 4.15.4.5.2-1: Subscription to UPF event exposure service for certain UE(s) via SMF

In the case of a group of UEs, the UPF event consumer (e.g. NWDAF) first issues an Nnrf_NFDiscovery_Request service operation to find the UDM providing the target Group ID and gets the NF profile of the UDM serves this group. Then, NWDAF obtains the list of SUPIs that correspond to the Group ID from UDM using Nudm_SDM_Get

NOTE 1:	It is assumed that all members of a Group ID belong to the same UDM.

Then, for each SUPI:

1.	The UPF event consumer (e.g. NWDAF) invokes Nudm_UECM_Get service operation to retrieve the appropriate SMF by providing UE ID, DNN, S-NSSAI and NF type = SMF.

2.	The UDM provides a Nudm_UECM_Get response with the corresponding SMF.

3.	The UPF event consumer sends the Nsmf_EventExposure Subscription request to the SMF to subscribe to UPF data, including the following information:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	Indication of UPF Event Exposure Service and Target subscription UPF Event Id.

-	Event Filter Information: S-NSSAI, DNN, DNAI,UPF Id, Traffic Description for the target traffic (e.g. Application Id), Area of Interest, SSID/BSSID.

-	Target of Event Reporting: a UE.

-	Reporting suggestion information.

-	Target Subscription information: Type of Measurement and granularity of the information requested.

If the consumer is NWDAF and the analytic filter information includes application server IP address/FQDN, the NWDAF may need to first obtain the DNAI from NEF as described in steps 2 and 3 in Figure 4.15.4.5.3-1.

4.	The SMF selects the PDU session(s) and the UPFs it has to send the request to. The SMF sends the request to the UPF including the UPF event consumer address, Notification Correlation Information, Event Filter Information, Reporting suggestion information, Target of Event Reporting and Target Subscription Information as required. Target of Event Reporting is a certain UE. The interaction mechanism used between SMF and UPF depends on UPF exposure event and which mechanism applies for each event as described in clause 5.2.26.2.1. For some events, the SMF shall contact UPF (4a) with N4 Session Modification with PFCP (TS 29.244 [69]), for other events (4b) with Nupf_event exposure subscribe request (as defined in clause 5.2.26.2.3).

In case 4b, if an I-SMF is used for this PDU session, for UPF events (except the QoS monitoring) involving UPFs controlled by the I-SMF, the SMF sends the Nsmf_EventExposure_Subscribe including the UPF event subscription related information to I-SMF if relaying of UPF events/service is supported by the I-SMF and I-SMF forwards the subscription to the local UPF.

NOTE 2:	Some events can require SMF interacts with RAN at this stage.

5.	Per Reporting suggestion information (if available), the UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

* * * Start of Change 2 * * *

##### 4.15.4.5.3	Information flow for UPF event exposure service for any UE

Figure 4.15.4.5.3-1: UPF Information Exposure to the UPF event consumer (e.g. NWDAF) of any UE scenario

1.	(For the case when the UPF event consumer is NWDAF) The analytics consumer sends a request to the NWDAF for analytics on any UE. The analytics consumer provides the value "any UE" in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, Application server IP address/FQDN, Application ID, DNAI, etc.

2.	(Optional and only when the UPF event consumer is NWDAF) If the Analytic Filter Information does not contain DNN/S-NSSAI, but only includes application server IP address/FQDN, the NWDAF should first obtain the DNAI from NEF. The NWDAF invokes Nnef_DNAIMapping_Subscribe service to request the DNAI information. The request includes EAS IP/IP range and/or FQDN.

3.	(Conditional, if step 2 took place) The NEF determines the suitable DNAI(s) and provides them to NWDAF.

4.	The UPF event consumer triggers the SMF(s)/UPF(s) discovery to NRF by Nnrf_NFDiscovery_Request providing the DNN, S-NSSAI, DNAI etc. This procedure is to discover the related SMF(s)/UPF(s) associated with any UE and support the indicated DNAI. Either SMF(s) or UPF(s) are discovered depending on whether the subscription request to UPF events meets the criteria for direct subscription to UPF as defined in clause 5.8.2.17 of TS 23.501 [2]).

5.	The NRF provides Nnrf_NFDiscovery_Response that may refer to several SMFs/UPFs.

6.	(Option 1) If the subscribed UPF events needs the SMF(s) to do a third-party subscription onto UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the same procedure as Indirect subscription via several SMFs (steps 3 - 5 in Figure 4.15.4.5.2-1(for single UE)) takes place via each discovered SMF.If the Local-UPF(s) controlled by the I-SMF(s), the UPF event consumer sends Nsmf_EventExposure_Subscribe to this I-SMF including the UPF event subscription related information.

7.	(Option 2) If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to all discovered UPFs. The information included in the subscription is:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	UPF Event Id.

-	Event Filter Information, i.e. one or more of the following parameters: S-NSSAI, DNN, either Application Id(s) or Traffic Filtering Information.

-	Target of Event Reporting: any UE.

-	Reporting suggestion information.

-	Type of Measurement and Granularity of Measurement.

NOTE:	As the Target of Event Reporting is "any UE", care needs to be taken with regards to load and major signalling impacts and it is expected that the subscription request contains Event Filter Information and appropriate Event Reporting Information (like Sampling ratio and Partitioning criteria, see Table 4.15.1-1) in order to minimize the UPF performance impact.

8.	Each of the UPFs invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

* * * Start of Change 3 * * *

#### 4.23.9.1	Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF

This clause describes a procedure to add a PDU Session Anchor and Branching Point or UL CL controlled by I-SMF.

Figure 4.23.9.1-1: Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF

1.	UE has an established PDU Session with a UPF including the PDU Session Anchor 1, which is controlled by SMF. The I-SMF and an I-UPF controlled by I-SMF have already been inserted for the PDU Session. Events described in item 1 and 2 of clause 4.23.9.0 have taken place.

2.	At some point, using the list of DNAI(s) of interest for this PDU Session received from the SMF, the I-SMF decides to establish a new PDU Session Anchor e.g. due to UE mobility. The I-SMF selects a UPF and using N4 establishes the new PDU Session Anchor 2 (PSA2) of the PDU Session. During this step:

-	(if needed) the PSA2 CN Tunnel Info of the local N9 termination on the PSA2 may be determined,

-	In the case of IPv6 multi-homing applies to the PDU Session, a new IPv6 prefix corresponding to PSA2 is allocated by the I-SMF or by the UPF supporting the PSA2.

3.	The I-SMF may select a UPF that will be acting as UL CL or Branching Point and replace the current I-UPF.

If a new UPF that will act as UL CL/Branching Point is selected (i.e. the existing I-UPF is replaced), the I-SMF uses N4 establishment to provide the 5G AN Tunnel Info, the PSA1 and (where applicable) PSA2 CN Tunnel Info to the new UPF.

NOTE 1:	If the Branching Point or UL CL and the PSA2 are co-located in a single UPF then steps 2 and 3 can be merged.

4.	The I-SMF invokes Nsmf_PDUSession_Update Request (Indication of UL CL or Branching Point insertion, IPv6 prefix @PSA2, DNAI(s) supported by PSA2, DL Tunnel Info of the new UL CL/Branching Point, if any) to SMF. Separate N4 contents are exchanged over N16a for the local UL CL/BP(s) and for the local PSA(s) controlled by the I-SMF  Multiple local PSAs (i.e. PSA2) may be inserted at one time, each corresponds to a DNAI and/or an IPv6 prefix in the case of multi-homing.

The I-SMF informs the SMF that a UL CL or Branching Point is inserted, the I-SMF provides DNAI(s) supported by PSA2 to the SMF. The DL Tunnel Info of UL CL/Branching Point is provided to SMF if a new UPF is selected to replace I-UPF in step 3.

In the case of IPv6 multi-homing PDU Session, the IPv6 prefix @PSA2 is also provided to SMF.

The SMF performs the Session Management Policy Modification procedure as defined in clause 4.16.5 to provide the new allocated IPv6 prefix to the PCF. The SMF may also send a notification to the AF, as described in clause 4.3.6.3.

The DNAI(s) supported by PSA2 may be used by the SMF to determine which PCC rules are to be applied at UPF(s) controlled by the I-SMF. The SMF acknowledges the Nsmf_PDUSession_Update from the I-SMF

5.	If a new DL Tunnel Info of UL CL/ Branching Point has been provided in step 4, the SMF updates the PSA1 via N4 with the CN Tunnel Info for the downlink traffic. Now the downlink packets from PSA1 are sent to UE via the new UPF which will act as Branching Point/UL CL. The SMF may also update the forwarding rules in PSA1 if some traffic is to be moved to UPFs controlled by I-SMF.

6.	The SMF provides I-SMF with N4 information for the PSA and for the UL CL with a SMF initiated Nsmf_PDUSession_Update Request (set of (N4 information, involved DNAI), Indication of no DNAI change, Indication of no local PSA change)). The SMF generates N4 information for local traffic handling based on PCC rules and CHF requests that will be enforced by UPFs controlled by I-SMF. The N4 information for local traffic handling corresponds to N4 rules (PDR, FAR, URR, QER, etc.) related with the support of a DNAI. This is described in clause 5.34.6 of TS 23.501 [2]. N4 information for local traffic handling may indicate information (as the 5G AN Tunnel Info) that the SMF does not know and that the I-SMF needs to determine itself to build actual rules sent to the UPF(s). If the rule is applied to the local PSA, the N4 information includes the associated DNAI.

If the "Indication of application relocation possibility" or "UE IP address preservation indication" attributes are included in the PCC rule, the SMF includes the corresponding Indication of no DNAI change and Indication no local PSA change respectively.

If the CN Tunnel Info at the PSA1 has changed, the SMF may also provide its new value.

The I-SMF uses N4 information for local traffic handling received from the SMF as well as 5G AN Tunnel Info received from the 5G AN via the AMF and local configuration to determine N4 rules to send to the UPF(s) it is controlling.

7.	The I-SMF updates the PSA2 via N4 providing N4 rules determined in step 6. It also provides the Branching Point or UL CL CN Tunnel Info for down-link traffic if the PSA2 and the UL CL/Branching Point are supported by different UPF(s).

8. The I-SMF updates the Branching Point or UL CL via N4 providing N4 rules determined in step 6.

NOTE 2:	If the Branching Point or UL CL and the PSA2 are co-located in a single UPF then step 7 and step 8 can be merged.

9.	The I-SMF Issues a Nsmf_PDUSession_Update Response to SMF that may include N4 information received from the local UPF(s).

10.	Steps 6-8 of clause 4.3.5.4 are performed. In the case of IPv6 multi-homing PDU Session, the SMF notifies the UE of the IPv6 prefix @PSA2 and updates the UE with IPv6 multi-homed routing rule via a PSA controlled by the SMF.

NOTE 3:	Step 6 of clause 4.3.5.4 is skipped if the current I-UPF is selected to act as Branching Point or UL CL.

11.	If a new UPF is selected to replace I-UPF in step 3, the I-SMF uses N4 Release to remove the I-UPF of the PDU Session. The I-UPF releases resources for the PDU Session.

* * * Start of Change 4 * * *

#### 5.2.8.1	General

The following table shows the SMF Services and SMF Service Operations.

Table 5.2.8.1-1: NF services provided by the SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_PDUSession | Create | Request/Response | V-SMF/I-SMF |
|  | Update | Request/Response | V-SMF/I-SMF, H-SMF |
|  | Release | Request/Response | V-SMF/I-SMF |
|  | CreateSMContext | Request/Response | AMF |
|  | UpdateSMContext | Request/Response | AMF |
|  | ReleaseSMContext | Request/Response | AMF |
|  | SMContextStatusNotify | Subscribe/Notify | AMF |
|  | StatusNotify | Subscribe/Notify | V-SMF/I-SMF |
|  | ContextRequest | Request/Response | AMF, V-SMF/I-SMF, SMF |
|  | ContextPush | Request/Response | SMF |
|  | SendMOData | Request/Response | AMF |
|  | TransferMOData | Request/Response | V-SMF/I-SMF |
|  | TransferMTData | Request/Response | SMF, H-SMF |
| Nsmf_EventExposure | Subscribe | Subscribe/Notify | NEF, AMF, NWDAF, SMF |
|  | Unsubscribe |  | NEF, AMF, NWDAF, SMF |
|  | Notify |  | NEF, AMF, NWDAF |
|  | AppRelocationInfo |  | AF |
| Nsmf_NIDD | Delivery | Request/Response | NEF |
| Nsmf_TrafficCorrelation | Notify | Subscribe/Notify | NEF |

* * * Start of Change 5 * * *

##### 5.2.8.3.1	General

Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on PDU Sessions. The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for an Event ID on PDU Session(s);

-	Allow the NWDAF to collect data for network data analytics from SMF as specified in TS 23.288 [50] and from UPF as specified in clause 4.15.4.5;

-	Notifying events on the PDU Session to the subscribed NFs; and

-	Allow consumer NFs to acknowledge or respond to an event notification.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE IP address / Prefix allocation/change: The event notification may contain a new UE IP address / Prefix or an indication of which UE IP address / Prefix has been released.

-	PDU Session Establishment and/or PDU Session Release.

The event notification may contain following information:

-	PDU Session Type.

-	DNN.

-	UE IP address/Prefix.

-	UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed.

The event notification may contain following information:

-	the type of notification ("EARLY" or "LATE").

-	for both the source and target UP path between the UE and the DN, the corresponding information is provided when it has changed:

-	DNAI.

-	UE IP address / Prefix.

-	N6 traffic routing information.

-	Candidate DNAI(s) for the PDU Session.

-	Change of common EAS.

NOTE 1:	UP path change notification, DNAI and N6 traffic routing information are further described in clause 5.6.7 of TS 23.501 [2].

-	QoS Monitoring: the event notification may contain the QoS Monitoring report for the QoS parameter(s) to be measured defined in clause 5.45 of TS 23.501 [2]. Implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) may trigger SMF to send this event notification.

-	Change of Access Type; The event notification contains the new Access Type for the PDU Session. For MA PDU Session the Change of Access Type may include two Access Type information that the user is currently using.

-	Change of RAT Type; the event notification contains the new RAT Type for the PDU Session.

-	PLMN change; The event notification contains the new PLMN Identifier for the PDU Session and may indicate:

-	whether local traffic offload is possible, i.e., mobility of the PDU session either towards HPLMN or towards a VPLMN where HR-SBO is supported and allowed; and

-	DNN and S-NSSAI of HPLMN.

-	Change of Satellite backhaul category; The event notification contains the new Satellite backhaul category for the PDU session.

-	Downlink data delivery status. The event notification contains the status of downlink data buffering in the core network including:

-	First downlink packet per source of the downlink IP traffic in extended buffering and Estimated maximum wait time.

-	First downlink packet per source of the downlink IP traffic discarded.

-	First downlink packet per source of the downlink IP traffic transmitted after previous buffering and/or discarding of corresponding packet(s).

-	QFI allocation: The event notification is sent whenever a new QoS Flow is established and contains:

-	If the Target of Event Reporting is a PDU Session and the QoS Flow is associated with this PDU Session, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI corresponding to the QoS Flow and the DNN, S-NSSAI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is a SUPI and the PDU Session is associated with this SUPI, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI corresponding to the QoS Flow and the DNN, S-NSSAI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is an Internal-Group-Id and the PDU Session is associated with this Internal-Group-Id (i.e. the PDU Session belongs to a UE belonging to this Internal-Group-Id), both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI corresponding to the QoS Flow and the DNN, S-NSSAI, PDU Session ID, SUPI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is any UE, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI corresponding to the QoS Flow and the DNN, S-NSSAI, PDU Session ID, SUPI corresponding to the PDU Session are also sent.

NOTE 2:	When the consumer NF is the NWDAF, the event QFI allocation is used to collect data for analytics as specified in TS 23.288 [50].

-	Total number of Session Management transactions:

-	The total number of Session Management transaction is used to collect the number of SM transactions of a SUPI or Internal Group ID, for example Dispersion Analytics as specified in TS 23.288 [50]. The transaction count is incremented when the NAS transactions from PDU Session Establishment, PDU Session Authentication, PDU Session Modification and PDU Session Release procedures is concluded. Only the periodic reporting mode applies.

-	Information on PDU Session for WLAN (i.e. Access Type is Non-3GPP and RAT Type is TRUSTED_WLAN).

-	User plane status information: The event notification contains:

-	PDU Session ID.

-	User Plane Inactivity Timer (as specified in TS 29.244 [69]).

-	PDU Session status (activated, deactivated).

NOTE 3:	When the consumer NF is the NWDAF, the event user plane status information is used to collect data for UE Communication analytics as specified in TS 23.288 [50].

-	Session Management Congestion Control Experience for PDU Session: The event notification contains the data related to Session Management Congestion Control experience per PDU Session as described in TS 23.288 [50].

-	UE session behaviour trends (see clause 4.15.4.3);

-	UE communications trends (see clause 4.15.4.3);

-	UP with redundant transmission: the event notification indicates if redundant transmission (see clause 5.33.2.2 of TS 23.501 [2]) has been activated or not for the PDU session;

-	User Data Usage Measures (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF; For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of SMF. Consumer receives the events directly from UPF.

-	User Data Usage Trends (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF. For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of the SMF. Consumer receives the events directly from UPF.

When the consumer NF is the NWDAF, the event Information on PDU Session for WLAN is used to collect data for WLAN performance analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the event Session Management Congestion Control Experience for PDU Session is used to collect data for Session Management Congestion Control Experience analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the events QoS Monitoring, User Data Usage Measures and User Data Usage Trends are used to collect data from UPF for analytics as specified in clause 4.15.4.5 and in TS 23.288 [50]. SMF conveys the subscription to UPF on behalf of the NWDAF or the SMF conveys the subscription to I-SMF on behalf of the NWDAF and the I-SMF conveys the subscription to UPF on behalf of SMF.

The consumer NF may request to subscribe the UPF exposure events (including event ID of exposed UPF event of QoS monitoring, User Data Usage Measures and User Data Usage Trends) via SMF indirectly by Nsmf_EventExposure. After receiving this subscription request, the SMF does a third-party subscription onto UPF on behalf of this consumer. The consumer should also provide the subscribed UPF event to SMF.

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for SMF exposure.

Table 5.2.8.3.1-1: Example of Event Filters for SMF exposure events

| Event ID for SMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| DNAI Change | None |
| Candidate DNAI(s) has changed | None |
| PDU Session Release | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| PDU Session Establishment | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QoS Monitoring | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = AoI, value = AoI1> <Parameter Type = UPF Id, value = UPF Id1> <Parameter Type = DNAI, value = DNAI1> |
| QFI allocation | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QFI allocation | <Parameter Type = Application Identifier, Value = Application Identifier1> |
| Transaction Count | <Parameter Type = TAI, Value = TA1> (NOTE) <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| User plane status information | <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = SUPI, Value = SUPI1> |
| Information on PDU Session for WLAN | <Parameter Type = Access Type, Value = Non-3GPP> && <Parameter Type = RAT Type, Value = TRUSTED_WLAN> |
| Session Management Congestion Control Experience for PDU Session | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| UP with redundant transmission | <Parameter Type = DNN, Value = DNN1> |
| User Data Usage Measures | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) <Parameter Type = AoI, value = AoI1> <Parameter Type = SSID/BSSID, Value = SSID/BSSID1> |
| User Data Usage Trends | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) <Parameter Type = AoI, value = AoI1> |
| NOTE 1: Optionally the SMF can fetch the location information from the AMF but transaction information correlation at the location can also be achieved without it and through transaction information associated with the requested time period, which corresponds to the UE's time span at the location of interest. NOTE 2: These Parameters are exclusive and only one of them can be provided. |  |

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI), an Internal Group Identifier, an indication that any UE is targeted (e.g. on a specific DNN), or an indication that any PDU session is the target.

When acknowledgment is expected the SMF also provides Notification Correlation Information to the consumer NF in the event notification.

The consumer NF may provide the following event-specific information when acknowledging an event notification:

-	For UP path change event:

-	N6 traffic routing information related to the target DNAI.

NOTE 4:	Acknowledgement to a UP path change event notification is further described in clause 5.6.7 of TS 23.501 [2].

* * * End of Change * * *