3GPP TSG-SA2 Meeting #164	S2-2408697

# Maastricht, Netherlands, 19 - 23 August 2024	       		   				    (revision of S2-2407608)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4851 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscriptions to UPF event Exposure targeting an individual UE for non IP PDU Sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2024-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | S2-2405691, now approved, 23.502 CR 4703 removed the possibility of SMF providing the SUPI as Input parameter in subscriptions to UPF event Exposure (Table 4.15.4.5.1-1) when the subscription target is a UE and specified that the UE IP address is the input to UPF in that case. This raises following questions:What about Ethernet PDU Sessions? |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Ethernet PDU sessions - the UE identification input to the UPF is the SUPI when the UPF is allowed by local SMF configuration to receive the SUPI. - Otherwise for Ethernet PDU session type, UPF event Exposure targeting a UE is not supported when the UPF is not allowed to receive the SUPI This is due to the fact that the SMF cannot generally know in advance the MAC addresses that are going to be used at the UE side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to request UPF event exposure for Ethernet PDU Sessions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.1 ; 5.2.26.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 4.15.4.5.1	General

This clause contains the detailed description and the procedures for how the UPF event exposure service (see clause 5.2.26.2) is used for UPF data collection.

The list of NF consumer which may receive UPF event notifications is defined in clause 5.8.2.17 of TS 23.501 [2].

To get exposure data from UPF, NF consumer may subscribe to the UPF directly or indirectly via SMF. This is further defined in clause 5.8.2.17 of TS 23.501 [2].

The UPF event exposure events are described in clause 5.2.26.2. In this Release of the specification, the following events are used for UPF Data collection:

-	QoS Monitoring. This event provides QoS Flow performance information.

-	User DataUsage Measures. This event provides information of user data usage of the User PDU Session.

-	User DataUsage Trends. This event provides statistics related to user data usage of the User PDU Session.

A consumer of UPF event exposure can subscribe to QoS monitoring event via SMF only and UPF sends the QoS Flow Performance information directly to this consumer. For this event, the interaction between SMF and UPF is over PFCP (TS 29.244 [69]). TS 23.501 [2] describes the QoS monitoring parameters that can be measured by means of QoS monitoring and how to enable the measurements for QoS flows. When the Subscription request for QoS monitoring event indicates that it is for the QoS Flow associated with the default QoS rule, based on local configuration the subscription request may trigger SMF to enable QoS monitoring.

NOTE 1:	Packets from multiple applications may share the QoS Flow associated with the default QoS Rule which may diminish the relevance of some measurements like data rate.

The subscription to QoS monitoring event can target the QoS flows bound to an application by including an Application Identifier. In this case, at subscription request and/or when the PCC rules change, SMF identifies the active PCC Rule that includes a DataCollection_ApplicationIdentifier matching that Application Identifier. SMF enables this consumer (e.g. NWDAF) to receive the QoS Monitoring reports enabled by that PCC Rule. The consumer may indicate that it can receive QoS Flow Performance information for the QoS Flow associated with the default QoS rule if there are no measurements available for the Application Identifier (that is, if no PCC rule is identified). In this case the SMF may instruct the UPF to perform QoS monitoring for the QoS Flow associated with the default QoS rule and include the Indication of QoS Flow associated with the default QoS Rule (see clause 5.8.2.18 of TS 23.501 [2]). The UPF will then include the Indication of QoS Flow associated with the default QoS Rule in the Nupf_EventExposure_Notify service operation when sending reports. Otherwise, the SMF may accept the request and indicate in the response that reporting will be activated when the measurements are enabled by a PCC rule or the SMF may reject the subscription request for that Application Identifier.

NOTE 2:	Extensive usage of QoS Monitoring has significant impact on load and signalling.

A consumer of UPF event exposure such as NWDAF may subscribe to User Data Usage events (i.e. User Data Usage Measures and User Data Usage Trends) directly to UPF (under the conditions defined in clause 5.8.2.17 of TS 23.501 [2]) or via SMF, and UPF sends the event notifications directly to this consumer. For these events, the interaction between SMF and UPF is over SBI. For User Data Usage events, the subscription request targets the traffic matching Event Filter Information (details are described in clause 5.2.26.2.1) for a user's PDU Session or for each PDU Session served by the UPF. The subscription request may indicate the granularity requested, that is whether the measurement reports should be provided per data flow, per application, or per PDU Session.

If the event notification can be delayed, i.e. delay tolerant, Reporting suggestion information is included. The Reporting suggestion information includes Report urgency and Reporting time information. Reporting urgency information indicates whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting time is also provided, which defines the last valid reporting time and UPF shall report the detected event before the last valid time.

If a consumer subscribes to an UPF event via the SMF including an AoI ("Area of Interest"), the SMF starts the subscription to the UPF only when the UE is located in the requested AoI. When the UE leaves the AoI, the SMF stops the subscription on the UPF. The SMF may subscribe to AMF about UE moving in or out of an AoI for this purpose. Clause 4.15.4.5.4 describes an alternative procedure for subscription via SMF to UPF event exposure service related with AOI.

Table 4.15.4.5.1-1: Input parameters in subscription to UPF event Exposure events

| Information | To SMF | To UPF (NOTE 4) |
| --- | --- | --- |
| UE identification (NOTE 5) | Y | Y |
| GroupID | Y | N |
| Any UE | Y | Y |
| DNN | Y | Y (NOTE 6) |
| S-NSSAI | Y | Y(NOTE 6) |
| AOI | Y | N |
| BSSID/SSID | Y | N |
| DNAI (NOTE 3) | Y | N |
| UPF Id (NOTE 3) | Y | not applicable |
| Type of Measurement | Y | Y |
| Granularity of Measurement (NOTE 2) | Y | Y |
| Application ID (NOTE 1) | Y | Y |
| Traffic Filtering Information (NOTE 1), (NOTE 2) | Y | Y |
| Reporting suggestion information | Y | Y |
| NOTE 1: Application ID and Traffic Filtering Information are exclusive. NOTE 2: This parameter does not apply to QoS monitoring event. NOTE 3: This parameter is used to indicate a UP Path as defined in Table 6.4.1-1 of TS 23.288 [50]. NOTE 4: This column is not applicable to events where SMF-UPF interactions are using PFCP (e.g. QoS Monitoring event). That interaction is described in clause 5.8.5 of TS 23.501 [2]. NOTE 5: Input parameter when the target is a UE. When the target is a UE, the SUPI is the the UE identification input to SMF. For IP PDU session type, the UE identification input to the UPF shall be the UE IP address associated with the PDU session. For non-IP PDU session types, when the UPF is allowed by local SMF configuration to receive the SUPI associated with a N4 session, the UE identification input to the UPF is the SUPI. UPF event Exposure targeting a UE is not supported for non-IP PDU session types, when the UPF is not allowed by local SMF configuration to receive the SUPI associated with a N4 session.. NOTE 6: In a deployment where UPF reporting filtered per DNN and /or S-NSSAI is expected, the SMF should provide UPF with the necessary information over N4. If the SMF is not configured to provide UPF with this information then such UPF reporting filtering is not possible. |  |  |

NEXT CHANGE (2)

##### 5.2.26.2.3	Nupf_EventExposure_Subscribe service operation

Service operation name: Nupf_EventExposure_Subscribe

Description: This service operation is used by an NF to subscribe or modify a subscription to UPF event exposure notifications e.g. for the purpose of UPF data collection on a specified PDU Session or for all PDU Sessions of one UE or any UE.

Input, Required: NF ID, Target of Event Reporting ("any UE" or as defined in Table 4.15.4.5.1-1 when a specific UE is targeted ), (set of) Event ID(s) defined in clause 5.2.26.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1 (TS 29.564 [93] defines the possible parameters).

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, either Application ID(s) or Traffic Filtering Information, Type of Measurement, Granularity of Measurement, Reporting suggestion information.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1). Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by UPF with this subscription.

NEXT CHANGE (3)

NEXT CHANGE (4)

END OF CHANGES