3GPP TSG-WG SA2 Meeting #164	S2-2408743

Maastricht, Netherlands, 19-23 August 2024	(revision of S2-2407764)

Source:	Apple

Title:	Update to Solution for different Ambient IoT Device Types

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper provides an update to solution #33 and resolves the Editor’s Notes in that solution.

# 1. Introduction

This contribution provides an update to solution #33 in TR 23.700-13 for different types of Ambient IoT devices.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * Start of changes * * * *

## 6.33	Solution #33: Solution for different Ambient IoT Device Types

### 6.33.1	Description

#### 6.33.1.1	Introduction

RAN has considered three types of AIoT devices (according to TR 38.848 [7]). The capabilities of these devices, vary quite significantly in terms of their baseband capabilities, storage, form factor from read-only devices to devices with writable memory, varying cost, complexity etc. As such it may not be possible to have the same solution for all AIoT device types and it may be prudent to classify AIoT devices (as an example) into two device types:

-	the type A AIoT device that may not need to address privacy requirements and may also use a fixed permanent device identifier;

-	the type B AIoT device that would use a temporary identifier, to address privacy concerns.

This solution describes for these two different types of Ambient IoT devices.

#### 6.33.1.2	Functional Description

The solution for type A AIoT device is based on the following principles.

-	The type A AIoT devices are ultra-low complexity devices with ultra-low power consumption for very-low end AIoT applications with limited or no energy storage capability, wherein UL transmission is backscattered, and the solution applies for both Topology 1 and 2.

-	The AIoT device is pre-provisioned with a fixed permanent device identifier (AIoT Device ID) that it uses for all communication.

-	The AIoT device does not perform the 5GC Registration procedure with the network and the device does not authenticate with the network, though the network may perform certain checks to validate the AIoT device subscription and service operation based on the AIoT Device ID. The AIoT device has no NAS or RRC states as defined in 5GS. The AIoT device can be paged.

-	There is no PDU session created for session management and data transfer is done using NAS messages. The device communicates with the network directly by sending data (Ethernet, Unstructured frames) using its AIoT Device ID.

The solution for type B AIoT devices is based on the following principles.

-	The type B AIoT device are relatively low complexity devices with low power consumption and limited energy storage capability that can generate their own UL transmission, and the solution applies for both Topology 1 and 2.

-	The AIoT device registers with the network based on a network trigger and the network authenticates the device and assigns the device temporary identifiers. The AIoT device maintains some notion of NAS states (registered, de-registered, etc.) and can be paged.

-	The AIoT devices performs data transfer using NAS messages.

-	The communication between the device and network is secure (may be encrypted and integrity protected).

The AIoT Device ID will be a combination of following:

-	a local device ID (similar to a RFID Tag ID or EPC) which can either be pre-provisioned or even be provisioned by the Reader, a device instance identifier and the operator identifier.

Editor's note:	Whether there is a need to differentiate Ambient IoT Type A and Type B devices is FFS.

#### 6.33.1.3	Reference Architecture

Figure 6.33.1.3-1: Reference Architecture to support Ambient IoT

The figure 6.33.1-3.1 shows the example reference architecture to support AIoT devices (both type A and type B) for Topology 2 wherein the UE (intermediate node) acts as the Reader. The same architecture also applies for Topology 1 (without the UE intermediate Node) and in that case the RAN acts as the Reader.

The 5GC has a new network function AIoTCF (Ambient IoT Controller Function) to support different AIoT service operations (command, inventory, etc.). The AIoTCF registers the Readers and authorizes the AFs for different AIoT service operations. The AIoTCF discovers and selects Readers for different service operations initiated by the AF. The AIoTCF can be integrated with AMF as well.

#### 6.33.1.4	Protocol Stack

Figure 6.33.1.4-1: Protocol Stack to support Ambient IoT

The figure 6.33.1.4.1 shows the example protocol stack to support AIoT devices (both type A and type B) for Topology 2. The Reader supports the A-Uu air interface with AIoT device. The AIoT C is the optional control protocol between AIoT device and the Reader and will be defined by RAN. The AIoT NGAP protocol is the interface between the Reader and AIoTCF and can be a lightweight version of NGAP protocol tailored for AIoT or a new different protocol as well, which will be in scope of RAN3. The AIoTCF exposes the Naiotcf interface and in case the AIOTCF is integrated with AMF, the Namf interface can be enhanced. The AIoT APIs will be defined by CT3 and will expose the AIoT functionality to external entities. The AIoT NAS protocol enhancements will be defined by CT1. All stage-2 architectural aspects will be defined by SA WG2.

NOTE: Whether AIoT C protocol is required, and its contents will be defined by RAN.

### 6.33.2	Procedures

#### 6.33.2.1	AIoT Service Operation Procedure for Type A AIoT Device

This clause describes the procedure for service operation (inventory, command, etc.) for a type A AIoT device as indicated in Figure 6.33.2.1-1. The procedure in general applies to both Topology 1 and Topology 2 configurations.

Figure 6.33.2.1-1: AIoT Service Operation Procedure for Type A AIoT Device

1.	The type A AIoT device is pre-configured with Device ID, Group ID, and any other appropriate credentials. The type A AIoT device does not have UICC and may use the permanent device identifier as Device ID.

2.	The Reader (UE intermediate node) registers with the AIoT Controller Function in the 5GC. The Reader reports (Reader ID, Capability to support AIoT, coverage area.) to AIoTCF. The AIoTCF aggregates the capability of all Readers and registers with NRF to provide AIoT service for AFs.

3.	The AF sends a AIoT service operation (inventory, command, etc.) request to NEF which discovers and selects the appropriate AIoTCF registered with NRF, based on the included information, and sends the request to the selected AIoTCF.  The AF may include the following information to identify the Reader and the AIoT Device.

- AIoT Device Identifier: This can be the individual AIoT Device ID or a AIoT Group ID corresponding to a group of devices and the number of AIoT devices for service operation. The AF may optionally also include the AppId (application identifier) to identify the specific application on the AIoT device. The AppId may be mapped to a AIoT Group ID in the AIoTCF.

- Reader Information: Area of Operation, optionally Reader Identifier, optionally IP address of the Reader

- Service Information: This will include the type of service operation, priority of operation, periodicity, service specific control information, reporting information

The AIoTCF authorizes the service request after verifying whether the AF is entitled to make the specified service operation request based on service agreements, operator policy, etc.

The AIoTCF selects the Ambient IoT capable Reader (or a set of Readers) based on the device information and information about the allowed service area. The selection of the intermediate node can be performed based on the geographical location of the target AIoT devices or the identity of the intermediate node from AF.

4.	For each of the selected Reader, the AIoTCF sends the AIoT NGAP message to the RAN which results in the RAN triggering a paging like procedure and the Reader paging the type A AIoT device. The AIoT device performs random access channel (RACH) like procedure and sends a Paging response (including its Device ID) to the Reader which in turn relays this to the RAN which then sends this response in an AIoT NGAP message to the AIoTCF.

5.	The AIoTCF retrieves the Device ID and performs validation on the type A AIoT device based on subscription information retrieved from UDM (device disabled or not, authorized for specified service operation, operating in permissible service area, etc.). If the deice is validated, the AIoTCF records the AIoT device location and performs the association with corresponding RAN and Reader. If the AIoT device cannot be validated the AIoTCF may invalidate the device and disable it, if possible.

6.	The AIoTCF constructs the service operation request and sends it in an DL AIoT-NAS message to the AIoT Device. The AIoT Device performs the service operation and generates the results for specified operation and includes them in a UL AIoT-NAS message which is sent to the AIoTCF.

7.	The AIoTCF can aggregate the results of service operation across Device IDs and report the results of service operation to NEF and the AF.

#### 6.33.2.2	AIoT Service Operation Procedure for Type B AIoT Device

This clause describes the procedure for service operation (inventory, command, etc.) for a type B AIoT device as indicated in Figure 6.33.2.1-2.

1-2.	These are same as in clause 6.33.2.1. The type B AIoT device may have a UICC.

3.	The trigger for AIoT device registration can be initiated from an external AF, wherein the external AF initiates devices in a specific area or in the range of a specific Reader to register with the network. The AIoT device registration can also be initiated by the network (AIoTCF and the RAN Reader). The Reader triggers the paging like procedure for the type B AIoT Device. The type B AIoT device performs random access channel (RACH) like procedure to establish AS connection with Reader and thereafter initiates the AIoT Device 'Registration triggered by Network' procedure. The AIoTCF authenticates the AIoT Device and assigns the type B AIoT Device temporary identifier (TempDevID) as part of the authentication procedure. The AIoT Device uses the temporary identifier in subsequent communication. The AIoT device enters the registered state.

NOTE:	It is expected that the temporary identifier (TempDevId) assigned to the type B AIoT device as part of Registration procedure can survive multiple "energy harvesting cycles". Otherwise, if the AIoT device loses its energy and cannot maintain results from the Registration procedure, it may not be feasible to conduct such a procedure, in its entirety.

4.	Void.

5-9. These are same as steps 3-7 in clause 6.33.2.1.

Figure 6.33.2.2-1: AIoT Service Operation Procedure for Type B AIoT Device

### 6.33.3	Impacts on services, entities and interfaces

New network entities and interfaces are proposed as part of this solution.

AIoTCF: New Network Function

- receives AF requests for AIoT service, performs mapping of AF request information to 5GS service information and registration of Readers.

UDM:

- stores the AIoT device subscription information.

NEF:

- authorizes AF request, retrieves AIoTCF information and handles service operation between AF and AIoTCF.

AIoT Device:

- receives service request from the AIoTCF and responds with service response to the AIoTCF.

# * * * * End of changes * * * *