SA WG2 Meeting #164	S2-2408747

Maastricht, NL, August 19-23 2024	(revision of S2-2408198)

Source:	ZTE, Futurewei

Title:	KI#1, 3, New Sol, Service Operation Aggregation for Topology 1 and Topology 2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: A new solution for key issue #1 and #3 is proposed to aggregate the service operation for Topology 1 with RAN Reader and Topology 2 with UE Reader.

1. Discussion

The paper is proposed to introduce a new solution to aggregate the service operation for Topology 1 with RAN Reader and Topology 2 with UE Reader.

In architecture assumptions of TR 23.700-13, it is clarified that two Topologies (1 and 2) are in the scope of study:

-	The following two connectivity topologies as defined in TR 38.848 [7] are to be studied:

-	Topology 1: BS <--> Ambient IoT Device;

-	Topology 2: BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control.

In the scenario which both Topology 1 and Topology 2 are deployed; and Topology 1 with RAN Reader is main deployment, and Topology 2 with UE Reader is supplementary deployment in case the devices cannot be accessed by RAN Reader; when AF sends service operation to the network, how to aggregate the operation for devices in both Topology 1 and Topology 2 needs to be studied.

In this solution, when receiving the service operation from AF, the network will firstly interact with RAN Readers to execute service operation to target devices; and then determines whether the operation for all target devices is completed based on the device operation results: if not completed the network will interact with UE Readers to execute service operation to the rest devices. When the service operation to all target devices is finished, the network aggregates the service operation results from RAN Readers and UE Readers, and sends the results to AF.

This solution addresses both Key Issue #1 (Architecture support of Ambient IoT Devices) and Key Issue #3 (Support of Ambient IoT Services) for the convergence of Topology 1 and Topology 2.

2. Proposal

It is proposed to capture the following changes into TR 23.700-13.

* * * * Start of Changes * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #1 |  | X |  |
| #2 |  | X |  |
| #3 | X | X | X |
| #4 | X | X | X |
| #5 | X | X | X |
| #6 | X | X | X |
| #7 | X | X |  |
| #8 | X | X |  |
| #9 | X | X | X |
| #10 | X | X |  |
| #11 |  |  | X |
| #12 | X | X | X |
| #13 |  | X |  |
| #14 |  | X |  |
| #15 |  | X | X |
| #16 |  | X |  |
| #17 | X | X | X |
| #18 |  |  | X |
| #19 | X |  | X |
| #20 | X | X | X |
| #21 | X | X | X |
| #22 | X | X | X |
| #23 |  |  | X |
| #24 |  | X |  |
| #25 |  | X |  |
| #26 |  |  | X |
| #27 |  |  | X |
| #28 |  |  | X |
| #29 | X | X |  |
| #30 | X | X | X |
| #31 |  | X | X |
| #32 | X |  | X |
| #33 | X | X | X |
| #34 |  | X |  |
| X | X |  | X |

* * * * Second Change * * * *

6.X	Service Operation Aggregation for Topology 1 and Topology 2

6.X.1	Description

6.X.1.1	General

This solution applies for the scenario in which both Topology 1 and Topology 2 are deployed; and Topology 1 with RAN Reader is main deployment, and Topology 2 with UE Reader is supplementary deployment in case the devices cannot be accessed by RAN Reader. The mixed deployment of both RAN Reader and UE Reader can expand the management coverage of AIoT devices. When AF sends service operation to the network, the request message includes the following information: service type, i.e. Inventory, Command; where the Command specifically includes Read, Write, Disable, etc. operations; the target device of the service request can be the Device ID of a single device, a group of Device IDs, multiple Device IDs with MASK filtering conditions, or devices in a specific area. The network will first interact with RAN Readers to execute service operation to target devices; and then determines whether the operation for all target devices is completed based on the device operation results: if not completed the network will interact with UE Readers to execute service operation to the rest devices. Alternatively, the network can select different type of Readers based on the policy (e.g., different priority): either RAN Reader or UE Reader could have high priority for certain service operation; or these two types of Reader have same priority which means they will be operated as same time. When the service operation to all target devices is finished, the network aggregates the service operation results from RAN Readers and UE Readers, and sends the results to AF.

In this solution, enhanced AMF or a new network function AIoTMF (EnAMF/AIoTMF) is used to interact with RAN Readers and UE Readers to execute service operation to devices targeted by service request, and aggregate the service operation results from RAN Readers and UE Readers.

6.X.1.2	Reference Architecture

Figure 6.X.1.2-1 depicts the reference architecture of service operation aggregation for Topology 1 and Topology 2.

Figure 6.X.1.2-1 Reference Architecture

In the architecture, EnAMF/AIoTMF is used to interact with RAN Readers and UE Readers to execute service operation to devices targeted by service request, and aggregate the service operation results from RAN Readers and UE Readers. The architecture with a single EnAMF/AIoTMF addresses the scenario of limited area coverage of devices; for large area coverage, multiple EnAMF/AIoTMF serving large numbers of multiple Readers ( UE or RAN Readers) may work together to handle the AF service request for the devices in large area. Other network elements include UDM, AUSF, NRF, NEF, and AF; these network elements fulfil the existing functions in 5G core network, and are enhanced to support AIoT services in this solution.

6.X.2	Procedures

6.X.2.1	Service Operation Aggregation For Topology 1 and Topology 2

Figure 6.X.2.1-1 depicts a flow diagram for service operation aggregation for Topology 1 and Topology 2.

Figure 6.X.2.1-1 Service Operation Aggregation For Topology 1 and Topology 2

The steps in the service operation aggregation for Topology 1and Topology 2 procedure are as follows:

0.a. RAN Reader reports its AIoT capability and serving area to EnAMF/AIoTMF; EnAMF/AIoTMF registers to NRF with the information of its serving RAN Readers.

0.b. UE Reader registers to the core network indicating its AIoT capability.

1. AF sends a service request for AIoT devices to NEF. The request message includes the following elements: service type, i.e. Inventory, Command; where the Command specifically includes Read, Write, Disable, etc. operations; the device information of the service request can be the Device ID of a single device, a group of Device IDs, multiple Device IDs with MASK filtering conditions, or devices in a specific area.

2. NEF verifies the service request from AF and queries NRF for the relevant EnAMF/AIoTMF information that serves the devices according to the device information in the request.

3. NEF sends the service request for AIoT devices to EnAMF/AIoTMF.

4. EnAMF/AIoTMF interacts with UDM to verify whether the service request is allowed; if allowed, EnAMF/AIoTMF discovers and selects RAN Readers serving devices according to the device information targeted by the service request.

Alternatively, EnAMF/AIoTMF can determine the priority of two types of Readers based on the policy: either RAN Reader or UE Reader could have high priority for service operation; or these two types of Readers have same priority which means they will be operated as same time. The following steps assume the RAN Reader has high priority for service operation.

5. EnAMF/AIoTMF sends the service request for AIoT devices to the selected RAN Reader.

6. RAN Reader interacts with multiple AIoT devices to complete the service operation.

7. RAN Reader returns the service operation results of the AIoT devices to EnAMF/AIoTMF.

The above steps 5~7 will repeat for multiple selected RAN Readers.

8. The service request target is for one or multiple specific devices, EnAMF/AIoTMF determines whether the operation for all devices in the service request is completed based on the device operation results sent from RAN Readers in steps 5~7: if completed, executes step 9.a.

If EnAMF/AIoTMF determines that the operation for all devices in the service request is not completed, then executes steps 9.b~12.b.

9.a. EnAMF/AIoTMF returns the service operation results of AIoT devices to AF through NEF.

9.b. EnAMF/AIoTMF sends the service request of AIoT devices to UE Reader through the base station serving UE Reader. The base station serving UE Reader is not the same base station as RAN Reader.

10.b. UE Reader interacts with AIoT devices to complete the service operation.

11.b. UE Reader returns the service operation results of AIoT devices to EnAMF/AIoTMF.

The above steps 9b~11b will repeat for multiple selected UE Readers.

12.b. EnAMF/AIoTMF aggregates the service operation results from RAN Readers and UE Readers, and returns the results to AF through NEF.

6.X.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

* * * * End of Changes * * * *