3GPP TSG-WG SA2 Meeting #164	S2-2408770

Maastricht, Netherlands, August 19 - 23, 2024	(revision of S2-240xxxx)

Source:	China Mobile

Title:	TR 23.700-66 KI#1 Conclusion update based on the way forward proposal

Document for:	Approval

Agenda Item:	19.4.1

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This pCR proposes to update the conclusion of KI#1.

1	Discussion

This paper provides conclusion update of KI#1: Network energy related information exposure.

2	Proposal

It is proposed to include the below changes into TR 23.700-66.

First Changes

8.1	Conclusion for KI#1: Network energy related information exposure

The following bullets are recommended for normative work for KI#1:

1)	A new network functionality is defined to collect and calculate the energy related information and exposes to the authorized consumer subject to operator's policy:

-	If the authorized consumer is AF, the granularities include: per per UE.

-	If the authorized consumer is 5GC NFs, the granularities include: per UE.

NOTE: The finer granularity e.g., per UE per AF, per PDU session or per QoS flow will be determined in normative phase.

NOTE: The new network functionality can be deployed standalone or co-located with other NFs which within SA2 responsibility (e.g., NEF, or NWDAF).

-	The energy related information for the granularities above include:

-	Energy consumption information;

-	Renewable energy information.

2)	The consumer NFs including 5GC NFs and /or AF, may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

3)	The new functionality supportingthe calculation of the Energy Consumption information includes the following aspects:

a)	OAM provides the energy consumption information at the gNB(s) and UPF(s) serving the UE to the new functionality.

b)	OAM: provides the overall data volume for the gNB to the new functionality;

c) The above OAM reporting includes start/end time. And the per Node-level information of a) and b) received from OAM, could be used by the new functionality for all the UEs serving by the NF Node.

NOTE: There is coordination between SA2 and SA5 to help OAM provides Node level EC information in normative phase.

d)	UPF: provides the overall data volume of the UPF;

e)	UPF: provides the data volume for the specific UE.

f) During the handover, the serving gNB ID will be sent to the new functionality through AMF/SMF.

4)	The new functionality determines the E2E energy consumption based on energy consumption per the granularities above at serving NF (i.e. NG-RAN and UPF).

5)	In the current release of the specification, only the energy-related information of user plane communication (not control plane signalling) is supported.

NOTE 1:	The exposure and collection of renewable energy information depend on the coordination with SA WG5.

NOTE 2:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting.

Editor's note: Whether in this release the NG-RAN will provide per UE level energy consumption information and state depending on RAN WGs.

NOTE 3:	This work will coordinate with SA WG5 for OAM and charging part.

End of Changes