3GPP TSG-SA WG2 Meeting #164	S2-24088817

19 August - 23 August 2024, Maastricht, Netherlands	(revision of S2-2408107)

Source:	CATT

Title:	Update the Conclusion for KI#9

Document for:	Discussion/Approval

Agenda Item:	19.3.1

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: this contribution proposes to update the conclusion for KI#9.

1.	Introduction

This contribution proposes to update the conclusion for KI#9.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First Change * * * *

## 8.9	Conclusions for Key Issue #9

The following aspects are concluded for the normative work:

-	For GBR QoS Flow, exposure of available data rate:

-	Based on the AF requests, the PCF instructs SMF to enable the available data rate report and exposure for a GBR QoS Flow.

-	Following the instruction of SMF:

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes based on mechanisms defined in clause 5.8.2.17 of TS 23.501 [2], and the available data rate is reported to the AF directly or via the (local) NEF.

-	 Available data rate refers to the RAN estimated data rate that RAN is able to provide in a short estimation time window based on implementation.

NOTE 1:	It is recommended that the available data rate exposure is not reported very frequently.

NOTE 2:  RAN may consider the GFBR values of the discrete set of Alternative QoS profiles (AQP), or in case the AQPs are not configured, the GFBR rate of the GBR QoS Flow, as the available data rate, which is up to RAN implementation.

NOTE 3：How the RAN measures the available UL and/or DL data rate is to be refined based on the potential feedback from RAN WG3.

# * * * * End of change* * * *