3GPP TSG-WG SA2 Meeting #164	S2-2408833

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-2408473)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5587 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MDBV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current MDBV (for delay-critical GBR resource type) is derived with the assumption that the PDB is smaller than the periodicity of burst arrivals. How to derive the MDBV is however not clear for the case that the periodicity is in the same range or smaller than the PDB. As an example, consider a service with a PDB of 4 ms with periodicity of 3 ms. In this case with a PDB larger than the periodicity of burst arrivals, there will be typically more than a single burst to be sent by the RAN within the 5G-AN PDB. It was discussed in XRM that the AF can take 5G-AN PDB into account to generate Burst Size but the CN-PDB is not known to the AF and it would be too complicated to transfer the CN-PDB to the AF (due to its dependency on the 5QI and the deployment specifics). Furthermore, for some AFs it may also not be possible to change the logic of how to derive the burst size (e.g. in the TSN case, how to generate burst size by TSN AF is standardized outside of 3GPP). It is therefore proposed to adjust the MDBV value in RAN under consideration of the periodicity and the 5G-AN PDB, whenever the periodicity is signalled to RAN and the periodicity is smaller than the 5G-AN PDB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MDBV adjustment by RAN is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MDBV cannot be used correctly if periodicity is smaller than the 5G-AN PDB. QoS requirements of the AF cannot be fulfilled by RAN when more than one burst arrives during the time interval of the requested delay (even though the PCF has accepted these QoS requirements and the corresponding SLA is applied). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.3.4, 5.7.3.5, 5.7.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.7.3.4	Packet Delay Budget

The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF. The PDB applies to the DL packet received by the UPF over the N6 interface, and to the UL packet sent by the UE. For a certain 5QI the value of the PDB is the same in UL and DL. In the case of 3GPP access, the PDB is used to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). For GBR QoS Flows using the Delay-critical resource type, a packet delayed more than PDB is counted as lost if the data burst is not exceeding the NG-RAN Adjusted MDBV (see clause 5.7.3.7) within the period of PDB and the QoS Flow is not exceeding the GFBR. For GBR QoS Flows with GBR resource type not exceeding GFBR, 98 percent of the packets shall not experience a delay exceeding the 5QI's PDB.

The 5G Access Network Packet Delay Budget (5G-AN PDB) is determined by subtracting a static value for the Core Network Packet Delay Budget (CN PDB), which represents the delay between any N6 termination point at the UPF (for any UPF that may possibly be selected for the PDU Session) and the 5G-AN from a given PDB.

NOTE 1:	For a standardized 5QI, the static value for the CN PDB is specified in the QoS characteristics Table 5.7.4-1.

NOTE 2:	For a non-standardized 5QI, the static value for the CN PDB is homogeneously configured in the network.

For GBR QoS Flows using the Delay-critical resource type, in order to obtain a more accurate delay budget PDB available for the NG-RAN, a dynamic value for the CN PDB, which represents the delay between the UPF terminating N6 for the QoS Flow and the 5G-AN, can be used. If used for a QoS Flow, the NG-RAN shall apply the dynamic value for the CN PDB instead of the static value for the CN PDB (which is only related to the 5QI). Different dynamic value for CN PDB may be configured per uplink and downlink direction.

NOTE 3:	The configuration of transport network on CN tunnel can be different per UL and DL, which can be different value for CN PDB per UL and DL.

NOTE 4:	It is expected that the UPF deployment ensures that the dynamic value for the CN PDB is not larger than the static value for the CN PDB. This avoids that the functionality that is based on the 5G-AN PDB (e.g. MDBV, NG-RAN scheduler) has to handle an unexpected value.

The dynamic value for the CN PDB of a Delay-critical GBR 5QI may be configured in the network in two ways:

-	Configured in each NG-RAN node, based on a variety of inputs such as different IP address(es) or TEID range of UPF terminating the N3 tunnel and based on different combinations of PSA UPF to NG-RAN under consideration of any potential I-UPF, etc;

-	Configured in the SMF, based on different combinations of PSA UPF to NG-RAN under consideration of any potential I-UPF. The dynamic value for the CN PDB for a particular QoS Flow shall be signalled to NG-RAN (during PDU Session Establishment, PDU Session Modification, Xn/N2 handover and the Service Request procedures) when the QoS Flow is established or the dynamic value for the CN PDB of a QoS Flow changes, e.g. when an I-UPF is inserted by the SMF.

If the NG-RAN node is configured locally with a dynamic value for the CN PDB for a Delay-critical GBR 5QI, and receives a different value via N2 signalling for a QoS Flow with the same 5QI, local configuration in RAN node determines which value takes precedence.

Services using a GBR QoS Flow and sending at a rate smaller than or equal to the GFBR can in general assume that congestion related packet drops will not occur.

NOTE 5:	Exceptions (e.g. transient link outages) can always occur in a radio access system which may then lead to congestion related packet drops. Packets surviving congestion related packet dropping may still be subject to non-congestion related packet losses (see PER below).

Services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB.

The PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, for a Delay-critical GBR resource type, packets delayed more than the PDB are added to the PER and can be discarded or delivered depending on local decision.

# * * * * Second change * * * *

#### 5.7.3.5	Packet Error Rate

The Packet Error Rate (PER) defines an upper bound for the rate of PDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For every 5QI the value of the PER is the same in UL and DL. For GBR QoS Flows with Delay-critical GBR resource type, a packet which is delayed more than PDB is counted as lost, and included in the PER unless the data burst is exceeding the NG-RAN Adjusted MDBV (see clause 5.7.3.7) within the period of PDB or the QoS Flow is exceeding the GFBR.

# * * * * Third change * * * *

#### 5.7.3.7	Maximum Data Burst Volume

Each GBR QoS Flow with Delay-critical resource type shall be associated with a Maximum Data Burst Volume (MDBV) which denotes the size of a data burst (i.e. the amount of data), i.e. a set of multiple PDUs generated and sent by the application in a short period of time.

Every standardized 5QI (of Delay-critical GBR resource type) is associated with a default value for the MDBV (specified in QoS characteristics Table 5.7.4.1). The MDBV may also be signalled together with a standardized 5QI to the (R)AN, and if it is received, it shall be used instead of the default value.

The MDBV may also be signalled together with a pre-configured 5QI to the (R)AN, and if it is received, it shall be used instead of the pre-configured value.

The NG-RAN applies the MDBV to derive the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB (see clause 5.7.3.4) and that amount of data is defined as NG-RAN Adjusted MDBV.

NOTE:	The NG-RAN Adjusted MDBV is needed to accommodate traffic characteristics where there is more than one data burst of size MDBV within the 5G-AN PDB.

If Periodicity (i.e. in TSCAI as described in clause 5.27) is not signalled to the NG-RAN or if Periodicity is signaled to the NG-RAN and the periodicity is greater than or equal to the 5G-AN PDB, the NG-RAN Adjusted MDBV is set to the MDBV (either default value or signalled value).

If Perodicity is signalled to NG-RAN and the Periodicity is smaller than 5G-AN PDB, the NG-RAN shall calculate the NG-RAN Adjusted MDBV from the MDBV (either default value or signalled value) based on operator configuration.

NOTE:	For example, the NG-RAN Adjusted MDBV could be calculated as MDBV * ceil ( 5G-AN PDB / Periodicity ) where “ceil” is the ceiling function, which returns the smallest integer greater than or equal to its argument.

The NG-RAN shall determine the NG-RAN Adjusted MDBV whenever the QoS Profile or the Alternative QoS Profile(s) contains an MDBV.

# * * * * End of changes * * * *