3GPP TSG-SA WG2 Meeting #164	S2-2408850

# Maastricht, NL, August 19-23, 2024													(revision of S2-2408155)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1428 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-Satellite-UE communication in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, CATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3_ARCH work item and the conclusion of FS_5GSAT_Ph3_ARCH captured in clause 8.3 of TR 23.700-29, this CR adds description on Support of UE-Satellite-UE communication in IMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Support of UE-Satellite-UE communication in IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

Annex AX (normative):
Support of optimized media routing using the satellite constellation IP-CAN

AX.1	General

This annex describes IMS architecture enhancements to support optimized media routing using the satellite constellation IP-CAN.

The satellite constellation IP-CAN is realized by 5GS with functionalities defined in clause x.x of TS 23.501 [93]. The optimized media routing described in this annex refers to a routing of media between UEs under the coverage of one or more serving satellites, using the satellite constellation IP-CAN without the media transiting through the ground segment. The ground fallback routing refers to the case with the media transiting through the ground segment.

P-CSCFs in the MO side and in the MT side exchange the identifier of a satellite serving the MO UE and the MT UE, respectively, and enable the satellite constellation IP-CAN of the MO side and of the MT side in coordination to provide a media routing path with or without the media transiting through the ground segment. P-CSCF also establishes IMS AGW either on ground or on satellite accordingly.

In this Release of the specification, this feature is supported only for IMS voice/video service and for UEs belonging to the same HPLMN and in the non-roaming scenario.

AX.2	Architecture and functional entities

AX.2.1	Architecture

AX.2.2	Functional entities

AX.2.2.1		P-CSCF

P-CSCF used for this feature is enhanced to support the following functionalities:

-	P-CSCF determines the possibility of optimized media routing and interacts with the satellite constellation IP-CAN, as described in Annex AX.3.

-	P-CSCF controls IMS AGW relocation, as described in Annex AX.4.

-	P-CSCF discovers and selects IMS AGW on satellite, as described in Annex AX.6.

AX.3	Determination of the possibility of optimized media routing

AX.3.1	At call setup

P-CSCF may receive the identifier of a satellite serving the local UE from PCF and find the identifier of a satellite serving the remote UE in a SIP message received from the remote network. The

P-CSCF shall determine that optimized media routing is possible based on those satellite identifiers.

Editor’s note: 		How exactly P-CSCF judges the possibility of optimized media routing based on satellite identifiers is FFS.

If P-CSCF determines optimized media routing is possible, P-CSCF shall inform it to PCF, so that the PCF proceeds to establish the satellite constellation IP-CAN for optimized media routing.

AX.3.2	At change of satellite

When

P-CSCF receives from PCF the identifier of a satellite that is about to serve the local UE, the P-CSCF shall determine that optimized media routing continues to be possible between the satellite and the satellite serving the remote UE based on identifiers of those satellites.

If P-CSCF determines that optimized media routing continues at change of satellite, P-CSCF shall send a positive message to PCF, so that the PCF proceeds to establish the satellite constellation IP-CAN on the target satellite for optimized media routing.

When P-CSCF receives from the remote network a SIP re-INVITE without satellite related information, the P-CSCF shall determine that optimized media routing becomes not possible.

If P-CSCF determine that optimized media routing becomes not possible at change of satellite, P-CSCF shall send a negative message to PCF, so that the PCF proceeds to establish the satellite constellation IP-CAN for ground fallback routing.

AX.4	IMS AGW relocation

## AX.4.1	At call setup

P-CSCF shall establish an IMS AGW on ground at call setup according to TS 23.334 [74], as long as the P-CSCF does not determine optimized media routing is possible. After the P-CSCF determines optimized media routing is possible, the P-CSCF shall release the IMS AGW on ground and establish an IMS AGW on satellite according to TS 23.334 [74].

AX.4.2	At change of satellite

## When P-CSCF is informed that the satellite serving UE changes,

the P-CSCF then shall initiate to establish IMS-AGW on the target satellite or on ground according to TS 23.334 [74] depending on whether optimized media routing continues to be possible.

After completion of establishment of the IMS AGW on the target satellite or on ground, the P-CSCF shall inform the local UE and the remote network to use the newly established IMS AGW(s).

## ---End of the Change---