3GPP TSG-WG SA2 Meeting #164	S2-2408897

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5468 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Control of UE access to MWAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The work item VMR_Ph2 was approved in SP#104 meeting therefore it is the time to introduce the features to the related specifications. Regarding the topic of “controlling the access of the UE”, as per the discussions during the study phase, there were two aspects that to be addressed, namly controlling the access of UE to the PNI-NPN, and controlling the access of UE to the SNPN. Note that it is not the purpose to extend the mechanisms specific for NPN (e.g., CAG/SNPN ID) to the general case. In addition, the conclusion of the TR can be found as follows: - The existing CAG mechanism (as defined in Rel-18 for MBSR) can be used as an optional method for managing UE's access to MWAB-gNB, for PNI-NPN deployment cases. - Existing SNPN control mechanism can be used to manage the UE's access to MWAB-gNB, in case the MWAB-gNB is serving a SNPN. - - When there is mixed deployment of fixed gNB(s) and MWAB-gNB(s) for SNPN, existing SNPN control mechanism can be reused to differentiate the cells, and no standards impact is expected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clauses to support for Mobile gNB with Wireless Access Backhauling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The controlling the access for the UE to MWAB is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (All Text New) * * * *

5.X.6	Control of UE access to MWAB

5.X.6.1	General

Control of UE access to MWAB is supported. If the MWAB-gNB is serving a PNI-NPN, the CAG mechanism described in clause 5.x.6.2 is used. If the MWAB-gNB is serving an SNPN, the SNPN control mechanism described in clause 5.x.6.3 is used.

The MWAB-gNB and the 5GC can reuse other existing mechanisms e.g., forbidden Tracking Area, UAC, to manage the UE's access to an MWAB-gNB cell.

NOTE:	MWAB-UE's access to the network is controlled with mechanisms defined in clause 5.2 for PLMN and clause 5.30.2/5.30.3 for SNPN/PNI-NPN, based on subscription data of the MWAB-UE. It is not addressed in this clause.

5.X.6.2	Control of UE access to an MWAB serving PNI-NPN

According to operator's policy, access control may be applied to UE access to MWAB-gNB cell supporting a PNI-NPN.

If the MWAB-gNB is serving a PNI-NPN, the CAG mechanism defined in clause 5.30.3 can be used for managing UE's access to the MWAB-gNB cell, with the following additional considerations:

-	The MWAB-gNB is configured either during the communication with the OAM of MWAB-gNB or (pre-)configuration mechanism, with a one or more CAG identifiers which are unique within the scope of this PLMN.

-	If the MWAB-gNB is (pre-)configured with the PLMN list supporting PNI-NPN, the corresponding CAG Identifiers per PLMN is also configured in the MWAB.

-	The MWAB-gNB perfroms UE access control based on the CAG identifier associated with the MWAB-gNB cell, as defined in clause 5.30.3.4. The MWAG-gNB takes the role of the NG-RAN.

NOTE 1:	In the case of UE not supporting CAG functionality, the MWAB-gNB and 5GC of MWAB Announced PLMN can use other existing mechanisms e.g. forbidden Tracking Area, to manage UE's access to a MWAB-gNB cell.

-	Time validity information may be provided to the UE together with the CAG Identifier(s) of the MWAB-gNB(s) that the UE can access. If the evaluation of the time validity information of an entry in the Allowed CAG list changes, the updated Allowed CAG list will be provided to UE and MWAB-gNB by the AMF for enforcement, to make sure that UE not accessing the corresponding MWAB-gNB cell outside of the time duration.

NOTE 2:	Dedicated CAG ID(s) can be used for the control of UE access to a PNI-NPN via MWAB cells.

5.X.6.3	Control of UE access to an MWAB broadcasted SNPN

If the MWAB-gNB is serving an SNPN, as illustrated in Annex X.3, the existing SNPN control mechanism defined in clause 5.30.2.5 can be used to manage the UE's access to MWAB-gNB.

NOTE:	Dedicated SNPN ID(s) or GIN(s) can be used if differentiation is desired in control of UE access to the SNPN via MWAB-gNB or other fixed gNBs.

# * * * * End of changes * * * *