3GPP TSG-WG SA2 Meeting #164	S2-2408898

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5469 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility of UEs served by MWAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As the work item VMR_Ph2 was approved in SP#104 meeting therefore it is the time to introduce the features to the related specifications. On the mobility aspects, the followings aspects were discussed and reflected in the conclusion clause: 1. No impact to the UE: No UE impact should be expected to support mobility involving a MWAB. 2. Connect to different PLMNs: MWAB-gNB and MWAB-UE may connect to different PLMNs. 3. UPF and/or AMF might be changed: Upon mobility, the UPF and/or AMF used for BH connections of the MWAB-UE may change. 4. Information broadcasted by the gNB: The TAC, gNB ID and/or cell ID advertised by the MWAB-gNB may change. 5. on the configuration: The configuration can be reconfigured by the OAM server 6. UE move to or leave from MWAB or between MWAB: Existing mechanism can be reused. 7. UE move together with MWAB: need to wait for RAN’s feedback. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clauses to support for Mobility support of UEs served by MWAB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobility regarding MWAB is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (All Text New) * * * *

5.X.5	Mobility support of MWAB

5.X.5.1	UE mobility when at least a MWAB-gNB cell is involved

The existing UE mobility procedures without MWAB-specific UE enhancements can be reused to handle a UE mobility to and from a MWAB-gNB while the UE is in CM-IDLE or CM-CONNECTED modes, irrespective of whether the MWAB is static or the MWAB is moving, i.e.:

-	For UEs in RRC_IDLE and RRC_INACTIVE state procedure for cell (re-)selection as specified in TS 38.304 [50] and TS 23.122 [17] for RRC_IDLE and RRC_INACTIVE is used as normal irrespective of cells are belonging to a MWAB-gNB or a gNB.

-	For UEs in RRC_CONNECTED state, the MWAB-gNB or gNB triggers the handover procedures to the neighbouring cells (irrespective of whether they are belonging to a MWAB-gNB or a gNB) for UEs in RRC_CONNECTED state.

UEs and MWAB-gNB use existing procedures as specified in clause 8.23 of TS 38.401 [42], TS 23.502 [3], or TS 38.304 [50].

5.X.5.2	UE mobility when the UE is moving together with a MWAB cell

For a UE served by an MWAB-gNB cell, the MWAB-gNB may trigger the handover procedures for UEs in CM-CONNECTED mode as specified in TS 38.401 [42].

The UE may observe change of TAC and/or cell IDs, even if it is still connected to the same MWAB-gNB. This can trigger mobility registrations of the CM-IDLE UE(s), as defined in TS 23.502 [3], if the new TAC is not in the TAI list in the RA.

Editor's Note:	This clause may need update once RAN3 concludes on mobility aspects for connected mode UEs upon MWAB mobility.

5.X.5.3	Impact of MWAB mobility

The mobility of MWAB includes the MWAB-UE and the MWAB-gNB aspect.

-	The mobility of the MWAB-UE reuses the exiting UE mobility procedures specified for normal UE.

-	The MWAB-gNB can obtain a new configuration from OAM server of MWAB Broadcasted PLMN during mobility.

-	The MWAB-UE may request new or modify existing PDU sessions for backhaul links related to N2/N3/Xn interface, upon request from the MWAB-gNB as it moves in the network and the MWAB-gNB updates the TNL association as specified in clause 5.21.1 and TS 38.401 [42] to use the new TNL information associated with the new PDU session(s) for backlinks as needed.

-	SSC mode 3 may be applied to the PDU sessions for backhaul linksto minimize traffic interruption when UPF re-allocation for these PDU sessions is required during the MWAB mobility.

-	When it is required to change AMF for the UEs served by the MWAB-gNB upon mobility of a MWAB into a new region, the TAC(s) and Cell ID(s) the MWAB-gNB announces also change.

# * * * * End of changes * * * *