3GPP TSG-WG SA2 Meeting #164	S2-2408908

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-2407849)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0142 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on SL Positioning using Located UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - And also, this paper proposes to clarify that the QoS requirement for Target UE's positioning may be used by the Target UE to derive the required QoS for Located UE(s) positioning. - The last point is to align the LMF logic on the selection of Ranging/SL Positioning and/or Uu absolute Positioning with 23.273: “For the undiscovered UEs or UEs for which the Ranging/Sidelink Positioning location measurements cannot be obtained based on their sidelink positioning capabilities among the other UEs 2 to n, the LMF may interact with GMLC to initiate the 5GC-MT-LR procedure using Application Layer ID for UE2 to n to get their absolute locations. In order to calculate the relative locations or distances and/or directions between UE1 and these UEs, if the absolute location of UE1 is not known yet, the LMF will trigger the 5GC-MT-LR procedure to derive the location of UE1. The LMF uses the absolute locations of all the UEs to calculate the relative locations or distances and/or directions between the pairs of these UEs. LMF may determine the Ranging/Sidelink Positioning location results in local coordinate if Coordinate ID is received in step 9. To fulfil the required QoS, the LMF may determine to use both Ranging/SL Positioning and Uu absolute Positioning to obtain the location results.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The QoS requirement for Target UE's positioning may be used by the Target UE to derive the required QoS for Located UE(s) positioning. - Determine to use both Ranging/SL Positioning and/or Uu absolute Positioning to obtain the location results |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure of Located UE is not properly working. Functionalities of LMF defined in 23.586 cause confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.8, 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.3.8	LMF

In addition to the functions defined in TS 23.273 [8], the LMF supports the following:

-	The Network based SL Positioning and Network assisted SL Positioning;

-	Determine using the Network based SL Positioning or Network assisted SL Positioning.

-	Trigger Ranging/Sidelink positioning.

-	Exchange the Ranging/Sidelink Positioning capability.

-	Exchange the Ranging/Sidelink Positioning assistant data.

-	Exchange Ranging/Sidelink positioning signal measurement data/result.

-	Support of receiving stored Ranging/Sidelink Positioning capability from AMF and support of providing updated Ranging/Sidelink Positioning capability to AMF.

-	Determine to apply UE based Sidelink Positioning or Network based SL Positioning.

-	Determine to use both Ranging/SL Positioning and Uu absolute Positioning based on the QoS requirement to obtain the location results (e.g. relative locations or distances and/or directions related to the UEs as defined in clause 4.4).

-	Determine to use Uu absolute Positioning to obtain the location results (as defined in clause 4.4) when the Ranging/SL Positioning fails.

-	Determine that Target UE or LMF selects Located UE.

-	Determine Ranging/SL positioning method based on the positioning QoS requirement of Target UE or Ranging/SL Positioning QoS requirement, UE’s Ranging/Sidelink Positioning capability.

-	Determine the required QoS for Located UE positioning.

-	If the scheduled location time is absent, optionally determine the same scheduled location time for the Ranging/SL positioning and the positioning of the Located UE(s).

-	Determine the location of the Target UE based on the Ranging/SL positioning measurement data and the location of Located UE(s) for Network based SL Positioning, or based on result reported by SL Positioning Server UE for Network assisted SL Positioning.

-	Interaction with GMLC to get the location of Located UE/SL Reference UE using the Application Layer ID.

-	Delivering the Ranging/Sidelink positioning service request/response for the Ranging service exposure to UE.

# * * * * Second change * * * *

6.6	Procedure of UE only Sidelink Positioning for Target UE using Located UE

This procedure is used to acquire absolute location of Target UE using Located UE, where LMF is not involved for SL Positioning/Ranging. The Located UE is assumed to already know its absolute location or can get its absolute location via the legacy 5GC-MO-LR Procedure, and its privacy profile allows its location to be shared with the Target UE.

Procedures for Ranging/Sidelink Positioning control (UE-only operation) defined in clause 6.8 applies with the following adaptations:

-	UE1 is the Target UE, and UE2 is the Located UE

-	Step 1, the service request from Target UE application layer indicates that absolute location of the Target UE is required

-	Before step 6, target UE sends a scheduled location time (Time T) to each Located UE. If the absolution location of Located UE is not known, 5GC-MO-LR procedure is triggered, as specified in clause 6.2 of TS 23.273 [8], with the Time T included in the 5GC-MO-LR Request. The QoS requirement for Target UE's positioning in step 1, may be used by the Target UE to derive the required QoS for Located UE(s) positioning. Target UE sends the required QoS for Located UE positioning to each Located UE, and the required QoS for Located UE positioning is included in the 5GC-MO-LR Request.

-	Ranging/Sidelink positioning between target UE and located UE is scheduled with the Time T.

-	Step 8 absolute location of the Target UE is calculated.

# * * * * End of changes * * * *